3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100202

# Electronic Meeting, 25 Jan – 5 Feb, 2021

Agenda item:	7.9.2

Source:	Apple Inc.

Title:	RRM Core requirements maintenance for eMIMO

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#97e topics for eMIMO RRM core maintenance were discussed and the remaining issues were captured in way forward [1]. In this contribution we address some open issues related to eMIMO RRM core requirements.

# 2. Discussion

## Requirements with Multi-TRxP

In RAN4#96e it was agreed for Rel-16 eMIMO multi-TRxP transmission [2]:

- No RRM core requirement impact identified on MRTD/MTTD values specified in Rel-15;

- It is RAN4 common understanding that MRTD/MTTD requirements in clauses 7.5.3, 7.6.3 and 7.6.4 is sufficient for support the deployment with multi-DCI based and single-DCI based multi-TRxP transmission.

The above conclusion is not captured in the spec. Hence, we propose to capture that all requirements are applicable to multi-TRxP unless otherwise specified.

Proposal #1: Update 38.133 to include that requirements are applicable to multi-TRxP deployments

The proposal is captured in our CR [3].

## Link Recovery Requirements in NR-DC

In Rel-16 SCell BFD and BFR was introduced and in 38.133, we have the following resource sharing factors (PBFD and PCBD):

The values of PBFD used in Table 8.5.3.2-1 and Table 8.5.3.2-2 are defined as For each CSI-RS resource in the set configured for PCell or PSCell - PBFD = 1,. For each CSI-RS resource in the set configured for a Scell - PBFD is the number of band(s) on which UE is performing beam failure detection only for Scell. ---- The values of PCBD used in Table 8.5.5.2-1 and Table 8.5.5.2-2 are defined as For each SSB resource in the set configured for Pcell or PSCell - PCBD = 1. For each SSB resource in the set configured for a Scell - PCBD is the number of band(s) on which UE is performing candidate beam detection only for Scell. ---- The values of PCBD used in Table 8.5.6.2-1 and Table 8.5.6.2-2 are defined as For each CSI-RS resource in the set configured for Pcell or PSCell - PCBD = 1. For each CSI-RS resource in the set configured for a Scell - PCBD is the number of band(s) on which UE is performing candidate beam detection only for Scell.

These values fine for SA, EN-DC and NE-DC when only PCell or PScell are configured. But, in NR-DC we have PCell and PScell and may be also SCell(s), hence PBFD and PCBD need to be re-defined. The sharing factor can be defined based on the searcher sharing.

In RAN4#97e 2 options for searcher sharing were discussed and captured in [1].

- Option 1: Searcher is shared between PScell and Scell(s) and higher priority for PScell.

- Option 2: Searcher is shared between PScell and Scell(s) with equal priority

Based on the options, we prefer that the searcher shared between PScell and Scell(s) and higher priority for PScell.

Proposal #2: Redefine PBFD and PCBD based on assumption that searcher shared between PScell and Scell(s) and higher priority for PScell.

Based on the searcher sharing, we recommend that the PBFD and PCBD are defined as:

| Scenario | PBFD and PCBD for FR1 PCC | PBFD and PCBD for FR2 PSCC | PBFD and PCBD for FR1+FR2 SCC |
| --- | --- | --- | --- |
| FR1 + FR2 NR-DC (FR1 Pcell and FR2 Pscell) | 1 | 2 | 2×Z |
| Note 1: Z is the number of band(s) on which UE is performing beam failure detection only for Scell |  |  |  |

Proposal #3: For NR-DC, define the sharing factors (PBFD and PCBD) as: 1 for PCell; 2 for PScell ; 2xZ for SCell.

These proposals are captured in our CR [4].

## Requirements for Pathloss RS Activation

In Rel-16 delay requirements for Pathloss RS activation via MAC CE are captured in  TS 38.133 section 8.14.

The known condition is defined as follows:

In the known condition definition, the period for known condition is defined starting from the last transmission of L1-RSRP measurement report. Based on our understanding the known condition shall apply from the last transmission of the RS for L1-RSRP report until the end of switching delay. Hence, we propose to update the period for known condition to start from the last transmission of RS for L1-RSRP measurement.

Proposal #4: Update the known condition period to start from the last transmission of RS for L1-RSRP measurement.

The known condition definition doesn’t consider RS QCLed to target pathloss RS.

Observation #1: Definition of known condition for pathloss RS doesn’t consider RS QCLed to target pathloss RS.

The known condition is mainly defined to determine if the UE knows the RX beam for the target RS and the RX beam can also be determined be a RS QCLed with Type D to it. Hence, we propose to extend the definition of known condition to include RS QCLed with Type-D to target PL-RS.

Proposal #5: Update the known definition of pathloss RS to include RS QCLed to target PL-RS.

In TS 38.133 the PL-RS switching delay is captured as:

The delay requirement captured is specifically for PL-RS not maintained by the UE, while TS 38.213 captures the switching delay for PL-RS maintained by the UE.

Observation #2: PL-RS activation delay in 38.133 captures delay when PL-RS is not maintained by UE.

Hence, we propose to capture that the delay requirements in section 8.14.3 or TS 38.133 are for PL-RS not maintained by UE.

Proposal #6: Update PL-RS activation delay to be applicable when PL-RS is not maintained by UE.

These proposals are captured in our CR [5].

# 3. Conclusion

In this paper, we address some open issues related to eMIMO RRM core requirements. Our observations and proposals are captured below:

Requirements with Multi-TRxP

Proposal #1: Update 38.133 to include that requirements are applicable to multi-TRxP deployments

Link recovery requirements in NR-DC

Proposal #2: Redefine PBFD and PCBD based on assumption that searcher shared between PScell and Scell(s) and higher priority for PScell.

Proposal #3: For NR-DC, define the sharing factors (PBFD and PCBD) as: 1 for PCell; 2 for PScell ; 2xZ for SCell.

Requirements for Pathloss RS Activation

Proposal #4: Update the known condition period to start from the last transmission of RS for L1-RSRP measurement.

Observation #1: Definition of known condition for pathloss RS doesn’t consider RS QCLed to target pathloss RS.

Proposal #5: Update the known definition of pathloss RS to include RS QCLed to target PL-RS.

Observation #2: PL-RS activation delay in 38.133 captures delay when PL-RS is not maintained by UE.

Proposal #6: Update PL-RS activation delay to be applicable when PL-RS is not maintained by UE.

# Reference

- R4-2017375, “WF on NR eMIMO RRM Performance requirements”, Samsung.

- R4-2012146, “WF on Completing Rel-16 eMIMO RRM Core Requirement”, Samsung.

- R4-2100203, “CR to 38.133 on RRM requirements for multi-TRP (R16)”, Apple.

- R4-2100205, “CR to 38.133 on Link Recovery requirements (R16)”, Apple.

- R4-2100207, “CR to 38.133 on Pathloss activation delay requirements (R16)”, Apple.