3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100209

# Electronic Meeting, 25 Jan – 5 Feb, 2021

Agenda item:	7.9.3.2

Source:	Apple Inc.

Title:	RRM Performance requirements for L1-SINR Measurement Accuracy

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#97e requirements for L1-SINR measurement accuracy were discussed and way forward [1] was agreed. In this contribution we present our views on the open issues for L1-SINR measurement accuracy requirements.

# 2. Discussion

The open issues in [1] related to L1-SINR measurement accuracy are:

- Implementation margin of L1-SINR measurement accuracy

  - FFS additional FR2 implementation margin in CMR + IMR scenario

    - Option 1: No obvious difference as it is SINR in CMR + IMR

    - Option 2: Consider RF margin [x] dB higher for FR2 than FR1 in CMR + IMR

  - FFS if any other implementation margin necessary to derive final requirement on the basis of simulation-based accuracy

- Accuracy requirements of L1-SINR under extreme condition

  - Option 1: 1dB higher than for normal condition (same logic as SS-SINR)

  - Option 2: 2dB higher than for normal condition

  - Option 3: other values

- Relative accuracy of L1-SINR measurement requirement

  - FFS the necessity and definition of relative accuracy of L1-SINR measurement requirement

  - FFS the methodology for deriving relative accuracy of L1-SINR measurement requirement

Implementation margin

For L1-RSRP measurement accuracy requirements, we consider extra RF margin in FR2, but we don’t have extra margin for SS-SINR measurements. The gain inaccuracies are cancelled out since SINR is a relative measurement. In case of SS-SINR measurement, the signal and noise power estimate are measured on the same resource at the same time. In case of CMR+IMR L1-SINR, the signal power and interference power are made at different time, there could be measurement inaccuracies that are not cancelled out. Hence, we propose to introduce 1 dB extra RF margin in FR2 for L1-SINR measurement for CMR+IMR case.

Proposal #1:  Introduce 1dB extra RF margin for CMR+IMR case in FR2.

Accuracy requirements under extreme condition

The accuracy for L1-SINR measurement in extreme condition could use the same extra margin as that for SS_SINR. For SS-SINR, we have 1dB extra margin in extreme condition compared to normal condition.

Proposal #2: For L1-SINR measurement accuracy, introduce 1 dB extra margin for extreme conditions.

Relative accuracy requirements

The L1-SINR report is given  in absolute value and differential values, similar to L1-RSRP. Extending the logic we should define relative accuracy requirements for L1-SINR as we have for L1-RSRP.

Proposal #3: Introduce relative accuracy requirements for L1-SINR measurement.

# 3. Simulation Results

For the simulation assumptions agreed in [2] , we provide updated simulation results below.

TBA

# 4. Conclusion

In this paper, we provide our views on the remaining open issues related to L1-SINR measurement accuracy requirements. Our proposals are captured below:

Proposal #1:  Introduce 1dB extra RF margin for CMR+IMR case in FR2.

Proposal #2: For L1-SINR measurement accuracy, introduce 1 dB extra margin for extreme conditions.

Proposal #3: Introduce relative accuracy requirements for L1-SINR measurement.

# Reference

- R4-2017375, “WF on NR eMIMO RRM Performance requirements”, Samsung.

- R4-1915850, “Way forward on NR eMIMO RRM”, Samsung.