3GPP TSG-RAN WG4 Meeting #98-e	R4-2100219

Electronic Meeting, 25 Jan – 5 Feb, 2021

Agenda Item:	11.9.2

Source:	Apple

Title:	UE measurement relaxation for RLM and/or BFD

Document for:	Discussion

# 1	Introduction

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4].

b) Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4]

    - Note: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS

In Rel-17 UE power saving WI [1], additional enhancement in CONNECTED state is proposed:

[3] captures the discussion in RAN4 97e. Evaluation assumption for mobility impact analysis and power saving benefit analysis is summarized in [4].

In this paper, we discuss the power saving benefit, and potential schemes RLM/BFD measurement relaxation.

# 2	Discussion

2.1 Potential power saving gain

Evaluation methodology for UE power consumption is based on the power model in annex of TR 38.840 [2]. Additional evaluation parameters are discussed and captured in section 3 of [4]. We focus on FR1 power evaluation with detailed evaluation parameters listed below.

Table I Evaluation assumptions for power saving gain of RLM/BFD relaxation

| Parameters | FR1 |
| --- | --- |
| longDRXCycle | 160ms for FTP model 3 Or 40ms for FTP model 3 |
| OnDurationTimer | 8ms with 160ms long DRX cycle for FTP Model 3 5ms with 40ms long DRX cycle for FTP Model 3 |
| DRX_InactivityTimer | 40ms with 160ms long DRX cycle for FTP Model 3 20ms with 40ms long DRX cycle for FTP Model 3 |
| Slot length | 0.5ms |
| Relaxation factor (X) | 2, 4, 8 |
| PDCCH-WUS configuration | Yes WUS offset: 4 slot No wake-up if WUS is missed |
| PDCCH cross slot scheduling | Enabled |
| Traffic Model | FTP Model 3 0.5Mbytes packet size 200ms mean inter-arrival time (Note: Other options are not precluded) |
| RLM-RS periodicity | SSB: 20 ms |
| RLM-RS location | SSB configuration is outside of DRX ON duration |

The power consumption performance evaluation is shown in Table II for 160ms DRX cycle, and Table III for 40ms DRX cycle. As expected, RLM/BFD relaxation achieve higher power saving gain with smaller DRX cycle.

Table II Power evaluation with 160ms DRX cycle.

|  | No relaxation | 2x relaxation | 4x relaxation |
| --- | --- | --- | --- |
| Average power per slot | 20.21 | 19.21(4.95%) | 18.71(7.42%) |

Table III Power evaluation with 40ms DRX cycle.

|  | No relaxation | 2x relaxation | 4x relaxation | 8x relaxation |
| --- | --- | --- | --- | --- |
| Average power per slot | 34.29 | 30.49 (11.08%) | 28.61 (16.56%) | 27.66 (19.34%) |

Observation: RLM/BFD relaxation achieve higher UE power saving gain with shorter DRX cycle, with up to 19.34% power saving gain is observed.

2.2 Scheme of RLM/BFD measurement relaxation

The following options has been discussed in [3]. At least extending evaluation period of RLM/BFD measurement can be considered as the scheme of RLM/BFD measurements relaxation. FFS schemes as follows

- Option 1a: RAN4 to further discuss use of a scaling factor for defining the relaxed RLM/BM evaluation period and indication intervals.

- Option 2: Reducing the number of candidate beams when UE fulfilled relaxed criteria can be a feasible way to reduce power consuming.

- Option 3: Reducing the sample number.

UE power consumption is related to RLM-RS configuration, DRX configuration, Tindication_interval and Tevaluation.  When short DRX cycle is used, a UE will wake up more often for RLM/BFD evaluation, such consume a lot of power. Furthermore, when a UE is in low mobility and/or not-cell edge condition, channel condition changes slowly and risk of radio link failure is low. Therefore, it is possible to relax RLM/BFD measurement in such scenario. Using a scaling factor to define the relaxed RLM/BFD evaluation period and indication intervals is a straightforward way to go. The relaxation factor depends on various factors including DRX cycle configuration, RLM-RS configuration, mobility and SINR conditions etc.

In FR2, reducing the number of candidate beams can be an alternative way to reduce power consumption, particularly for SSB based RLM/BFD. However, since beam sweeping is used in other L3 measurement, reducing number of beams for SSB based RLM/BFD can be difficult. Reducing the number of samples can reduce the accuracy, which should be further evaluated.

On relaxation criterion, R16 RRM measurement relaxation defined different criterions, such as not cell edge criterion, low mobility criterion, or both. These criterions can be used as a starting point for RLM/BFD measurement relaxation. For not cell edge criterion, SINR condition can be specified on top of RSRQ and RSRP.

RLM/BFD measurement relaxation should not sacrifice the measurement accuracy and network performance. UE should also fall back to normal RLM measurement when relaxation criterion does not meet. In addition, UE should fall back to normal RLM measurement when N310 starts to count.

Proposal 1: RLM/BFD relaxation through scaling on DRX cycle is supported.

Proposal 2: Relaxation factor depends on various factor including DRX cycle configuration, RLM-RS configuration, mobility and channel conditions etc.

Proposal 3: R16 RRM relaxation criterion can be used as baseline for RLM/BFD relaxation.

Proposal 4: UE revert back to normal operation when criterion is not met, or when N310 start to count.

# 3	Summary

Observation: RLM/BFD relaxation achieve higher UE power saving gain with shorter DRX cycle, with up to 19.34% power saving gain is observed.

Proposal 1: RLM/BFD relaxation through scaling on DRX cycle is supported.

Proposal 2: Relaxation factor depends on various factor including DRX cycle configuration, RLM-RS configuration, mobility and channel conditions etc.

Proposal 3: R16 RRM relaxation criterion can be used as baseline for RLM/BFD relaxation.

Proposal 4: UE revert back to normal operation when criterion is not met, or when N310 start to count.

# References

- RP-200938, UE power saving enhancements, MediaTek Inc, July 2020

- 3GPP TR 38.840: Study on UE Power Saving (Rel-16).

- R4-2017269, WF on NR UE power saving enhancement, MediaTek

- R4-2017306, Evaluation assumptions for R17 RLM/BFD relaxation, vivo, MediaTek