3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100271

# Electronic Meeting, 25th January – 5th February, 2021

Source:		Verizon, Ericsson

Title:			TP for TR 37.826 for DC_66_n77

Agenda item:		9.20.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.826 to include DC_66_n77 according to the request in [1].

# Reference

[1] RP-20xxxx, Revised WID on High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band

# Text Proposal

<Start of Text Proposal>

## 6.x	DC_66A_n77A

### 6.x.1	Transmitter Characteristics

#### 6.x.1.1	Maximum Output Power

Table 6.x.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 6.x.1.2	Co-existence study

According to the PC3 CA_n66A-n77A study, the same results are listed in below,

- The 2nd and 5th order IMD products generated from dual uplinks of band 66 and n77 may fall into band 66 Rx frequency range

Thus additional MSD for IMD 2 and 5 should be considered to mitigate the impact of the interference for PC2 DC_66A_n77A combination.

### 6.x.2	Receiver Characteristics

#### 6.x.2.1		MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

Table 6.x.2.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD for PC2 | Duplex mode | Source of IMD |
| Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| DC_66A-n77A | 66 | 1775 | 5 | 25 | 2175 | 37 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | 66 | 1730 | 5 | 25 | 2130 | 20 | FDD | IMD5 |
|  | n77 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |

<End of Text Proposal>