3GPP TSG RAN WG4 Meeting #98-e	R4-2100286

Online, 25th Jan. – 05th Feb., 2021

Agenda item:	9.20.2

Source:	LG Electronics

Title:	TP on MSD test results for Power Class 2 UE for DC (1 LTE FDD band +1 NR TDD band) UE

Document for:	Approval

1	Introduction

In the latest RAN Plenary, some new DC band are included to support PC2 DC band combinations in Rel-17 as follow [1].

Table 1: New PC2 DC band combinations in Rel-17

| EN-DC configuration | contact name, company | Contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- |
| DC_1A_n78A | Basaier Jialade, China Unicom | basejld@chinaunicom.cn | China Unicom, Huawei, HiSilicon, ZTE, vivo | Completed for PC3, Completed for PC2 |
| DC_8A_n78A | Basaier Jialade, China Unicom | basejld@chinaunicom.cn | China Unicom, Huawei, HiSilicon, ZTE, vivo, CHTTL | Completed for PC3, Completed for PC2 |
| DC_2A_n41A | Bill Shvodian, T-Mobile USA | bill.shvodian@t-mobile.com | Nokia, Ericsson, Deutsche Telekom | Completed for PC3, New for PC2 |
| DC_66A_n41A. | Bill Shvodian, T-Mobile USA | bill.shvodian@t-mobile.com | Nokia, Ericsson, Deutsche Telekom | Completed for PC3, New for PC2 |
| DC_7A_n78A | Donghyeon Kim, SK Telecom | dhthomas.kim@sk.com | Samsung, LGE, Nokia, CHTTL, Bell Mobility | Completed for PC3, New for PC2 |
| DC_2A_n77A | Zheng Zhao, Verizon | zheng.zhao@verizonwireless.com | Ericsson, Nokia, Samsung, Qualcomm | Completed for PC3, New for PC2 |
| DC_5A_n77A | Zheng Zhao, Verizon | zheng.zhao@verizonwireless.com | Ericsson, Nokia, Samsung, Qualcomm | Completed for PC3, New for PC2 |
| DC_13A_n77A | Zheng Zhao, Verizon | zheng.zhao@verizonwireless.com | Ericsson, Nokia, Samsung, Qualcomm | Completed for PC3, New for PC2 |
| DC_66A_n77A | Zheng Zhao, Verizon | zheng.zhao@verizonwireless.com | Ericsson, Nokia, Samsung, Qualcomm | Completed for PC3, New for PC2 |

Based on the revised WID, we analyse the self interference problems for the new DC band combinations.

2	Self interference analysis

In this session, we analyze the self interference problems to the own Rx bands as follow

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_2_n41 | DC_2A-n41A | No harmonic problem between two band | - No IMD problem into B2 - No IMD problem into n41 | Need to define MSD for cross band isolation issue in Band 2 | 0.6 dB MSD value for PC3 was defined. need to define new MSD values for PC2 |
| DC_66_n41 | DC_66A_n41A | No harmonic problem between two band | - No IMD problem into B66 - 4th IMD fall into n41. But TDD band only allow Tx or Rx in the same time No issue | Need to define MSD for cross band isolation issue in Band 66 | 3.5 dB MSD value for PC3 was defined. need to define new MSD values for PC2 |
| DC_7_n78 | DC_7A_n78A | No harmonic problem between two band | - No IMD problem into B7 - 4th IMD fall into n78. But TDD band only allow Tx or Rx in the same time No issue | Need to define MSD for cross band isolation issue in Band 7 | 4.5 dB MSD value for PC3 was defined. need to define new MSD values for PC2 |
| DC_2_n77 | DC_2A_n77A | 2nd harmonic from B2 fall into n77. But TDD band only allow Tx or Rx in the same time No issue | - 2nd , 4th & 5th IMDs fall into B2 - 4th & 5th IMDs fall into n77. But TDD band only allow Tx or Rx in the same time No issue | No cross band isolation issues | 26.0dB/ 8.0dB MSD values by 2nd/ 4th IMD are specified for PC3 DC_2A_n78A UE. need to define new MSD values for PC2 UE |
| DC_5_n77 | DC_5A_n77A | 4th & 5th harmonics from B5 fall into n77. But TDD band only allow Tx or Rx in the same time No issue | - 4th & 5th IMDs fall into B5 - 2nd & 5th IMDs fall into n77. But TDD band only allow Tx or Rx in the same time No issue | No cross band isolation issues | 8.3 dB MSD value by 4th IMD is specified for PC3 DC_5A_n78A UE. need to define new MSD values for PC2 UE |
| DC_13_n77 | DC_13A_n77A | 5th harmonics from B13 fall into n77. But TDD band only allow Tx or Rx in the same time No issue | - 5th IMD fall into B13 - 2nd IMD fall into n77. But TDD band only allow Tx or Rx in the same time No issue | No cross band isolation issues | There is no MSD requirements for PC3 DC_13A_n77A UE. need to define new MSD values for PC2 UE |
| DC_66_n77 | DC_66A_n77A | 2nd harmonic from B66 fall into n77. But TDD band only allow Tx or Rx in the same time No issue | - 2nd & 5th IMDs fall into B66. - 4th & 5th IMDs fall into n77. But TDD band only allow Tx or Rx in the same time No issue | No cross band isolation issues | 5.0 dB MSD value by 5th IMD is specified for PC3 DC_66A_n78A UE. need to define new MSD values for PC2 UE |

There are cross band isolation issues and IMD problems to the own Rx band when dual uplink is applied. Hence, RAN4 need to define the MSD requirmeents for PC2 DC (1 LTE FDD band + 1 NR TDD band) band combinations in Rel-17.

3. MSD proposal

3.1 Cross band isolation issues

Table 1 show the MSD requirements for cross band isolation issues for power class 3 DC UE with LTE 1 FDD band and NR 1 TDD band combinations in TS38.101-3.

Table 1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n13 | 3 | [3] | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |  |
| n1 | 40 | 6.6 | 6.6 | 6.6 | 6.6 |  |  |  |  |  |  |  |  |  |
| 13 | n3 | 3 | 2.2 | 1.9 | 1.7 | 1.6 | 1.5 | [1.4] |  |  |  |  |  |  |
| 1 | n40 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 |  | 6.6 |  |  |
| 1 | n41 |  | 6.1 | 6.1 | 6.1 |  | [6.1] | 6.1 | 6.1 | 6.1 |  | 6.1 | 6.1 | 6.1 |
| n3 | 11 | 6.4 | 6.1 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | n41 |  | 0.7 | 0.7 | 0.7 |  | [0.7] | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.7 | 0.7 |
| 3 | n51 | 6.4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 8.3 | 8.3 | 8.3 | 8.3 | [8.3] | [8.3] | 8.3 |  |  |  |  |  |  |
| n3 | 41 | 0.7 | 0.7 | 0.7 | 0.7 |  |  |  |  |  |  |  |  |  |
| n5 | 28 | 4.5 | 3 | 2.2 | 0.3 |  |  |  |  |  |  |  |  |  |
| 7 | n40 | 3.7 | 3.4 | 3.2 | 3.1 | [3.1] | [3.1] | 3.1 | 3.1 | 3.1 |  | 3.1 |  |  |
| n38 | 1 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| n38 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n38 | 4 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| n38 | 66 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| n40 | 1 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |  |  |  |
| n41 | 4 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |  |
| 40 | n1 | 8.3 | 8.3 | 8.3 | 8.3 | [8.3] | [8.3] | [8.3] | [8.3] |  |  |  |  |  |
| n40 | 7 | 3.7 | 3.7 | 3.7 | 3.7 |  |  |  |  |  |  |  |  |  |
| n41 | 1 | 9.1 | 9.1 | 9.1 | 9.1 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| 41 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | [0.6] |  |  |  |  |  |  |
| n41 | 661 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n50 | 3 | 2.5 | 1.9 | 1.6 | 1.5 |  |  |  |  |  |  |  |  |  |
| n77 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n77 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| 41 | n77 |  | 8.3 | 8.3 | 8.3 | [7.3] | [6.5] | 6.3 | 5.3 | 4.5 | [4.3] | 4.0 | 3.9 | 3.8 |
| n78 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | 3.3 | 3.3 | 3.3 | 3.3 |  |  |  |  |  |  |  |  |  |
| n78 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n78 | 46 |  |  |  | 7 |  |  |  |  |  |  |  |  |  |
| 41 | n78 |  | 8.3 | 8.3 | 8.3 | [7.3] | [6.5] | 6.3 | 5.3 | 4.5 | [4.3] | 4.0 | 3.9 | 3.8 |
| n843 | 3 | 3 | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2515 – 2690 MHz. NOTE 3: These requirements apply when the uplink is active in Band n1, n84 and the separation between the lower edge of the uplink channel in Band n1, n84 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band n1, n84. NOTE 4: The DL victim band should be configured using the lowest SCS that is compatible with the highest CBW for which an MSD is specified. NOTE 5: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

In here, RAN4 only allow 23dBm + 23dBm or 23 dBm +26 dBm power combinations per LTE or NR system for PC2 DC UE. And do not allow PC2 transmission per E-UTRA or NR cell group.

Based on the specified MSD value, we analyse the MSD value for the PC2 DC_2A_n41A, DC_66A_n41A and DC_7A_n78A UE with follow simulation assumptions in Table 2 when NR Transmission power is configured with 26dBm

Table 2.Simulation asummptions for MSD requirements

Table 2-1 Assumptions for DC_2A_n41A

| Band n41 PA noise @Band 2 Rx (dBm/Hz) | -118 |
| --- | --- |
| RFIC IIP2 (dBm) | 50 |
| n41 front end filter Loss (dB) | 4 |
| B2 front end filter Loss (dB) | 4 |
| PA to antenna and LNA to antenna IL (dB) | 4 |
| B2/n41 diplexer isolation (dB) | 10 |
| n41 filter attenuation @ B2 RX (dB) | 30 |
| Cross-band isolation (dB) (n41 Tx --> B2 Rx) | 44 |
| Rx Antenna isolation (dB) | 10 |

Table 2-2 Assumptions for DC_66A_n41A

| Band n41 PA noise @Band 66 Rx (dBm/Hz) | -117 |
| --- | --- |
| RFIC IIP2 (dBm) | 50 |
| n41 front end filter Loss (dB) | 4 |
| B66 front end filter Loss (dB) | 4 |
| PA to antenna and LNA to antenna IL (dB) | 4 |
| B66/n41 diplexer isolation (dB) | 10 |
| n41 filter attenuation @ B66 RX (dB) | 30 |
| Cross-band isolation (dB) (n41 Tx --> B66 Rx) | 42 |
| Rx Antenna isolation (dB) | 10 |

Table 2-3 Assumptions for DC_7A_n78A

| Band n78 PA noise @Band 7 Rx (dBm/Hz) | -115 |
| --- | --- |
| RFIC IIP2 (dBm) | 50 |
| n78 front end filter Loss (dB) | 4 |
| B7 front end filter Loss (dB) | 4 |
| PA to antenna and LNA to antenna IL (dB) | 4 |
| B7/n78 diplexer isolation (dB) | 10 |
| n78 filter attenuation @ B7 RX (dB) | 30 |
| Cross-band isolation (dB) (n78 Tx --> B7 Rx) | 40 |
| Rx Antenna isolation (dB) | 10 |

Base on the above assumptions, the MSD values are derived as shown in Table 3 by using MRC combining.

Table 3 MSD due to cross band isolation for PC2 for DC band combinations for case 2 (23dBm +26dBm)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n41 | 2 | 2.3 | 2.3 | 2.3 | 2.3 |  |  |  |  |  |  |  |  |
| n41 | 66 | 5.3 | 5.3 | 5.3 | 5.3 |  |  |  |  |  |  |  |  |
| n78 | 7 | 6.5 | 6.5 | 6.5 | 6.5 |  |  |  |  |  |  |  |  |

3.2 Inter-modulation problems by dual uplink

In this session, we analyze the required MSD values in the own Rx band by IMD product from dual uplink transmission. In table 4 and Table 5, we show the basic RF simulation assumptions.

Table 4: UE RF Front-end component parameters

| UE ref. architecture Component | Diplexer Architecture w/ single ant. |  |  |  |
| --- | --- | --- | --- | --- |
|  | PC2 for DC_2A_n77A, DC_5A_n77A, DC_13A_n77A and DC_66A_n77A |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 |
| Triplexer | 110 | 72 | 55 | 52 |
| Diplexer | 115 | 87 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 |
| PA Forward | 28.0 | 32 | 30 | 28 |
| PA Reversed | 40 | 30.5 | 30 | 30 |
| LNA | 10 | 0 | 0 | -10 |

Table 5 show the isolation levels according to the RF component.

Table 5: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Triplexer | 20 | High/low band isolation |
| Diplexer | 25 | High/low band isolation |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |

The existing MSD levels in TS38.101-3 show in Table 6.

Table 6: MSD test requirements for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n78A DC_2A_n78(2A) | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  |  |  |  |  |  | 28.74 |  |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_2A_n78A DC_2A_n78(2A) | 2 | 1885 | 5 | 25 | 1965 | 8.0 | IMD4 |
|  |  |  |  |  |  | 10.74 |  |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_5A_n78A DC_5A_n78(2A) | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_66A_n78A | 66 | 1730 | 5 | 25 | 2150 | 5.0 | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 | N/A | N/A |

Based on above assumptions and test configuration, we proposed the MSD levels as below.

Table 7: Proposed MSD test configuration and results by IMD problems for PC2 DC UE

| DC bands | UL DC | IMD | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n77A | 2 | IMD2 ¦fB2 -fn77¦ | 1855 | 5 | 25 | 1935 | 5 | 32.6 |
|  | n77 |  | 3790 | 10 | 50 | 3790 | 10 | N/A |
|  | 2 | IMD4 ¦3*fB2 -fn77¦ | 1885 | 5 | 25 | 1965 | 5 | 17.5 |
|  | n77 |  | 3690 | 10 | 50 | 3690 | 10 | N/A |
| DC_5A_n77A | 5 | IMD4 ¦3*fB5 -fn77¦ | 844 | 5 | 25 | 889 | 5 | 17.7 |
|  | n77 |  | 3421 | 10 | 50 | 3421 | 10 | N/A |
| DC_13A_n77A | 13 | IMD5 ¦4*fB13 -fn77¦ | 782 | 5 | 25 | 751 | 5 | 11.3 |
|  | n77 |  | 3879 | 10 | 50 | 3879 | 10 | N/A |
| DC_66A_n77A | 66 | IMD2 ¦fB66 -fn77¦ | 1730 | 5 | 25 | 2130 | 5 | 34.6 |
|  | n77 |  | 3860 | 10 | 50 | 3860 | 10 | N/A |
|  | 66 | IMD5 ¦3*fB66 -2*fn77¦ | 1730 | 5 | 25 | 2130 | 5 | 10.8 |
|  | n77 |  | 3660 | 10 | 50 | 3660 | 10 | N/A |

4	Conclusion

Above simulation results, we recommend to merged the other interested companies’ simulation results as average manner to derive MSD levels and propose as follow

Proposal 1: For cross-band isolation issue of PC2 DC UE, the proposed MSD values in Table 3 shall be considered in TS38.101-3.

Proposal 2: For IMD problem by dual uplink transmission, the proposed MSD values in Table 7 shall be considered in TS38.101-3.