3GPP TSG-WG RAN4 Meeting #98-e	R4-2100332

# Electronic meeting, 25th January – 5th February, 2021

Title: 	Discussion on the new SUL band n99 UE requirements

Source: 	Huawei, HiSilicon

Agenda item:	9.27.1

Document for:	Discussion

Introduction

At RAN#88e, a new work item related to introduction of SUL band in 1.6 GHz was approved in [1]. A work plan for the WI was also approved in [2] during RAN4#96e. The new proposed SUL band overlaps with the exact frequencies of band n24. With the previous experiences in introducing the SUL band n95 which overlaps with the TDD band n34, it is agreed in RAN4#97e in [3] that RAN4 introduces the corresponding RF requirements to facilitate complete introduction of the new SUL band n99.

For UE RF requirements, introducing SUL band n99 involves the items listed in the below box which are all specified in TS 38.101-1:

UE RF requirements - Operating bands - UE channel bandwidth - Channel raster - UE MOP and MPR/AMPR - Spurious emissions

For the BS requirements, introducing SUL band n99 involves the BS coexistence requirements: Additional spurious emission and co-location emission requirements. The related specs are: TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2. A parallel discussion paper in [4] discussed the BS requirements.

In this paper, we discuss the necessary UE RF requirements to be specified in order to accomplish introduction of the new SUL band n99. The only remaining issue after discussions carried out in the last meeting now is how to put a series of changes in the operating band table to clarify that the A-MPR and additional emissions requirements for n99 should be specified by reference to tables and requirements developed for the UL of n24 as part of the WI related to the introduction of n24 (WI: NR_band_n24).

UE RF Requirements

Operating band number

It is consensus in the last meeting that the SUL band for 1626.5-1660.5 MHz is named with n99.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n99 | 1626.5 MHz –1660.5 MHz | N/A | SUL |

Proposal 1:  Introduce the new SUL band for 1626.5-1660.5 MHz as band n99.

UE channel bandwidth

It is natural that for the new SUL band n99, the UE supporting this band should support all the channel bandwidths specified for band n24 as listed below, since n24 and n99 have the exact same frequencies for UL.

Table 5.3.5-1 Channel bandwidths for each NR band

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n99 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |

Channel raster

The channel raster for n99 should follow exactly the one for band n24 UL.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n99 | 100 | 325300 – <20> – 332100 | N/A |

MOP and A-MPR

The MOP requirements of PC3 for SUL band n99 should follow the ones for band n24 and a few others.

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n99 |  |  |  |  | 23 | ±2 |

For MPR requirements, the generic ones apply to n99.

For A-MPR requirements, it is reasonable that band n99 should follow band n24 at least with NS_01 and NS_52.

Table 6.2.3.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n99 | NS_01 | NS_52 |  |  |  |  |  |  |

Spurious emissions

Regarding spurious emissions, band n99 should have the same requirements as those for band n24 as below.

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE |  |  |
| n24, n99 |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  | NR Band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |

Proposal 2:  Specify UE RF requirements for the new SUL band n99 following UL of band n24.

Conclusions

In this paper, we discuss the necessary UE RF requirements to be specified in order to accomplish introduction of the new SUL band n99. We propose:

Proposal 1:  Introduce the new SUL band for 1626.5-1660.5MHz as band n99.

Proposal 2:  Specify UE RF requirements for the new SUL band n99 following UL of band n24.

References

[1] RP-201251, New WID proposal on introduction of 1.6 GHz NR SUL band with same uplink frequency range of Band 24, Ligado Networks

[2] R4-2010745, Work plan for Introduction of 1.6 GHz NR SUL Band with same uplink frequency range of Band 24, Ligado Networks

[3] R4-2016897, WF on work item NR_SUL_UL_n24, Ligado Networks, Huawei, Hisilicon

[4] R4-2100333, Discussion on the new SUL band n99 BS requirements, Huawei, HiSilicon