3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100377

Electronic Meeting, Jan. 25-Feb. 5, 2021

Title: 	Discussion on NR repeater emission requirements for FR1/FR2

Source: 	CATT

Agenda item:	11.11.2.2

Document for:	Approval

- Introduction

A new WI NR repeater was approved in RAN#90 meeting to define NR repeater for both FR1 and FR2 bands including FDD and TDD mode. This contribution provides our analysis for RF repeater’s emission requirements for both FR1 and FR2.

- Discussion

BS unwanted emission requirements include ACLR, operating band unwanted emissions and transmitter spurious emissions. For LTE repeater, ACLR was not defined because the ACLR can’t be measured due to the fact that the this signal is below the amplifier thermal noise of the repeater amplifier chain [6] [3]. ACRR is defined for LTE repeater but the requirements are less stringent compared with E-UTRA BS. For NR repeater, we think the situation doesn’t change. ACLR with NR adjacent channel may be omitted. ACLR with LTE adjacent channel needs some discussion for the decision.

Observation: ACLR with NR adjacent channel may be omitted. ACLR with LTE adjacent channel needs some discussion for the decision.

For the operating band unwanted emissions and transmitter spurious emissions, reusing BS requirements for both downlink and uplink is reasonable which was done for LTE repeater and NR IAB.

Proposal: Reuse BS operating band unwanted emissions and transmitter spurious emissions requirements for NR repeater.

For LTE repeater, the operating band unwanted emissions requirement was reformed [4]. The necessity for NR repeater can be discussed further when some general issues such as BW are decided.

- Conclusion

This contribution provides the proposals for NR repeater emission requirements.

Observation: ACLR with NR adjacent channel may be omitted. ACLR with LTE adjacent channel needs some discussion for the decision.

Proposal: Reuse BS operating band unwanted emissions and transmitter spurious emissions requirements for NR repeater.

Reference

[1] RP-202927, New WID on NR Repeaters

[2] TS 36.106

[3] TR 25.956

[4] R4-081491, “Text proposal 36.106: Operating band unwanted emissions” Andrew Wireless Systems GmbH; Powerwave Technologies

[5] R4-081639, “Text proposal 36.106: Spurious emissions”, Andrew Wireless Systems GmbH; Powerwave

[6] R4-071750, “LTE Repeater specification Items”, Powerwave Technologies

[7] R4-081640, “Text proposal 36.106:ACRR”,