3GPP TSG-RAN WG4 Meeting #98-e                                                              R4-2100384

Electronic Meeting, 25 Jan.-5 Feb., 2021

Source: 	CATT

Title: 	Discussion on co-existence for 52.6-71GHz

Agenda item:	12.2.4

Document for:	Approval

- Introduction

In RAN4#97-e, whether ACLR and ACS for 70GHz derived from co-existence simulation in TR 38.803 can be applicable for 52.6-71GHz was discussed[1,2], but there was no consensus. The decision relies on if the co-existence simulation parameters for 70GHz still make sense. In this contribution, we provide our views on some parameters of the co-existence study for 52.6-71GHz and try to have some clear view.

- Discussion

From our understanding, many co-existence simulation assumptions for 52.6-71GHz can be same as those used for 70GHz in TR 38.803[3]. However, looking at system simulation assumptions in TR 38.808[4], Some important parameters, for example carrier frequency, bandwidth, BS and UE max TX power, BS and UE antenna model, BS and UE noise figure, are different with those for 70GHz in TR 38.803. We went through some of them to see if the difference should be paid attention and if there’s some expected impact for the co-existence simulation results.

Carrier frequency

The carrier frequency difference between TR 38.803 and TR 38.808 is summarized in Table 2-1.

Table 2-1: Carrier frequency difference between TR 38.803 and TR 38.808

|  | TR 38.803 | TR 38.808 |
| --- | --- | --- |
| Carrier frequency | 70GHz | 60GHz Optional:70GHz |

From RF perspective, 70GHz path loss and hardware challenge are larger than 60GHz. The ACLR/ACS requirement for 70 GHz may be less stringent than 60 GHz, or at least the same as 60 GHz. So 70 GHz carrier frequency assumption is ok for above 52.6 GHz, 60 GHz can also be an extra choice if it’s also considered a typical center frequency.

Observation 1: The 70GHz carrier frequency assumption in R15 NR co-existence simulation is applicable for 52.6-71GHz study, 60 GHz can also be considered as an extra choice.

Bandwidth

The bandwidth difference between TR 38.803 and TR 38.808 is summarized in Table 2-2.

Table 2-2: Bandwidth difference between TR38.803 and TR 38.808

|  | TR 38.803 | TR 38.808 |
| --- | --- | --- |
| Bandwidth | 200MHz | 2000 MHz 400 MHz |

The CBW is not decided yet for 52.6-71 GHz, but the maximum CBW will be much larger than current FR2 bands according to the current discussion. We’re not very clear if large CBW such as 1.6 GHz or 2GHz has much impact to co-existence simulation results. However, this is the first time in RAN4 to study such large CBW, there should at least some discussion to see the impact.

Observation 2: Large CBW assumption for above 52.6 GHz may have some impact to the co-existence simulation.

BS and UE antenna modelling

The BS and UE antenna modelling difference between TR 38.803 and TR 38.808 is summarized in Table 2-3. The Antenna element radiation pattern difference between TR 38.803 and TR 38.808 is summarized in Table 2-4.

Table 2-3: BS and UE antenna modelling difference between TR 38.803 and TR 38.808

|  |  | TR 38.803 |  | TR 38.808 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Indoor | Dense urban | Indoor | Dense urban |
| BS | (Mg, Ng, M, N, P) | (1, 1, 8, 16, 2) | (1, 1, 8, 16, 2) | (1,1,4,8,2) Optional: (1,1,8,16,2) | (1,3,8,16,2) Note: 3 Panel single sector gNB with {0,+120,-120} degree boresight orientations. The gNB will only utilize 1 panel at given moment. |
|  | (dv, dh) | (0.5λ, 0.5λ) | (0.5λ, 0.5λ) | (0.5λ, 0.5λ) | (0.5λ, 0.5λ) |
|  | Antenna element gainGE,max | 5dBi | 8 dBi | 5 dBi | 5 dBi |
|  | Antenna element radiation pattern | Table 5.2.3.2.3-1 of TR38.803 | Table 7.3-1 of TR38.901 | Table A.2.1-7 of TR38.802 for ceiling mount | Table 7.3-1 of TR38.901 |
|  | Array gain per polarization per pannel | 26dBi | 29dBi | 20dBi Optional: 26dBi | 26dBi |
| UE | (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) | (1, 1, 2, 2, 2) | (1,2,2,2,2) Optional: (1,2,4,4,2) | (1,2,2,2,2) Optional: (1,2,4,4,2) |
|  | (dv, dh) | (0.5λ, 0.5λ) | (0.5λ, 0.5λ) | (0.5λ, 0.5λ) | (0.5λ, 0.5λ) |
|  | Antenna element gainGE,max | 5dBi | 5dBi | 5 dBi | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-8 of TR38.802 | Table A.2.1-8 of TR38.802 | Table A.2.1-8 of TR38.802 | Table A.2.1-8 of TR38.802 |
|  | Array gain per polarization per pannel | 11dBi | 11dBi | 11dBi Optional: 17dBi | 11dBi Optional: 17dBi |

Table 2-4:  Antenna element radiation pattern difference between TR 38.803 and TR 38.808

| Antenna element radiation pattern in Table 5.2.3.2.3-1 of TR38.803 | Antenna element vertical radiation pattern (dB) |  |
| --- | --- | --- |
|  | Antenna element horizontal radiation pattern (dB) |  |
|  | Combining method for 3D antenna element pattern (dB) |  |
| Antenna element radiation pattern for ceiling-mount in Table A.2.1-7 of TR38.802 | Antenna element vertical radiation pattern (dB) |  |
|  | Antenna element horizontal radiation pattern (dB) |  |
|  | Combining method for 3D antenna element pattern (dB) |  |

The antenna size assumption (Mg, Ng, M, N, P) in TR 38.803 is more appropriate than TR 38.808 because larger antenna size is needed for higher frequency. The assumed antenna element gain GE,max for BS for dense urban in TR 38.808 is 5 dBi, which is more reasonable than 8dBi in TR 38.803 considering element spacing (0.5λ, 0.5λ). Ceiling mount is considered in TR 38.808, which leads to different antenna element radiation pattern compared with the pattern in TR 38.803. The antenna model as a whole may impact the beamforming performance thus may have some impact with the co-existence simulation. It’s not easy to estimate the antenna model’s impact to the co-existence simulation results before the simulation is conducted.

Observation 3: Antenna element gain, antenna element radiation pattern and ceiling mount scenario may need to be discussed for 52.6-71 GHz. It’s not easy to say if the simulation results will change a lot with new antenna model assumption.

BS and UE max TX power

The BS and UE TX power difference between TR 38.803 and TR 38.808 is summarized in Table 2-5.

Table 2-5: BS and UE max TX power difference between TR 38.803 and TR 38.808

|  | TR 38.803 |  | TR 38.808 |  |
| --- | --- | --- | --- | --- |
|  | Indoor | Dense urban | Indoor | Dense urban |
| BS max TX power | 23dBm | 33dBm | 40 dBm EIRP Optional: 60 dBm EIRP Maximum TxP adjusted to meet EIRP limits | 40 dBm EIRP Optional: 60 dBm EIRP Maximum TxP adjusted to meet EIRP limits |
| UE max TX power | 23dBm | 23dBm | 25 dBm EIRP with 21 dBm max TxP Optional: 40dBm EIRP with 21 dBm max TxP | 25 dBm EIRP with 21 dBm max TxP Optional: 40dBm EIRP with 21 dBm max TxP |

When R15 co-existence simulation was conducted, the BS/UE output power requirements were not decided yet. The assumption capability was assumed much larger than the final requirements, especially for UE. TR 38.808 assumes EIRP capability, which is more reasonable when two NR releases have been finished. From that aspect, the assumption in TR 38.808 is more reasonable at least the EIRP assumption is correct from RF perspective. However, it’s very similar with the antenna model parameters that the impact to the co-existence simulation’s final result can’t be easily estimated without the real simulation.

Observation 4: The BS/UE output power assumption in TR 38.803 is more optimistic than the real capability. New approach such as EIRP assumption in TR 38.808 may be more appropriate. It’s hard to estimate the impact to the simulation results.

BS and UE noise figure

The BS and UE noise figure difference between TR38.803 and TR38.808 is summarized in Table 2-6.

Table 2-6, BS and UE noise figure difference between TR38.803 and TR 38.808

|  | TR 38.803 | TR 38.808 |
| --- | --- | --- |
| Noise figure | BS: 13 UE: 15 dB | 1) Referring to ETSI technical report TR 101 854, NF is in range of 10 – 13 dB. 2) Referring to the system level evaluation assumptions in annex A, BS: 7 dB, UE: 10 dB (13 dB optionally). |

Noise figure is not decided yet but we think the assumption in TR 38.803 can be used as a starting point. Thus this parameter is not a big problem for the simulation in TR 38.803.

Observation 5: Noise figure assumption in TR 38.803 can be reused for co-existence simulation for 52.6-71GHz.

From the above analysis, large CBW, antenna model and BS/UE output power need to be reviewed if R15 NR co-existence simulation results are reused by 52.6-71 GHz. Conducting co-existence simulation is technically safer to decide the requirements but the simulation work is complicated and very time consuming. RAN4 should have some decision on this to further discuss the related requirements.

Proposal: RAN4 should decide how to handle the co-existence simulation for 52.6-71 GHz as soon as possible in order to further discuss the related requirements.

- Conclusion

In this contribution, the some parameters of the co-existence study for 52.6-71GHz are discussed. The following observations and proposals are summarised as below:

Observation 1: The 70GHz carrier frequency assumption in R15 NR co-existence simulation is applicable for 52.6-71GHz study, 60 GHz can also be considered as an extra choice.

Observation 2: Large CBW assumption for above 52.6 GHz may have some impact to the co-existence simulation.

Observation 3: Antenna element gain, antenna element radiation pattern and ceiling mount scenario may need to be discussed for 52.6-71 GHz. It’s not easy to say if the simulation results will change a lot with new antenna model assumption.

Observation 4: The BS/UE output power assumption in TR 38.803 is more optimistic than the real capability. New approach such as EIRP assumption in TR 38.808 may be more appropriate. It’s hard to estimate the impact to the simulation results.

Observation 5: Noise figure assumption in TR 38.803 can be reused for co-existence simulation for 52.6-71GHz.

Proposal: RAN4 should decide how to handle the co-existence simulation for 52.6-71 GHz as soon as possible in order to further discuss the related requirements.

- References

- R4-2015200，TP to TR 38.808: BS RF for NR beyond 52.6 GHz，Nokia, Nokia Shanghai Bell，RAN4#97-e.

- R4-2014401, Discussion on the BS requirements for 52.6-71GHz, CATT, RAN4#97-e.

- TR 38.803, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects.

- TR 38.808, Study on supporting NR from 52.6 GHz to 71 GHz (Release 17).