3GPP TSG-RAN4 Meeting #98-e	                                       R4-2100425

Electronic meeting, Jan. 25 – Feb. 5, 2021

Title:	Simulation results for CSI-SINR measurement accuracy

Source:	CATT

Agenda Item:	7.14.2.1.3

Document for:	Discussion

- Introduction

In RAN4#92 meeting [1], it was agreed to conduct simulation studies to evaluate measurement bandwidth of CSI-RS for RRM measurement requirement. And the simulation assumption for CSI-RS based measurement is agreed in [2].

In RAN4 #96e meeting [3], it was agreed that the CSI-RS based L3 measurement requirements in R16 are defined for CSI-RS configuration {D=3 with PRBs ≥ 48}.

Based on the latest updated simulation assumption [4], in this contribution, we provide the simulation results for CSI-SINR measurement.

- Discussion

2.1 Simulation results for accuracy requirements

In this section, we provide the simulation results of delta SINR under AWGN and EPA5 channel for 15 kHz and 30 kHz and provide the simulation results of delta SINR under TDL-A and TDL-C channel for 60kHz and 120kHz. Table 2.1-1 to table 2.1-8 provide the delta SINR based on different samples, different transmission power which is indicated by SINR for 15kHz, 30kHz and 60kHz, respectively. The simulations are based on the CSI-RS configuration {D=3, PRB = 48}.

For FR1：

15 kHz case:

Table 2.1-1: Delta SINR, SINR=-5.97 dB, td= CP, 15 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.29 | -0.16 | 1.09 | 1.13 | 1.25 | 1.25 |
|  | 5 | -1.05 | -0.16 | 0.86 | 0.89 | 1.02 | 1.02 |
|  | 10 | -0.76 | -0.16 | 0.60 | 0.6 | 0.76 | 0.76 |
| EPA5 | 3 | -1.55 | -0.16 | 1.33 | 1.39 | 1.49 | 1.49 |
|  | 5 | -1.18 | -0.16 | 0.97 | 1.02 | 1.13 | 1.13 |
|  | 10 | -0.94 | -0.16 | 0.66 | 0.78 | 0.82 | 0.82 |

Table 2.1-2: Delta SINR, SINR=-3.97 dB, td= CP, 15 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.13 | -0.13 | 0.96 | 1 | 1.09 | 1.09 |
|  | 5 | -0.92 | -0.13 | 0.76 | 0.79 | 0.89 | 0.89 |
|  | 10 | -0.66 | -0.13 | 0.51 | 0.53 | 0.64 | 0.64 |
| EPA5 | 3 | -1.34 | -0.10 | 1.14 | 1.24 | 1.24 | 1.24 |
|  | 5 | -1.02 | -0.10 | 0.84 | 0.92 | 0.94 | 0.94 |
|  | 10 | -0.80 | -0.10 | 0.58 | 0.7 | 0.68 | 0.7 |

30 kHz case:

Table 2.1-3: Delta SINR, SINR=-5.97 dB, td= CP, 30 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.24 | -0.17 | 0.96 | 1.07 | 1.13 | 1.13 |
|  | 5 | -1.07 | -0.17 | 0.66 | 0.9 | 0.83 | 0.9 |
|  | 10 | -0.75 | -0.17 | 0.47 | 0.58 | 0.64 | 0.64 |
| EPA5 | 3 | -1.74 | -0.25 | 1.45 | 1.49 | 1.7 | 1.7 |
|  | 5 | -1.47 | -0.25 | 1.01 | 1.22 | 1.26 | 1.26 |
|  | 10 | -1.07 | -0.25 | 0.66 | 0.82 | 0.91 | 0.91 |

Table 2.1-4: Delta SINR, SINR=-3.97 dB, td= CP, 30 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.10 | -0.14 | 0.87 | 0.96 | 1.01 | 1.01 |
|  | 5 | -0.92 | -0.14 | 0.58 | 0.78 | 0.72 | 0.78 |
|  | 10 | -0.67 | -0.14 | 0.42 | 0.53 | 0.56 | 0.56 |
| EPA5 | 3 | -1.43 | -0.18 | 1.24 | 1.25 | 1.42 | 1.42 |
|  | 5 | -1.21 | -0.18 | 0.87 | 1.03 | 1.05 | 1.05 |
|  | 10 | -0.86 | -0.18 | 0.57 | 0.68 | 0.75 | 0.75 |

For FR2：

60 kHz case:

Table 2.1-5: Delta SINR, SINR=-5.97 dB, td= CP, 60 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDL-A | 3 | -2.10 | -0.28 | 1.67 | 1.82 | 1.95 | 1.95 |
|  | 5 | -1.80 | -0.28 | 1.15 | 1.52 | 1.43 | 1.52 |
|  | 10 | -1.38 | -0.28 | 0.75 | 1.1 | 1.03 | 1.1 |
| TDL-C | 3 | -1.53 | -0.13 | 1.39 | 1.4 | 1.52 | 1.52 |
|  | 5 | -1.22 | -0.13 | 1.06 | 1.09 | 1.19 | 1.19 |
|  | 10 | -0.98 | -0.13 | 0.65 | 0.85 | 0.78 | 0.85 |

Table 2.1-6: Delta SINR, SINR=-3.97 dB, td= CP, 60 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDL-A | 3 | -1.73 | -0.23 | 1.35 | 1.5 | 1.58 | 1.58 |
|  | 5 | -1.45 | -0.23 | 0.97 | 1.22 | 1.2 | 1.22 |
|  | 10 | -1.11 | -0.23 | 0.63 | 0.88 | 0.86 | 0.88 |
| TDL-C | 3 | -1.30 | -0.07 | 1.18 | 1.23 | 1.25 | 1.25 |
|  | 5 | -1.01 | -0.07 | 0.89 | 0.94 | 0.96 | 0.96 |
|  | 10 | -0.82 | -0.07 | 0.59 | 0.75 | 0.66 | 0.75 |

120 kHz case:

Table 2.1-7: Delta SINR, SINR=-5.97 dB, td= CP, 120 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDL-A | 3 | -2.77 | -0.58 | 1.74 | 2.19 | 2.32 | 2.32 |
|  | 5 | -2.22 | -0.58 | 1.21 | 1.64 | 1.79 | 1.79 |
|  | 10 | -1.76 | -0.58 | 0.72 | 1.18 | 1.3 | 1.3 |
| TDL-C | 3 | -2.59 | -0.44 | 1.64 | 2.15 | 2.08 | 2.15 |
|  | 5 | -2.16 | -0.44 | 1.10 | 1.72 | 1.54 | 1.72 |
|  | 10 | -1.67 | -0.44 | 0.72 | 1.23 | 1.16 | 1.23 |

Table 2.1-8: Delta SINR, SINR=-3.97 dB, td= CP, 120 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDL-A | 3 | -2.27 | -0.38 | 1.53 | 1.89 | 1.91 | 1.91 |
|  | 5 | -1.82 | -0.38 | 1.09 | 1.44 | 1.47 | 1.47 |
|  | 10 | -1.39 | -0.38 | 0.63 | 1.01 | 1.01 | 1.01 |
| TDL-C | 3 | -2.10 | -0.29 | 1.48 | 1.81 | 1.77 | 1.81 |
|  | 5 | -1.79 | -0.29 | 1.01 | 1.5 | 1.3 | 1.5 |
|  | 10 | -1.37 | -0.29 | 0.69 | 1.08 | 0.98 | 1.08 |

According to the above simulation results, the following observations are given:

For absolute accuracy:

Observation 1: When the sample number is 5, the absolute measurement error can be within ±1.5dB for the Es/Iot = -5.97dB and Es/Iot = -3.97dB in FR1.

Observation 2: When the sample number is 5, the absolute measurement error can be within ±2.5dB for the Es/Iot = -5.97dB and within ±2dB for Es/Iot = -3.97dB in FR2.

For relative accuracy:

Observation 3: When the sample number is 5, the relative measurement error can be within ±1.5dB for the Es/Iot = -5.97dB and within ±1dB for Es/Iot = -3.97dB in FR1.

Observation 4: When the sample number is 5, the relative measurement error can be within ±2dB for the Es/Iot = -5.97dB and within ±1.5dB for Es/Iot = -3.97dB in FR2.

2.2 Simulation results for upper limit of side condition

To evaluate the upper limit of the side condition for CSI-SINR measurement, in this section, we provide the simulation results of delta SINR for 15 kHz and 30 kHz under the Es/Iot 10dB, 12dB and 15dB respectively.

For 15kHz：

Table 2.2-1: Delta SINR, SINR=10 dB, td= CP, 15 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -2.43 | -0.26 | 1.51 | 2.17 | 1.77 | 2.17 |
|  | 5 | -2.17 | -0.26 | 0.89 | 1.91 | 1.15 | 1.91 |
|  | 10 | -1.78 | -0.26 | 0.47 | 1.52 | 0.73 | 1.52 |
| EPA5 | 3 | -0.76 | -0.01 | 0.61 | 0.75 | 0.62 | 0.75 |
|  | 5 | -0.59 | -0.01 | 0.44 | 0.58 | 0.45 | 0.58 |
|  | 10 | -0.39 | -0.01 | 0.36 | 0.38 | 0.37 | 0.38 |

Table 2.2-2: Delta SINR, SINR=12 dB, td= CP, 15 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -4.01 | -0.51 | 1.69 | 3.5 | 2.2 | 3.5 |
|  | 5 | -3.40 | -0.51 | 0.90 | 2.89 | 1.41 | 2.89 |
|  | 10 | -2.17 | -0.51 | 0.50 | 1.66 | 1.01 | 1.66 |
| EPA5 | 3 | -0.69 | -0.02 | 0.56 | 0.67 | 0.58 | 0.67 |
|  | 5 | -0.55 | -0.02 | 0.41 | 0.53 | 0.43 | 0.53 |
|  | 10 | -0.36 | -0.02 | 0.32 | 0.34 | 0.34 | 0.34 |

Table 2.2-3: Delta SINR, SINR=15 dB, td= CP, 15 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -5.97 | -1.11 | 1.87 | 4.86 | 2.98 | 4.86 |
|  | 5 | -4.72 | -1.11 | 1.29 | 3.61 | 2.4 | 3.61 |
|  | 10 | -3.71 | -1.11 | 0.69 | 2.6 | 1.8 | 2.6 |
| EPA5 | 3 | -0.59 | 0.01 | 0.49 | 0.6 | 0.48 | 0.6 |
|  | 5 | -0.48 | 0.01 | 0.36 | 0.49 | 0.35 | 0.49 |
|  | 10 | -0.36 | 0.01 | 0.26 | 0.37 | 0.25 | 0.37 |

For 30kHz：

Table 2.2-4: Delta SINR, SINR=10 dB, td= CP, 30 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -2.83 | -0.38 | 1.25 | 2.45 | 1.63 | 2.45 |
|  | 5 | -2.38 | -0.38 | 0.94 | 2 | 1.32 | 2 |
|  | 10 | -2.08 | -0.38 | 0.42 | 1.7 | 0.8 | 1.7 |
| EPA5 | 3 | -0.54 | 0 | 0.50 | 0.54 | 0.5 | 0.54 |
|  | 5 | -0.40 | 0 | 0.41 | 0.4 | 0.41 | 0.41 |
|  | 10 | -0.25 | 0 | 0.30 | 0.25 | 0.3 | 0.3 |

Table 2.2-5: Delta SINR, SINR=12 dB, td= CP, 30 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -4.74 | -0.63 | 1.44 | 4.11 | 2.07 | 4.11 |
|  | 5 | -3.37 | -0.63 | 1.05 | 2.74 | 1.68 | 2.74 |
|  | 10 | -2.53 | -0.63 | 0.45 | 1.9 | 1.08 | 1.9 |
| EPA5 | 3 | -0.46 | 0 | 0.39 | 0.46 | 0.39 | 0.46 |
|  | 5 | -0.32 | 0 | 0.34 | 0.32 | 0.34 | 0.34 |
|  | 10 | -0.24 | 0 | 0.25 | 0.24 | 0.25 | 0.25 |

Table 2.2-6: Delta SINR, SINR=15 dB, td= CP, 30 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -6.12 | -1.44 | 1.85 | 4.68 | 3.29 | 4.68 |
|  | 5 | -5.49 | -1.44 | 1.26 | 4.05 | 2.7 | 4.05 |
|  | 10 | -3.99 | -1.44 | 0.45 | 2.55 | 1.89 | 2.55 |
| EPA5 | 3 | -0.34 | 0 | 0.53 | 0.34 | 0.53 | 0.53 |
|  | 5 | -0.24 | 0 | 0.25 | 0.24 | 0.25 | 0.25 |
|  | 10 | -0.18 | 0 | 0.18 | 0.18 | 0.18 | 0.18 |

According to the simulation results, when the Es/Iot of CSI-RS resources reach to a certain value, the accuracy of CSI-SINR measurement will degrade as the value of Es/Iot become larger under AWGN channel. And when the Es/Iot = 10dB, the measurement error indicated by delta SINR will increase to almost 3dB.

Observation 5：When the Es/Iot of CSI-RS resources reach to a certain value, the accuracy of CSI-SINR measurement will degrade as the value of Es/Iot increase.

Observation 6：The CSI-SINR measurement error indicated by delta SINR will reach to almost 3dB when the Es/Iot = 10dB.

- Conclusion

In this contribution, we provide the simulation results for CSI-SINR measurement, and provide the observations as follows:

Observation 1: When the sample number is 5, the absolute measurement error can be within ±1.5dB for the Es/Iot = -5.97dB and Es/Iot = -3.97dB in FR1.

Observation 2: When the sample number is 5, the absolute measurement error can be within ±2.5dB for the Es/Iot = -5.97dB and within ±2dB for Es/Iot = -3.97dB in FR2.

Observation 3: When the sample number is 5, the relative measurement error can be within ±1.5dB for the Es/Iot = -5.97dB and within ±1dB for Es/Iot = -3.97dB in FR1.

Observation 4: When the sample number is 5, the relative measurement error can be within ±2dB for the Es/Iot = -5.97dB and within ±1.5dB for Es/Iot = -3.97dB in FR2.

Observation 5：When the Es/Iot of CSI-RS resources reach to a certain value, the accuracy of CSI-SINR measurement will degrade as the value of Es/Iot increase.

Observation 6：The CSI-SINR measurement error indicated by delta SINR will reach to almost 3dB when the Es/Iot = 10dB.

- Reference

[1] R4-1910701, RAN4 #92 Meeting report, RAN4

[2] R4-1910949, Link-level simulation assumptions for CSI-RS based measurements, CATT

[3] Draft_MeetingReport_96-e__2020-08-31_rev3

[4] R4-2017230, Updated link-level simulation assumptions for CSI-RS based L3 measurements, CATT