3GPP TSG-RAN4 Meeting #98-e	R4-2100447

# Electronic meeting, Jan. 25-Feb.5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1480 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PRS configuration for test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Perf |  |  |  |  |  | Date: |  |  | 2021-1-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | PRS based measurement for positioning was introduced in 38.133. The PRS configuration for test cases needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the PRS configuration for positioning measurement in 38.133. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PRS configuration for positioning measurement is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.X, A.3.X.1, A.3.X.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.3.X	Configurations of PRS for positioning measurement

A.3.X.1	Configuration of PRS for positioning for FR1

A.3.X.1.1	TDD

Table A.3.X.1.1-1: PRS configuration for SCS=15kHz

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | PRS.1.1 TDD | PRS.1.2 TDD |
| Bandwidth |  | BW of Active BWPNote 1,2 |  |
| SCS | kHz | 15 |  |
| Comb size |  | 2 |  |
| PRS periodicity | slots | 160 |  |
| PRS RE offset |  | k0=0 |  |
| PRS slot offset | slots | 1 | 11 |
| First OFDM symbol in the slot |  | l0 = 5 |  |
| Number of OFDM symbols |  | 4 Note 2 |  |
| Resource repetition |  | 1 Note 2 |  |
| Resource repetition gap |  | N/A Note 2 |  |
| Resource muting |  | Disabled |  |
| PRS Start PRB |  |  |  | 0 |  |  |
| Note 1: BW of PRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |  |

Table A.3.X.1.1-2: PRS configuration for SCS=30kHz

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | PRS.2.1 TDD | PRS.2.2 TDD |
| Bandwidth |  | BW of Active BWPNote 1,2 |  |
| SCS | kHz | 30 |  |
| Comb size |  | 2 |  |
| PRS periodicity | slots | 320 |  |
| PRS RE offset |  | k0=0 |  |
| PRS slot offset | slots | 1 | 21 |
| First OFDM symbol in the slot |  | l0 = 5 |  |
| Number of OFDM symbols |  | 4 Note 2 |  |
| Resource repetition |  | 1 Note 2 |  |
| Resource repetition gap |  | N/A Note 2 |  |
| Resource muting |  | Disabled |  |
| PRS Start PRB |  |  |  | 0 |  |  |
| Note 1: BW of PRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |  |

A.3.X.2	Configuration of PRS for positioning for FR2

A.3.X.2.1	TDD

Table A.3.X.2.1-1: PRS configuration for SCS=120kHz

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | PRS.3.1 TDD | PRS.3.2 TDD |
| Bandwidth |  | BW of Active BWPNote 1,2 |  |
| SCS | kHz | 120 |  |
| Comb size |  | 2 |  |
| PRS periodicity | slots | 1280 |  |
| PRS RE offset |  | k0=0 |  |
| PRS slot offset | slots | 1 | 81 |
| First OFDM symbol in the slot |  | l0 = 5 |  |
| Number of OFDM symbols |  | 4 Note 2 |  |
| Resource repetition |  | 1 Note 2 |  |
| Resource repetition gap |  | N/A Note 2 |  |
| Resource muting |  | Disabled |  |
| PRS Start PRB |  |  |  | 0 |  |  |
| Note 1: BW of PRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |  |

<End of Change 1>