3GPP TSG-RAN WG4 Meeting #98-e                                                R4-2100477

Electronic Meeting, 25 Jan. – 05 Feb., 2021

Title: 	Initial performance evaluation for cell identification in NR FR2 HST scenario

Source: 	CATT

Agenda item:	11.7.4

Document for:	Discussion

Introduction

In this contribution, the initial link level simulation results are provided for cell identification in NR FR2 high speed scenario.

Simulation assumption

In RAN4#97-e meeting, five deployment scenarios are shown in [1]; the first one is chosen for simulation.

Ds = 800m Dmin = 10m

The simulation assumptions for cell identification are given in Table 1.

Table 1: Link Simulation Parameters for cell identification

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Carrier frequency | GHz | 28 | 28 |
| Subcarrier Spacing | kHz | 120 | 120 |
| BW | MHz | 100 | 100 |
| Number of RB |  | 66 | 66 |
| Power boosting for PSS and SSS | dB | 0 | 0 |
| SSB periodicity | ms | 20 | 20 |
| Data Modulation |  | QPSK | QPSK |
| Power boosting for Data and Control | dB | 0 | 0 |
| Frame Structure Type |  | TDD | TDD |
| CP Length |  | Normal | Normal |
| Eor/Ioc | dB | 6.4 | 7.25 |
| Number of Tx antennas |  | 1 | 1 |
| Relative Delay of 1st Path (synchronous) |  | 0 | CP/2 |
| Channel |  | AWGN, HST |  |

Simulation results

The simulation results of cell identification are shown in the tables below.

Table 2: The Cell identification results (SCS=120kHz)

|  | Detection ratio | Number of samples |
| --- | --- | --- |
| AWGN | 100% | 1 |
| HST 150km/h | 90.3% | 1 |
| HST 350km/h | Cannot meet 90% | - |

Reference

[1] R4-2017828, “WF on NR support for HST in FR2” Samsung, RAN4 Meeting #97-e