3GPP TSG-RAN WG4 Meeting # 98-e	R4-2100572

# Electronic Meeting, , Jan. 25-Feb. 5, 2021

Source: 	Skyworks Solutions, Inc.

Title: 	PC2 non-contiguous UL CA UE Architecture and MPR/A-MPR evaluation

Agenda Item:	11.2.2.5 - HPUE for TDD intra-band non-contiguous UL CA [NR_RF_FR1_enh-Core]

Document for:	Approval

# Introduction

In the RAN4#97 e-meeting a way forward on assumptions [1] for PC2 non-contiguous UL CA MPR/A-MPR was approved. This contribution discusses the transmitter architecture and provides the evaluation assumptions for n77 and n41 MPR and NS04 A-MPR.

# Discussion

## PC2 Architecture, SEM and ACLR

The same SEM/ACLR approach than for PC3 non-contiguous UL CA are used with only ACLR requirement being 1dB more stringent for PC2. UL CA cases being requested in [1] are n77(2A) and n41(2A) with support of the U.S. n77 and Asia n78 spectrums, at least up to 400MHz total transmit bandwidth shall be supported and thus 2 PC2 PA architecture needed like for PC3 2 PA approach. Each PA covering one CC.

Proposal for PC2 baseline architecture and requirements:

- PC3 non-contiguous UL CA SEM requirement applicable to PC2

- PC3 ACLR definition is applicable to PC2 with 31dB ACLR instead of 30 dB

- MPR and A-MPR values are derived from a two PC2 PAs and antennas each supporting one of the CC

When looking at the current specification status we find that the PC3 2xPA solution for non-contiguous UL CA has worse MPR than the single PA PC3 case for contiguous UL CA and almost the same as the single PA PC2 results presented in [4]. Table 1 compares a few cases and points at some inconsistencies.

Table 1: Different MPR/A-MPR per UL CA and Power class cases

| Case | PC | Arch. | NS | Req. | WC MPR /AMPR | source | comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PC3 Cont UL CA | 3 | 1x PC3 PA | 01 | -30dBm/MHz IM5 | 14 | R16 38.101-1 | based on outer 2 class C |
|  |  |  | 01-04 | -13dBm/MHz IM3 | 7 |  | based on outer 1 class C |
|  |  |  | 04 | -25dBm/MHz IM3 | 13 |  | should only be valid for Cont UL CA, but -30dBm/MHz IM5 is worse than -25dBm/MHz IM3 |
| PC3 Non-cont UL CA | 3 | 2x PC3 PA | 01 | -30dBm/MHz IM3 | 15 | R16 38.101-1 | 2PA 1dB worse than single PA – same as ENDC PC2? |
|  |  |  | 01-04 | -13dBm/MHz IM3 | 9 |  | 2PA 2dB worse than single PA - same as ENDC PC2? |
|  |  |  | 04 | -25dBm/MHz IM3 | ? |  | Needs update as n41(2A) UL is specified |
| PC2/ PC1.5 NC ENDC | 2/ 1.5 | 2x PC2 PA | 01 | -30dBm/MHz IM3 | 18*=15 | R16 38.101-3 | *the A-MPR is corrected by 3dB to account for the different reference for back-off. |
|  |  |  | 01-04 | -13dBm/MHz IM3 | 12*=9 |  |  |
|  |  |  | 04 | -25dBm/MHz IM3 | 15*=12 |  |  |
| PC2 Cont UL CA | 2 | 1x PC2 PA | 01 | -30dBm/MHz IM5 | 14 | R4-2100544 | IMD3 at -25dBm requires higher back-off than IMD5 at -30dBm which is logical Results for PC2 are OK for PC3 MPR where A-MPR needs 1.5dB difference |
|  |  |  | 01-04 | -13dBm/MHz IM3 | 7 |  |  |
|  |  |  | 04 | -25dBm/MHz IM3 | 15.5 |  |  |
| PC3 Non-cont UL CA | 2 | 2x PC3 PA | 01 | -30dBm/MHz IM3 | 8 | R4-2010301 | Results on 2xn77 PAs at low voltage. Each PA has 3dB back-off by default => -13dBm/MHz should be small Band 41 PA is worse than n77 |
|  |  |  | 01-04 | -13dBm/MHz IM3 | 0 |  |  |
|  |  |  | 04 | -25dBm/MHz IM3 | 7 |  |  |

Observations:

- PC3 contiguous UL CA -25dBm/MHz IM3 has lower back-off than -30dBm/MHz IM5 specifications which is not logical

- PC2 contiguous UL CA non-contiguous allocations back-off similar to PC3 but NS04 1.5dB worse than WC MPR

- 2xPC3 PA 1RB+1RB worst case back-off is 1-2dB worse than 1xPC3 PA equivalent

- PC3 non-contiguous UL CA MPR/NS04 AMPR is similar than PC2 non-contiguous ENDC which is not consistent

- R16 38.101-1 is missing NS04 A-MPR for 2xPC3 PA

Overall, it seems that a few Release 16 PC3 UL CA numbers are on the high side and our results are more aligned. NS04 2xPC3 PA A-MPR is still needed as the current A-MPR specification only covers single PC3 PA case.

Proposal 2 on consistency checks:

- While PC2 UL CA contiguous and non-contiguous UL CA cases are evaluated, the PC3 numbers should further be verified for consistency.

- Missing PC3 non-contiguous UL CA NS04 A-MPR for two PC3 PA architecture is evaluated (input exists in R4-2010301)

## MPR and A-MPR evaluation Assumptions

Updating the agreed the PC3 assumptions in [2] and similar test conditions than in PC3 counterpart study in [3]:

- Based on 2x26dBm PAs

- Antenna isolation is 10dB and 4dB post-PA losses

- Each PA calibrated for 31dBc ACLR at 26dBm with 20MHz 100RB0 DFT-s-OFDM QPSK waveform

- Equal PSD and Equal back-off power split

Scenarios evaluated:

- Since same MPR is targeted CP-OFDM is used in each carrier but both CP-OFDM and DFT-s-OFDM can be evaluated

- Worst case back-off  IMD3 at -13dBm/MHz and -30dBm/MHz for 1RB+1RB at 15kHz and 30kHz SCS for MPR with 31dBc ACLR and

- Worst case back-off  IMD3 at -13dBm/MHz and -25dBm/MHz for 1RB+1RB at 15kHz and 30kHz SCS for NS04 A-MPR

- 1RB+1RB separation of ~100, 200, 600MHz to cover variation across BW separation classes

- Other allocations sizes are recommended but the MPR vs allocation BW behavior from PC3 MPR can also be reused

- 20MHz channel 15kHz SCS and 40MHz channel 15kHz SCS with a gap of 20MHz (100MHz class and in gap ACLR)

- 40MHz channel 15kHz SCS and 40MHz channel 15kHz SCS with a gap of 120MHz (200MHz class)

- 100MHz channel 30kHz SCS and 100MHz channel 30kHz SCS with a gap of 400MHz (600MHz class)

Proposal 3 for PC2 MPR/A-MPR evaluation:

- Antenna isolation is 10dB and 4dB post-PA losses

- Usual PC2 calibration for each PA

- Equal PSD and Equal back-off power split

- The detailed list of scenarios above are used for PC2 non-contiguous UL CA MPR and NS04 A-MPR evaluation

# Conclusions

In this contribution, we discuss the baseline PA architecture and MPR/A-MPR evaluation assumptions for PC2 non-contiguous UL CA. We also provide a status of the different Release 16 ENDC and UL CA MPR/A-MPR and make the following proposals.

Proposal for PC2 baseline architecture and requirements:

- PC3 non-contiguous UL CA SEM requirement applicable to PC2

- PC3 ACLR definition is applicable to PC2 with 31dB ACLR instead of 30 dB

- MPR and A-MPR values are derived from a two PC2 PAs and antennas each supporting one of the CC

Proposal 2 on consistency checks:

- While PC2 UL CA contiguous and non-contiguous UL CA cases are evaluated, the PC3 numbers should further be verified for consistency.

- Missing PC3 non-contiguous UL CA NS04 A-MPR for two PC3 PA architecture is evaluated (input exists in R4-2010301)

Proposal 3 for PC2 MPR/A-MPR evaluation:

- Antenna isolation is 10dB and 4dB post-PA losses

- Usual PC2 calibration for each PA

- Equal PSD and Equal back-off power split

- The detailed list of scenarios above are used for PC2 non-contiguous UL CA MPR and NS04 A-MPR evaluation

# References

[1] RP-202799 WID revision: RF requirements enhancement for NR frequency range 1 (FR1), Huawei, RAN#90e

[2]	R4-2008132 WF on remaining issues on MPR and AMPR for intra-band non-contiguous CA, Skyworks, Qualcomm, Huawei, HiSilicon, Nokia, RAN4#95-e

[3] R4-2010301 [FR1ULCA] UE TX measurements and requirements for non-contiguous ULCA MPR and A-MPR, Skyworks Solutions Inc., RAN4#96-e

[4] R4-2100544 PC2 Class C UL CA UE Architecture and MPR&A-MPR, Skyworks Solutions Inc., RAN4#98-e