3GPP TSG-RAN WG4 Meeting # 98-e                	R4- 2100579

Electronic Meeting, Jan. 25-Feb. 5, 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	NR-U PRACH simulation results

Agenda item: 	7.1.7.4.4

Document for: 	Discussion

Introduction

This paper presents simulation results considering some of the previously agreed parameters in the outcome of RAN4 #97-e meeting on that topic [1].

Simulation results

Table 1 Simulations parameters for PRACH sequences in NR-U

| Parameter | Value |
| --- | --- |
| PRACH formats | A2, B2 and C2 |
| SCS | 15 kHz (LRA = 1151) and 30 kHz (LRA = 571) |
| Antenna configuration | 1x2 |
| Propagation conditions | AWGN TDLC300-100 TDLA30-10 |
| Frequency offset | 400Hz |
| Ncs | 164 for LRA=1151 and 190 for LRA=571 |
| Test metric | The false alarm probability < 0.1% The probability of detection > 99% |
| Carrier frequency | 5 GHz |
| Channel bandwidth | 20 MHz |

Table 2 Time error tolerance based on Table 8.4.2.1-1 of 38.104 [2].

| PRACH SCS | AWGN | TDLC 300 | TDLA 30Note1 |
| --- | --- | --- | --- |
| 15 kHz | 0.52 us | 2.03 us | 0.67 us |
| 30 kHz | 0.26 us | 1.77 us | 0.41 us |
| Note 1: The time error tolerance for TDLA 30 is not available in 38.104 [2] and was calculated as TA command resolution + delay for the second last tap of the channel model in Table G.2.1.1-2 of [2]. Refer to our discussion paper for details on this calculation [3] |  |  |  |

Table 3 Simulation results for PRACH in unlicensed bands

| PRACH format | SCS (LRA) | Propagation condition | Frequency offset (Hz) | SNR @ 1% Pmd |
| --- | --- | --- | --- | --- |
| Format A2 | 15kHz (1151) | AWGN | 400 | -23.4 |
|  |  | TDLC300-100 | 400 | -17.0 |
|  |  | TDLA30-10 | 400 | -17.5 |
|  | 30kHz (571) | AWGN | 400 | -20.6 |
|  |  | TDLC300-100 | 400 | -14.1 |
|  |  | TDLA30-10 | 400 | -13.7 |
| Format B2 | 15kHz (1151) | AWGN | 400 | -23.4 |
|  |  | TDLC300-100 | 400 | -17.0 |
|  |  | TDLA30-10 | 400 | -17.4 |
|  | 30kHz (571) | AWGN | 400 | -20.5 |
|  |  | TDLC300-100 | 400 | -14.1 |
|  |  | TDLA30-10 | 400 | -13.6 |
| Format C2 | 15kHz (1151) | AWGN | 400 | -23.3 |
|  |  | TDLC300-100 | 400 | -17.1 |
|  |  | TDLA30-10 | 400 | -17.4 |
|  | 30kHz (571) | AWGN | 400 | -20.6 |
|  |  | TDLC300-100 | 400 | -14.0 |
|  |  | TDLA30-10 | 400 | -13.6 |

Conclusion

In this contribution we provided PRACH simulation results for NR-U.

References

- R4-2017468	Way forward on PRACH demodulation requirements, Nokia, 3GPP TSG-RAN WG4 Meeting # 97-e

- TS 38.104 V16.6.0, NR; Base Station (BS) radio transmission and reception

- R4-2100578, PRACH Demodulation performance requirements for operation in unlicensed bands, Nokia, 3GPP TSG-RAN WG4 Meeting # 98-e