3GPP TSG RAN WG4 Meeting #98-e	R4-2100644

Electronic Meeting, 25 Jan. - 5 Feb., 2021

Agenda item:	9.7.2

Source:	SoftBank Corp.

Title:	TP update for TR 37.717-11-21: EN-DC_11_n28-n77

Document for:	Approval

1	Introduction

EN-DC of 1B LTE and 2B NR of DC_11_n28-n77 was approved in RAN#88 [1]. This TP is to update the REFSENS exception results for 2nd IMD and reflect the REFSENS exception from R4-2014316 and R4-2014686.

2	Text Proposal

[Unchanged Parts Skipped]

6.X	DC_11_n28-n77

6.X.1	Operating bands for DC

Table 6.X.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n28-n77 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  |  | n28 |  | 703 MHz |  | – |  | 748 MHz |  | 758 MHz |  | – |  | 803 MHz |  | FDD |  |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.X.2	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_11A_n28A-n77A | DC_11A_n28A DC_11A_n77A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_11A_n28A-n77(2A) | DC_11A_n28A DC_11A_n77A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.X.3	Co-existence studies

Based on the co-existence studies of DC_11A-n28A and DC_11A-n77A captured in TR 37.716-11-11 and 37.863-01-01,

- 3rd and 4th order IMD generated by dual uplink of Band 11 + Band n28 may also fall into own Rx of Band n77

- 3rd and 4th order IMD generated by dual uplink of Band 11 + Band n77 may also fall into own Rx of Band n28.

6.X.4	∆TIB and ∆RIB values

For DC_11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n28-n77 | 11 | 0.4 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n28-n77 | 11 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

6.X.5	MSD

As mentioned in 6.X.3, IMD3 of B11 and n28 to n77 Rx and IMD3 of B11 and n77 to n28 Rx need to be addressed for REFSENS relaxation. The following values are proposed based on [2]:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_11A_n28A-n77A DC_11A_n28A-n77(2A) | 11 | 1443 | 5 | 25 | 1491 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n77 | 3629 | 10 | 50 | 3629 | 17.5 | TDD | IMD3 |
|  | 11 | 1443 | 5 | 25 | 1491 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n77 | 3586 | 10 | 50 | 3586 | 11.2 | TDD | IMD4 |
|  | 11 | 1443 | 5 | 25 | 1491 | N/A | FDD | N/A |
|  | n77 | 3684 | 10 | 50 | 3684 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 15.8 | FDD | IMD3 |
|  | 11 | 1443 | 5 | 25 | 1491 | N/A | FDD | N/A |
|  | n77 | 3564 | 10 | 50 | 3564 | N/A | TDD | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 11.0 | FDD | IMD4 |

[Unchanged Parts Skipped]

7.  Reference

[1]  RP-201372  New WID on Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL), LG Electronics

[2]  R4-2014316 MSD results for new DC LTE x bands (xDL/1UL, x=1,2,3,4) + NR 2 bands (2DL/1UL) ban combinations in Rel-17, LG Electronics