3GPP TSG-WG RAN4 Meeting #98-e	R4-2100751

# Online, 25th January – 5th February, 2021

Source: 				Nokia, Nokia Shanghai Bell

Title: 				EIS requirements for n262 and TP to TR 38.847

Agenda Item: 	9.25.1.3 [NR_47GHz_Band -Core]

Document for:	Approval

# Introduction

A link budget analysis for power class 3 UE has been made in RAN4#97e and the way-forward on UE RF requirement for n262 has been agreed in [1]. In this contribution, we propose to finalize the UE EIS requirements; TP to TR 38.847 is also proposed.

# Discussion

##### Power class 3 REFSENS

The link budget analysis for REFENS of the power class 3 is summarized in the following table. The range is -75.98 dBm and -81.5 dBm for the channel bandwidth 100 MHz.

|  |  | R4-2014263 Qualcomm | R4-2015855 Sony, Ericsson | R4-2015888 Intel | R4-2016229 vivo | R4-2016296 Apple |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | Value |  |  |  |  |
| Band number |  | n262 |  |  |  |  |
| Frequency range | GHz | 47.2 – 48.2GHz |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| SNR requirement | dB | -1 | 0 including 1 dB IM | -1 | -1 | -1 |
| Bandwidth | MHz | 100 | 100 | 50 | 100 | 50 |
| Thermal noise | dBm/Hz |  | -174 | -174 | -174 | -174 |
| Noise Figure | dB | 12.1 | 12.5 | 12 | 16 | 16 |
| Number of antenna in an array |  | 4 | 4 |  | 4 | 4 |
| Array gain | dB | 0.4 | 5.5 | 7.5 (effective) | 8.5 (realized) | 8.3 (realized) |
| Element gain | dBi |  | 4.5 |  | 4 | 3.75 |
| Diversity gain | dB | 2 chains | 0 |  | 0 | 0 |
| Antenna gain roll-off over frequency | dB |  | 0.5 |  | -1.5 | -1.5 |
| Beamforming loss | dB |  | 1 |  | -1 | -2 |
| Total insertion loss | dB |  | 8 |  | -9 | -11.3 |
| [Mismatch and transmission line loss] | dB |  |  |  | -2 | -3.3 |
| [Form factor Integration losses] | dB |  |  |  | -6 | -6 |
| [Implementation loss] | dB | 3 |  | 10.3 |  |  |
| [Packaging loss] | dB | 4.6 |  |  |  |  |
| REFSENS (50 MHz) | dBm |  |  | -83.2 |  | -78.98 |
| REFSENS (100 MHz) | dBm | -81.5 | -81.5 |  | -78.5 | -75.98 |
| REFSENS (200 MHz) | dBm |  |  |  |  | -72.98 |
| REFSENS (400 MHz) | dBm |  |  |  |  | -69.98 |

While discussing how to conclude the PC3 REFSENS requirement, two values, -79.3 dBm and -79.9 dBm, are suggested based on the arithmetic mean and logarithmic mean. There is no real technical justification which means to be used, especially when we only have 5 samples.

We have proposed to use the arithmetic mean for EIRP to target a higher EIRP in our companion paper, the same arithmetic mean is proposed for EIS, i.e., -79.3 dBm.

Proposal 1: REFSENS requirement for n262 power class 3 UE is -79.3 dBm at 100 MHz channel bandwidth.

##### Power class 3 EIS spherical coverage

The EIS spherical coverage requirement for existing FR2 bands are listed in the following table. The EIS drop between REFSENS and 50%-tile EIS is 10.9-12.9 dB, which is the same as EIRP drop. If the EIRP performance drop is large, the network performance cannot be guaranteed anymore. At least similar level as n259 is required. Therefore, it is proposed the drop is no more than 10.9 dB

|  | n257 | n258 | n259 | n260 | n261 | Proposal for n262 |
| --- | --- | --- | --- | --- | --- | --- |
| REFSENS (dBm) | -85.3 | -85.3 | -81.7 | -82.7 | -85.3 | -79.3 |
| EIS spherical coverage (dBm) | -74.4 | -74.4 | -68.9 | -70.1 | -74.4 | -68.4 |
| EIS drop (dB) | 10.9 | 10.9 | 12.9 | 12.6 | 10.9 | 10.9 |

Proposal 2: EIS spherical coverage requirement for n262 power class 3 UE is based on +10.9 dB from REFSENS.

##### Multiband relaxation for EIS requirement

The multi-band relaxations for power class 3 UE specified in TS 38.101-2 are summarized in the following. The relaxations of n259 and n260 for REFSENS and EIS spherical coverage are 0.5 and 0.4 dB, respectively. 28 GHz bands (n257, n258 and n261) have larger relaxation due to better EIS requirement. For n262, where EIS requirement is even worse than 39 GHz bands (n259 and n260), a larger relaxation should not be needed.

Proposal 3: Multi-band relaxation values for n262 are the same as n259/n260.

UE multi-band relaxation factors for power class 3

| Band | MBP,n (dB) | MBS,n (dB) |
| --- | --- | --- |
| n257 | 0.73 | 0.73 |
| n258 | 0.6 | 0.7 |
| n259 | 0.5 | 0.4 |
| n260 | 0.51 | 0.41 |
| n261 | 0.52,4 | 0.74 |
| Proposed values for n262 | 0.5 | 0.4 |
| Note 1: n260 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n260 Note 2: n261 peak relaxation is 0 dB for UE that exclusively supports n261+n260 Note 3: n257 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 Note 4: n261 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 |  |  |

##### EIS requirements for other power classes

There was no link budget analysis for n262 other than the power class 3 UE in RAN4#97e. The following table lists the PC1/2/4 EIS requirement specified in TS 38.101-2.

|  |  | n257 | n258 | n259 | n260 | n261 | Proposal for n262 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PC1 | min peak | -94.5 | -94.5 |  | -91.5 | -94.5 | -89.5 |
| PC1 | spherical | -86.5 | -86.5 |  | -83.5 | -86.5 | -81.5 |
| PC2 | min peak | -89.0 | -89.0 |  |  | -88.5 | -84.5 |
| PC2 | spherical | -78.0 | -78.0 |  |  | -78.0 | -74.0 |
| PC4 | min peak | -94.0 | -94.0 |  | -92.0 | -94.0 | -90.0 |
| PC4 | spherical | -85.0 | -85.0 |  | -80.0 | -85.0 | -78.0 |

Some degradation from the existing F2 bands may be expected for n262 as analysed for PC3, where about 2 dB from 40GHz band n259 is estimated in the link budget analysis of EIS. For PC1/2/4, form factor constraint is not as stringent as PC3, we propose to assume 2 dB from n260 or 4 dB from n257/n258/n261. The proposed values are also listed in the above table.

Proposal 5: REFSENS and EIRS spherical coverage requirements for n262 are derived assuming 2 dB degradation from n260 or 4 dB from n257/258/n261.

# Conclusion

In this contribution EIS requirement for n262 UE has been discussed. The following proposal is made.

Proposal 1: REFSENS requirement for n262 power class 3 UE is -79.3 dBm at 100 MHz channel bandwidth.

Proposal 2: EIS spherical coverage requirement for n262 power class 3 UE is based on +10.9 dB from REFSENS.

Proposal 3: Multi-band relaxation values for n262 are the same as n259/n260.

Proposal 5: REFSENS and EIRS spherical coverage requirements for n262 are derived assuming 2 dB degradation from n260 or 4 dB from n257/258/n261.

# Reference

[1] R4-2016879	WF on UE RF requirement of n262	Qualcomm, Nokia, Sony

[2] R4-2014263	Discussion on PC3 EIRP and EIS in n262	Qualcomm Incorporated

[3] R4-2015855	Link budget for PC3 for n262	Sony, Ericsson

[4] R4-2015888	PC3 minimum peak EIRP and EIS requirements for band n262	Intel Corporation

[5] R4-2016229	EIRP and EIS evaluation for band n262	vivo

[6] R4-2016296	Peak EIRP and Peak EIS for band n262	Apple Inc.