3GPP TSG- RAN WG4 Meeting #98-e	R4-2100818

# Electronic Meeting, Jan. 25-Feb. 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0131 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS38.101-4, test for FR1 TDD PDCCH DCI format 2_6 demodulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-Perf |  |  |  |  |  | Date: |  |  | 2021-1-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | PDCCH-WUS and PDCCH joint test and test requirements have been agreed to be defined in RAN4#97-e meeting. Demodulation performance requirement for power saving UE needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The requirements for demodulation performance for power saving UE in FR1 TDD are introduced in section 5.3.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The demodulation performance for power saving will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.4 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

5.3.4	Requirements for Power Saving UE

The PDCCH demodulation test in this section is applicable for UEs declared to support power saving. During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

5.3.4.2	TDD requirements

The parameters specified in Table 5.3.4.2-1 are valid for all TDD tests unless otherwise stated.

Table 5.3.4.2-1: Test Parameters

| Parameter | Unit | 1 Tx Antenna |
| --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |
| CCE to REG mapping type |  | interleaved |
| Interleaver size |  | 3 |
| REG bundle size |  | 2 |
| Shift Index |  | 0 |
| DRX cycle | ms | 10 |
| ps-WakeUp-r16 |  | absent |
| ps-PositionDCI-2-6-r16 |  | ‘1’ |

5.3.4.2.1	2RX requirements

For the parameters specified in Table 5.3.4.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 5.3.4.2.1-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.4.2.1-1: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.0 |
|  |  |  |  | TBD | TBD |  |  |  |  |
| Note: UE that already fulfil this test can skip the test case#2 in subclause 5.3.2.2.1 |  |  |  |  |  |  |  |  |  |

5.3.4.2.2	4RX requirements

For the parameters specified in Table 5.3.4.2-1, For the parameters specified in Table 5.3.4.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 5.3.4.2.2-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.4.2.2-1: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x4 Low | 1 | -0.9 |
|  |  |  |  | TBD | TBD |  |  |  |  |
| Note: UE that already fulfil this test can skip the test case#2 in subclause 5.3.3.2.1 |  |  |  |  |  |  |  |  |  |

<< End of Changes >>