3GPP TSG-RAN WG4 Meeting # 98-e 										R4-2100830

Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda item:	11.11.2.2

Source: 	CMCC

Title: 	emission related conducted requirements for repeaters

Document for:	Discussion

1. Introduction

In last RAN plenary #90 meeting, a new WID on NR repeaters is approved to specify RF and EMC requirements for both FDD and TDD bands. To enhancement cellular network coverage, different types of network nodes have been considered to offer blanket coverage, among which RF repeater is regarded as one kind of simplest and cost-effective node. In this contribution, we focus on emission related requirements analysis for repeater.

2. Discussion

In [1], it is suggested to seek the leveraging of RF requirements between IAB and LTE repeater. Following table lists emission related conducted requirements to show distinguishes.

|  | IAB-DU | IAB-MT | LTE repeater |
| --- | --- | --- | --- |
| Transmitted signal quality |  |  |  |
| Frequency error | Identical to BS | Differential measurement | More stringent |
| EVM | Identical to BS | Similar to BS, IAB-MT channels and EVM measurement procedure is still under discussion | Only one value, don’t distinguish modulation scheme |
| Carrier leakage In-band emission EVM equalizer spectrum flatness | No | No | No |
| Time alignment error | Identical to BS | NO | NO |
| Unwanted emissions |  |  |  |
| Occupied bandwidth | Identical to BS | Identical to BS (no MR) | NO |
| Adjacent Channel Leakage Power Ratio | Identical to BS | FR1: Identical to BS (no MR) FR2: new definition | NO |
| ACPR | NO | NO | New definition When coexist with UTRAN: 33dB and 22dB clarified by output power level No requirements when coexist with E-UTRAN |
| Out of band gain | NO | NO | New definition |
| Operating band unwanted emissions | Identical to BS | Identical to BS (no MR) | Little modification to adapt variable pass band bandwidth |
| Transmitter spurious emissions |  |  |  |
| Mandatory requirements | Identical to BS | Identical to BS (no MR) | Identical to BS |
| Protection of the BS receiver of own or different BS | Identical to BS | Identical to BS (no MR) | Relaxed requirement reflecting current state of art technology |
| Additional spurious | Identical to BS | Identical to BS (no MR) | Identical to BS |
| Co-location with other BS/IAB | Identical to BS | Identical to BS (no MR) | Identical to BS |
| Transmitter intermodulation | Identical to BS | Identical to BS (no MR) | Identical to BS |
| Receiver intermodulation | Identical to BS | Identical to BS (no MR) | The increased REFSENSE and interference signal type are all different |

All the requirements for NR BS still apply for IAB-DU. For IAB-MT, except for frequency error, ACLR and EVM measurement requirements, all remaining conducted signal quality requirements and unwanted emission requirements of BS still apply for.

For LTE repeater, new out-of-band gain and ACPR requirements are added and almost all requirements except for regulatory requirements have been re-analysed and specified.

Proposal 1: we suggest to involve the following emission requirements for repeater as the start point. It is noted all the regulatory requirements of BS could be reused.

- Transmitted signal quality, including Frequency error and EVM while IBE, carrier leakage and EVM equalizer spectrum flatness requirements could be eliminated.

- Unwanted emissions, including ACPR, Out of band gain, Operating band unwanted emissions

- Transmitter spurious emissions, including Mandatory requirements, Protection of the BS receiver of own or different BS, Additional spurious and Co-location requirements with other BS/repeaters

- Transmitter intermodulation

- Receiver intermodulation

3. Conclusions

In this contribution, emission related conducted requirements are discussed with following proposal:

Proposal 1: we suggest to involve the following emission requirements for repeater as the start point. It is noted all the regulatory requirements of BS could be reused.

- Transmitted signal quality, including Frequency error and EVM while IBE and EVM equalizer spectrum flatness requirements could be eliminated.

- Unwanted emissions, including ACPR, Out of band gain, Operating band unwanted emissions

- Transmitter spurious emissions, including mandatory requirements, Protection of the BS receiver of own or different BS, Additional spurious and Co-location requirements with other BS/repeaters

- Transmitter intermodulation

- Receiver intermodulation

4. Reference

[1] RP-202927, New WID on NR Repeaters