3GPP TSG-RAN WG4 Meeting # 98-e	R4-2100840

Electronic Meeting, Jan. 25-Feb. 5, 2021

Source:	ZTE Corporation

Title:	On test cases for handover under NR-U

Agenda item:	7.1.6.3.3

Document for:	Discussion

# 1	Introduction

In RAN4 97-e, the first meeting to discuss performance part of NR-U, companies has some preliminary discussions on the scope of test cases, test configurations and the work plan. During the meeting, whether to define test cases for some of the scenarios of handover was discussed. Companies had different views on the scope test cases while some consensus was captured in the WF [1] as shown below.

Issue 3-3-3: Handover (delay and interruption) test cases For Handover, at least the following test cases are to be specified in NR-U - Intra-frequency handover from NR-U to NR-U; known target cell - Intra-frequency handover from NR-U to NR-U; unknown target cell - Inter-frequency handover from NR-U to NR-U; unknown target cell - Inter-RAT handover from E-UTRAN (FDD and TDD) to NR-U Other cases discussed in RAN4 97e: 5. Inter-frequency handover from NR-U to NR-U; known target cell 6. Inter-frequency handover from NR to NR-U; known target cell 7. Inter-frequency handover from NR to NR-U; unknown target cell 8. Inter-RAT handover from NR-U to E-UTRAN (FDD, TDD) 9. Inter-frequency handover from NR-U to NR; known target cell 10. Inter-frequency handover from NR-U to NR; unknown target cell

- This paper provides our view on this topic.

2	Discussion

For handover, some companies suggested not to define test cases for case 5-7. The proposal was backed by the argument that cases 5-7 can be functionally verified by the cases 1-3 which already reflect the UE capability and essential functions and if the UE can pass the tests for cases 1-3 then the UE is assumed to be able to meet requirements for cases 5-7 as well, and thus, no need to specify test cases for cases 5-7.

- We don’t agree with the above argument and think that test cases for cases 5-7 shall be specified.

If the UE can be assumed to meet requirements for cases 5-7 conditioned that the UE passes tests for cases 1-3, then it doesn’t mean much burden for the UE to also be tested for cases 5-7 since no extra implementation is required.

To guarantee that the UEs can meet all requirements, tests are more promising than analysis. Thus, we prefer to define test cases for all scenarios corresponding to core requirements. This is also the common practice for R15 UEs.

Define test cases for all scenarios corresponding to core requirements for handover in NR-U.

3	Conclusion

Proposal 1: Define test cases for all scenarios corresponding to core requirements for handover in NR-U.

References

- R4-2017089