3GPP TSG-RAN WG4 Meeting #98-e              	R4-2100924

Electronic meeting, 25th Jan – 5th Feb, 2021

Agenda item:	7.18.2

Source:	Samsung

Title:	Simulation results for BS 2-step RACH requirement

Document for:	Discussion

# 1	Introduction

In the last RAN#97-e meeting, the general issues with demodulation requirement for NR 2-step RACH was discussed and the related simulation assumption for requirement was captured in the WF [1].

In this contribution, the view on the remaining issue is provided. Meanwhile, the updated simulation results are provided for requirement derived.

# 2	Discussion

During the previous meeting, it was agree to define the requirement with 2 PRBs for each SCS level, without specified the related CBW. For BS conformance test, the AWGN level at the BS input should be matched with given CBW.

Proposal 1: Specify MsgA PUSCH requirement with 2PRB within the minimum CBW per each SCS as

- 15 kHZ SCS/5 MHz BW, 30 kHz SCS/10 MHz

- 60 kHz SCS/50 MHz BW, 120 kHz SCS/50 MHz

And set AWGN level at the BS input as

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | [5] | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | [10] | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| BS type 2-O | 60 | [50] | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52MHz |
|  | 120 | [50] | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and clause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

# 3	Simulation Results

In this section, based on agreed simulation assumption, the updated simulation results are provided in the table 1 and table 2

Table 1:  FR1 requirement

| SCS/BW | MCS | Tx/Rx | DMRS | Symbol length | Mapping type | Channel | To range | SNR @ 1% BLER (ideal) | SNR@ 1% BLER (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1+1 (2,7,11) | 14 | Type A | TDLC 300-100 | [0:0.2:3.8] | 5.19 | 7.19 |
| 15KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1(2,11) | 14 | Type A | TDLC 300-100 | [0:0.2:3.8] | 5.26 | 7.26 |
| 30KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1+1 (2,7,11) | 14 | Type A | TDLC 300-100 | [0:0.1:2] | 5.04 | 7.04 |
| 30KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1(2,11) | 14 | Type A | TDLC 300-100 | [0:0.1:2] | 5.21 | 7.21 |
| 15KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1+1 (0,5,10) | 14 | Type B | TDLC 300-100 | [0:0.2:3.8] | 5.27 | 7.27 |
| 15KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1(0,10) | 14 | Type B | TDLC 300-100 | [0:0.2:3.8] | 5.24 | 7.24 |
| 30KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1+1 (0,5,10) | 14 | Type B | TDLC 300-100 | [0:0.1:2] | 5.11 | 7.11 |
| 30KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1(0,10) | 14 | Type B | TDLC 300-100 | [0:0.1:2] | 5.14 | 7.14 |

Table 1:  FR2 requirement

| SCS/BW | MCS | Tx/Rx | DMRS | Symbol length | Mapping type | Channel | To range | SNR @ 1% BLER (ideal) | SNR @ 1% BLER (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60KHz, 2PRB | MCS 2 from table 1 | 1T2R | 1+1(0,8) | 10 | Type B | TDLA 30-300 | [0:0.1:0.6] | 6.43 | 8.43 |
| 120 KHz, 2PRB | MCS 2 from table 1 | 1T2R | 1+1(0,8) | 10 | Type B | TDLA 30-300 | [0:0.1:0.5] | 6.02 | 8.02 |

# 4 Conclusion

In this contribution, the updated simulation results are provided to finalize the MsgA PUSCH requirement.

Proposal 1: Specify MsgA PUSCH requirement with 2PRB within the minimum CBW per each SCS as

- 15 kHZ SCS/5 MHz BW, 30 kHz SCS/10 MHz

- 60 kHz SCS/50 MHz BW, 120 kHz SCS/50 MHz

And set AWGN level at the BS input as

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52MHz |
|  | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and clause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

# References

- [1] R4-2017580 WF on BS demodulation requirements for 2-step RACH, ZTE