3GPP TSG-RAN WG4 Meeting #98e		R4-2101256

Electronic, 25th January – 5th February, 2021

Agenda item:	7.9.4.2.1

Source:	Intel Corporation

Title:	Simulation results for single-DCI based multi-TRP SDM Tx scheme

Document for:	Discussion

# Introduction

In RAN4 #97e meeting WF on PDSCH demodulation requirements and general aspects of NR eMIMO Rel-16 was agreed[1]. All test parameters for single-DCI based SDM Tx scheme were finalized. In this paper we provide simulation results for corresponding scenario.

# Discussion

## Simulation results

In Figures 1-2 simulation results for agreed single-DCI based SDM multi-TRP Tx scenarios are provided for FDD and TDD duplex modes for test case 1a and 2b. The simulation assumptions can be found in [2]. The summary of demodulation performance at 70% at max achievable throughput is provided in Table 1 for ideal and impairment results as well.

| Test case 1a: Frequency offset + Negative Time offset |  |
| --- | --- |
| Figure 1. Demodulation performance for test case 1a |  |

| Test case 1b: Positive Time offset |  |
| --- | --- |
| Figure 2. Demodulation performance for test case 1b |  |

Table 1. Demodulation performance at 70% @max achievable throughput

|  | Duplex mode | Antenna configuration | Ideal results, dB | Impairment results, dB |
| --- | --- | --- | --- | --- |
| Test case 1a | FDD | 2x2 | 15.2 | 17.7 |
|  |  | 2x4 | 8.9 | 11.4 |
|  | TDD | 2x2 | 14.9 | 17.4 |
|  |  | 2x4 | 8.9 | 11.4 |
| Test case 1b | FDD | 2x2 | 16.6 | 19.1 |
|  |  | 2x4 | 10 | 12.5 |
|  | TDD | 2x2 | 16 | 18.5 |
|  |  | 2x4 | 9.6 | 12.1 |

# Conclusion

In this paper we have provided simulation results to define performance requirements for single-DCI based SDM Tx scheme.

# References

- R4-2017529 “WF on NR eMIMO PDSCH requirement”, Intel, RAN4 #97-e, November 2020

- R4-2017530 “Simulation assumption for PDSCH requirement with single-DCI and Multi-DCI transmission schemes”, Ericsson, Intel, RAN4 #97-e, November 2020