3GPP TSG-RAN WG4 Meeting # 98-e 	     													 R4-2101271

Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda Item:	11.5.2.3

Source:	Intel Corporation

Title:	Discussion on NCSG in NR

Document for:		Discussion

# Introduction

In this contribution we consider the following objective in NR measurement gap enhancement work item [1].

- Network Controlled Small Gap (NCSG) specification [RAN4, RAN2] - RRM requirements for NCSG [RAN4] - Requirements for Visible Interruption Length (VIL) for different numerologies in FR1 and FR2 - Specification of NCSG patterns, Measurement Length (ML), and Visible Interruption Repetition Period (VIRP) - Requirements for DL reception and UL transmission during ML, before start VIL and after end VIL - Measurement requirements with NCSG - Specification of applicability of NCSG patterns [RAN4] - Procedures and signaling for NCSG patterns [RAN2]

# Discussion

Firstly, before RAN4 discusses the specific RRM requirements and applicability of NR NCSG patterns, it is better to clarify the possible and feasible using scenarios of NCSG in NR.

Figure 1. An example of NR NCSG using scenario

In our views, in NR the measurements on the deactivated SCell will cause interruption to other active carriers as required in 8.2.1.2.5	of TS38.133 [2]. If UE can support multiple RF chains one of which can be used for the measurement for the deactivated carrier as illustrated in Figure 1 above, the network can schedule a small gap for this measurement. That is the main task of NCSG in NR can be:

- Enable measurement on unused RF chains with interruption controlled on activated CC

- Eliminate/reduce interruptions on the serving carriers due to deactivated SCell measurement

Proposal 1: Similar as in LTE, RAN4 can prioritize the following NR NCSG using scenario:

    - Eliminate/reduce interruptions to the serving carriers due to RF chain states transition when measuring the deactivated SCells

Secondly, regarding introduction of NCSG, another critical issue is the feasible NCSG patterns definition including Visible Interruption Length (VIL) , Measurement Length (ML), and Visible Interruption Repetition Period (VIRP) as defined in LTE [3, TS36.133].

- Visible Interruption Length (VIL)

In principle, how to decide VIL depends on the feasible RF retuning time which could introduce the unexpected interruption to the serving carriers/cells. As in NR Rel15, the assumption on the duration of RF retuning in LTE/NR PCell/PSCell can be less than 500us and 250us for FR1 and FR2 respectively. In our view, in Rel17 the same assumption can be reused.

In addition to the interruption in case of both synchronous and asynchronous DC are different in MR DC, the VIL in NR NCSG for the different FR can be defined as:

|  | VIL in Sync (ms) | VIL in Async(ms) |
| --- | --- | --- |
| FR1 | 1 | 2 |
| FR2 | 0.75 |  |

- Measurement Length (ML)

Measurement length is highly dependent on measurement object configuration. For an example, for SSB measurement gap, ML is up to SMTC configuration. Hence ML defined in NR legacy gap patterns [2, TS38.133] can be feasible.

- Visible Interruption Repetition Period (VIRP)

VIRP can rely on the SCell measurement cycle configure for SCell. And regarding to such NCSG will be applicable for all SCell carriers, if there are multiple deactivated SCells to be measured, more than one NCSG are needed. That is the VIRP can be less than measCycleSCell. Meanwhile, it is better to align these SMTC window for multiple SCells.

In short words, it is straightforward to reuse some of the legacy MG patterns in [2] as the new NCSG patterns in NR.

Table 9.1.2-1: Gap Pattern Configurations [2,TS 38.133]

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |
| 24 | 10 | 80 |
| 25 | 20 | 160 |

On the other hand, from the technical perspective, some of these legacy patterns are infeasible, which are also highlighted in the table above. For an example, the gap patterns which have the quite shorter MGRP/VIRP (e.g. 20ms), in case of SCell measurement with “measCycleSCell” which shall be larger than 160 slots [4], it seems no necessary to defined the same NCSG. And for gap#24, 25 which is used for PRS measurement, can be not in the candidate NCSG patterns.

MeasObjectNR ::= SEQUENCE { ssbFrequency ARFCN-ValueNR OPTIONAL, -- Cond SSBorAssociatedSSB ssbSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond SSBorAssociatedSSB smtc1 SSB-MTC OPTIONAL, -- Cond SSBorAssociatedSSB smtc2 SSB-MTC2 OPTIONAL, -- Cond IntraFreqConnected …… measCycleSCell ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280} OPTIONAL -- Need R ]], [[ smtc3list-r16 SSB-MTC3List-r16 OPTIONAL, -- Need R rmtc-Config-r16 SetupRelease {RMTC-Config-r16} OPTIONAL, -- Need M t312-r16 SetupRelease { T312-r16 } OPTIONAL -- Need M ]] }

And if MGRP in [2] can be reused as the VIRP of NCSG in NR, the length (ML) will can be “MGL- 2*VIL”.

Proposal 2: In order to minimize RAN4 and other RAN group’s standardization efforts, RAN4 can define  NR NCSG patterns based on existing NR legacy MG patterns in [2].

In the table below, the several new NCSG patterns based on the legacy gap pattern(e.g. MGRP) are defined.

Table 2: Gap Pattern Configurations for NCSG

| Gap Pattern Id | VIL1 (MGL, ms) | ML | VIL2 | Measurement Gap Repetition Period (VIRP, ms) |
| --- | --- | --- | --- | --- |
| ncsg0 | 1, sync 2 , async | 6 - 2*1, sync 6 - 2*2, async | 1, sync 2 , async | 40 |
| ncsg 1 | 1, sync 2 , async | 6 - 2*1, sync 6 - 2*2, async | 1, sync 2 , async | 80 |
| ncsg 2 | 1, sync | 3 - 2*1, sync | 1, sync | 40 |
| ncsg 3 | 1, sync | 3 - 2*1, sync | 1, sync | 80 |
| ncsg 5 | 1, sync 2 , async | 6 - 2*1, sync 6 - 2*2, async | 1, sync 2 , async | 160 |
| ncsg 7 | 1, sync | 4 - 2*1, sync | 1, sync | 40 |
| ncsg 8 | 1, sync | 4 - 2*1, sync | 1, sync | 80 |
| ncsg 9 | 1, sync | 4 - 2*1, sync | 1, sync | 160 |
| ncsg 11 | 1, syn | 3 - 2*1, sync | 1, sync | 160 |
| ncsg 13 | 0.75, sync +async | 5.5 - 2*0.75, | 0.75, sync +async | 40 |
| ncsg 14 | 0.75, sync +async | 5.5 - 2*0.75, | 0.75, sync +async | 80 |
| ncsg 15 | 0.75, sync +async | 5.5 - 2*0.75, | 0.75, sync +async | 160 |
| ncsg 17 | 0.75, sync +async | 3.5 - 2*0.75, | 0.75, sync +async | 40 |
| ncsg 18 | 0.75, sync +async | 3.5 - 2*0.75, | 0.75, sync +async | 80 |
| ncsg 19 | 0.75, sync +async | 3.5 - 2*0.75, | 0.75, sync +async | 160 |

Thirdly, if NCSG is used for the measurement for UE who can perform with multiple RF chains, the RRM requirements with NCSG shall be investigated. For an example, the interruption requirements shall be updated if NCSG is used. (e.g. interruption for SCell activation in 8.2.1.2.5.1 of TS38.133[2] and 7.32.2.6 in TS36.133)

Proposal 3: The interruption requirements during measurements on SCC defined in TS38.133 and TS36.133 shall be revisited because of NCSG is used.

As stated in [1], UE measurement via the NCSG required RAN2 to design the necessary signaling support. Regarding to in Rel16 “Needforgap” signaling was introduced [4]. In our view, this signaling can indicate the gap for each band/carrier. The similar signaling of needForGap can be applicable for NR NCSG also.

Proposal 4: The “NeefForGap” signaling structure can be reused for NR NCSG as a start point.

# Conclusion

In this contribution, serval issues related to the measurement gap enhancement WI are discussed. The proposals can be drawn as:

Proposal 1: Similar as in LTE, RAN4 can prioritize the following NR NCSG using scenarios:

    - Eliminate/reduce interruptions to the serving carriers due to RF chain states transition when measuring the deactivated SCells

Proposal 2: In order to minimize RAN4 and other RAN group’s standardization efforts, RAN4 can define NR NCSG patterns based on existing NR legacy MG patterns in [2].

Proposal 3: The interruption requirements during measurements on SCC defined in TS38.133 and TS36.133 shall be revisited because of NCSG is used.

Proposal 4: The “NeedForGap” signaling structure can be reused for NR NCSG as a start point.

# References

- RP-202658 Revised WID on NR and MR-DC measurement gap enhancements

- TS38.133 v16.6.0

- TS36.133 v16.5.0

- TS38.331 v16.2.0