3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2101408

# Electronic Meeting, 25th, Jan.–5th, Feb., 2021

Agenda item:      7.13.1.2

Source:                Intel Corporation

Title:                    Discussion of RRC based BWP switching on multiple CCs

Document for:    Discussion

# Introduction

In last meeting, there is one open issue for RRC based BWP switching[1]:

- Simultaneous case or simultaneous + partial Sub1: For RRC based BWP switching - Postpone defining testcases with RRC based simultaneous BWP switch and clarify the scenario first.

In this contribution, we will continue to discuss the remaining issues.

# RRC based BWP switching

In companied contribution[2], we shared our view regarding RRC based BWP switch on single CC. We understand that it’s better to wait for the reply of RAN2, since it will have impact on the Rel-16 performance part which is assumed to be finished in this meeting. Due to the limited time, we prefer that RRC based BWP switching on a single CC apply for SpCell only.

If RRC based BWP switching is not applicable for SCell, there is no scenario for RRC based simultaneous BWP switching on multiple CCs.

Proposal 1: There is no scenario for RRC based simultaneous BWP switching on multiple CCs. Don’t need to design test case for the scenario.

For non-simultaneous case, since the spec already outlines that the requirement apply for NR-DC(FR1+FR2) case, we can further clarify that the requirement apply if there is only one CC in either FR1 or FR2 cell group.

#### 8.6.3A.2 Non-simultaneous RRC based BWP switch delay on multiple CCs In non-simultaneous case, the RRC-based BWP switch on multiple CCs is triggered over partially overlapping time period n different Cell groups. The delay requirements in this clause apply only if: BWP switching on multiple CCs in different cell groups are triggered by separate RRC commands, and UE is operating in NR-DC (FR1+FR2), and UE is capable of per-FR gap, and BWP switch does not involve SCS change. There is only one CC in either PCell or PSCell.

The previous delay time for non-simultaneous RRC based BWP switch on multiple CC is:

TWaiting+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRC*(M-1)NR slot length

if M=1, then the delay time can be further modified to:

TWaiting+TRRCprocessingDelay+TBWPswitchDelayRRCNR slot length

We provide corresponding CR in[3].

Proposal 2: For non-simultaneous RRC based multiple BWP switching case, clarify that the requirement apply if there is only one CC in either PCell or PSCell.

Proposal 3: Delay time for non-simultaneous RRC based BWP switch on multiple CC will be updated to:

TWaiting+TRRCprocessingDelay+TBWPswitchDelayRRCNR slot length

# Conclusion

In this paper we provide our views on the remaining open issues related to BWP switching on multiple CCs. Our proposals are captured below:

Proposal 1: There is no scenario for RRC based simultaneous BWP switching on multiple CCs. Don’t need to design test case for the scenario.

Proposal 2: For non-simultaneous RRC based multiple BWP switching case, clarify that the requirement apply if there is only one CC in either PCell or PSCell.

Proposal 3: Delay time for non-simultaneous RRC based BWP switch on multiple CC will be updated to:

TWaiting+TRRCprocessingDelay+TBWPswitchDelayRRCNR slot length

# Reference

[1] R4-2017362, WF on NR RRM enhancements - BWP switching on multiple CCs and UL spatial info switch, Intel

[2] R4-2101407, Discussion of RRC based BWP switching on single CC, Intel

[3] R4-2101409, CR on RRC based BWP switching on multiple CCs (Rel-16), Intel