3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2101874

# Electronic Meeting, 25th Jan- 5th Feb., 2021

Title: 	on V2X HPUE power class signaling

Source: 	Xiaomi

Agenda item:	11.10.5

Document for:	Approval

Background

The HPUE power class signaling issue has been raised in RAN4#96-e meeting in [1]. Since currently work plan is trying to introduce PC2 for Sidelink enhancement, the power class issue needs to be solved at this stage. In this paper, we try to give some analysis of LTE Sidelink PC2 signaling and provide some way forward to solve the PC2 signaling issue for V2X HPUE.

Discussion

During the discussion in Rel-16 period, it is agreed in [1] that:

However, considering the PC2 introduction is under processing in Rel-17 WID, we believe the signaling issue needs to be finalized. The current problem is power class may be different for V2X UE supporting PC3 on each antenna connector but can support PC2 when summing up the power of two antennas. In this time, as the capability signaling define only one IE “ue-PowerClass” as per band basis for different kind of UE.

If we look back to LTE V2X, in TS 36.306, a power class capability signaling IE “v2x-HighPower-r14” is defined as:

Observation 1: IE “v2x-HighPower-r14” is specifically used to indicate V2X sidelink PC2 for LTE

This IE is specifically used to indicate V2X sidelink PC2 for LTE. However, this IE is not defined in current TS 38.306 yet.

Proposal 1: To define IE for PC2 V2X UE for V2X sidelink transmission in a band.

Furthermore, the on-going discussion for different signaling for one antenna port or two antenna ports is also under discussion in main forum. This is not a specific V2X question supporting SL-MIMO or TXD. The background can go back to [2] as it is agreed that:

This is similar issue for SL-MIMO and currently the another WF [3] has been agreed as below:

From the WF agreement we can see there is still on-going discussion and new signaling is one of the options as option 1b. Hence, before the discussion in main forum has settled down, it is recommended to postpone the new signaling discussion.

Proposal 2: To wait for main forum to settle down the new capability signaling discussion.

Conclusions

In this contribution, we give discussion on remaining issue on V2X HPUE signaling and the observations are shown as below：

Observation 1: IE “v2x-HighPower-r14” is specifically used to indicate V2X sidelink PC2 for LTE

Proposal 1: To define IE for PC2 V2X UE for V2X sidelink transmission in a band.

Proposal 2: To wait for main forum to settle down the new capability signaling discussion.

Reference

- R4-2011848	Email discussion summary for [96e][108] 5G_V2X_NRSL_UE_RF, Moderator (LG Electronics)

- R4-2005652	WF on Uplink Full Power Transmission, Samsung

- R4-2016830	Way Forward on NR TxD & Power Class, VIVO