3GPP TSG-RAN WG4 Meeting #98-e	R4-2102064

Electronic Meeting, 25 January – 5 February 2021

Source:	Ericsson, Rogers

Title:	TP to TR 37.717.11-21: Addition of DC configurations for DC_12_n2-n38

Agenda item:	9.7.2

Document for:	Approval

# Background

This text proposal for TR 37.717-11-21 [1] to add DC_12_n2-n38 configurations as defined in WID [2].

# Text Proposal

##### ---Start of changes---

## 6.75	DC_12_n2-n38

### 6.75.1	Operating bands for DC

Table 6.75.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n2-n38 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

### 6.75.2	Channel bandwidths per operating band for DC

Table 6.75.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_12A_n2A-n38A | DC_12A_n2A DC_12A_ n38A | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 70 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 6.75.3	Co-existence studies

Based on co-existence studies, there are no IMD generated by dual uplink of 12_n38 into own Rx of band n2.

Based on co-existence studies, IMD2 generated by dual uplink of 12_n2 may fall into own Rx of band n38.

### 6.75.4	∆TIB and ∆RIB values

For DC_12_n2-n38, the TIB,c and RIB,c values are derived from DC_2_n41-n71 and are given in the tables below.

Table 6.75.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n2-n38 | 12 | 0.3 |
|  | n2 | 0.5 |
|  | n38 | 0.5 |

Table 6.75.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12_n2-n38 | 12 | 0 |
|  | n2 | 0 |
|  | n38 | 0 |

### 6.75.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 12 and NR band n2 that impact the reference sensitivity of NR band n38.

The required MSD levels and test configuration are shown in the following Table.

Table 6.75.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n2A-n38A | 12 | IMD2 | ¦fn2 +fB12¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 |  |
|  | n38 |  |  | 2608 | 5 | 25 | 2608 | 5 | 28.7 |

##### ---End of changes---

# Reference

[1]		R4-2014304, TR37.717-11-21 v0.2.0 for DC LTE x Bands DL_1UL (x=1,2,3,4) and 2 NR bands DL_1UL basket WI_r2, LG Electronics

[2]		RP-202292, Revised WID on DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) in Rel-17, LG Electronics