3GPP TSG-RAN WG4 Meeting #98-e	R4-2102079

Electronic Meeting, 25 January – 5 February 2021

Source:	Ericsson, Rogers

Title:	TP to TR 37.717.11-21: Addition of DC configurations for DC_66_n2-n66

Agenda item:	9.7.2

Document for:	Approval

# Background

This text proposal for TR 37.717-11-21 [1] to add DC_66_n2-n66 configurations as defined in WID [2].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_66_n2-n66

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n2-n66 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n2A-n66A | DC_66A_ n2A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

Based on co-existence studies, IMD5 generated by dual uplink of 66_n2 may fall into own Rx of band n66.

### 6.x.4	∆TIB and ∆RIB values

For DC_66_n2-n66, the TIB,c and RIB,c values are derived from DC_66_n66 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n2-n66 | 66 | 0.5 |
|  | n2 | 0.5 |
|  | n66 | 0.5 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n2-n66 | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n66 | 0.3 |

### 6.x.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD5 generated by dual uplink of 66_n2 may fall into own Rx of band n66. MSD of DC_2A_n66A is reused.

The required MSD levels and test configuration are shown in the following Table.

Table 6.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n2A-n66A | B66 | IMD5 | ¦2*fB66 -3*fn2¦ | 1720 | 5 | 25 | 2120 | 5 | N/A |
|  | n2 |  |  | 1870 | 5 | 25 | 1950 | 5 | N/A |
|  | n66 |  |  | 1770 | 5 | 25 | 2170 | 5 | 4.0 |

##### ---End of changes---

# Reference

[1]		R4-2014304, TR37.717-11-21 v0.2.0 for DC LTE x Bands DL_1UL (x=1,2,3,4) and 2 NR bands DL_1UL basket WI_r2, LG Electronics

[2]		RP-202292, Revised WID on DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) in Rel-17, LG Electronics