3GPP TSG-RAN WG4 Meeting #98-e	R4-2102169

# Electronic meeting, January 25th – February 5th, 2021

Source:		Ericsson

Title:			New band n67 – BS RF impacts

Agenda item:		9.28.2

Document for:		Discussion

# Introduction

In last RAN#90-e, a new WI has been approved to specify the new NR band n67, re-farmed from LTE band 67.

This contribution is discussing the core BS RF impacts when introducing this band.

# Discussion

## Presentation

Band n67 is a SDL band. It corresponds to the DL part of band n28.

It shall support following channel bandwidths:

- 5 MHz channel bandwidth and for 15 kHz subcarrier spacing.

- 10, 15 and 20 MHz channel bandwidth and for 15 kHz and 30 kHz subcarrier spacing.

## General

### Band definition

The band n67 shall be added to list of NR operating bands in FR1, table 5.2-1 as below:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n67 | N/A | 758 MHz – 803 MHz | SDL |

### BS channel bandwidth

Following channel BWs shall be added to table 5.3.5-1:

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| n67 | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Channel arrangement

NR-ARFCN for n67 shall be added in table 5.4.2.3-1:

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n67 | 100 | N/A | 151600 – <20> – 160600 |

And sync raster information shall be added to table 5.4.3.3-1:

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n67 | 15 kHz | Case A | 1901 – <1> – 2002 |

## Tx requirements

### OBUE

Band n67 shall be added to the list of bands mentioned for Wide Area cat B option 1 limits for bands below 1GHz, in clause 6.6.4.2.2:

For BS operating in Bands n5, n8, n12, n20, n26, n28, n29, n67, n71, the basic limits are specified in table 6.6.4.2.2.1-1:

### Spurious

Band n67 limits shall also be added to the coexistence spurious emissions limits, table 6.6.5.2.3-1:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 28 or NR Band n28 | 758 – 803 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n20, n67 or n28. |
|  | 703 – 748 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in band n67, it applies for 703 MHz to 736 MHz. |
| E-UTRA Band 67 or NR band n67 | 738 – 758 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n28. |
| NR Band n83 | 703 – 748 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n67, it applies for 703 MHz to 736 MHz. |

## Rx requirements

As n67 is SDL band, there is no Rx requirement applicable for that band (the blocking requirement for co-located BS is now band agnostic).

There is so no impact on Rx requirement.

# Conclusion

In this contribution, we analyzed the BS RF impacts when introducing band n67. The corresponding updates shall be done in TS 38.141-1 as well.

Proposal: Agree with the listed changes in this contribution.

# References

- RP-202279, New WID on introduction of n67, Ericsson, Telefonica