3GPP TSG-RAN WG4 Meeting #98-e	R4-2102171

# Electronic meeting, January 25th – February 5th, 2021

Source:		Ericsson

Title:			New band n85 – BS RF impacts

Agenda item:		9.29.2

Document for:		Discussion

# Introduction

In last RAN#90-e, a new WI has been approved to specify the new NR band n85, re-farmed from LTE band 85.

This contribution is discussing the core BS RF impacts when introducing this band.

# Discussion

## Presentation

The NR band n85 is re-farmed E-UTRA band 85.

The WI objectives are:

- Specify a new NR FDD operating band n85 (the uplink band for this new band is 18 MHz, 698-716 MHz; the downlink band is 18 MHz, 728-746 MHz; the duplex gap is 12 MHz, 716-728 MHz) to support subcarrier spacing of 15 kHz for 5 MHz channel bandwidth, and subcarrier spacing of 15 kHz and 30 kHz for 10 MHz channel bandwidth.

- Specify RF characteristics of the new band

- Specify requirements for the new band with assumption that the UE supporting band n85 can also support band n12.

## General

### Band definition

The band n85 shall be added to list of NR operating bands in FR1, table 5.2-1 as below:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n85 | 698 MHz – 716 MHz | 728 MHz – 746 MHz | FDD |

Moreover, n85 shall be added to the list of NR bands supporting NB-IoT:

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n41, n65, n66, n70, n71, n74, n85, n90 which are defined in Table 5.2-1.

### BS channel bandwidth

Following channel BWs shall be added to table 5.3.5-1:

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
| n85 | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Channel arrangement

NR-ARFCN for n85 shall be added in table 5.4.2.3-1:

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n85 | 100 | N/A | 145600 – <20> – 149200 |

And sync raster information shall be added to table 5.4.3.3-1:

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n85 | 15 kHz | Case A | 1826 – <1> – 1858 |

## Tx requirements

### Output power

In TS 36.104, there is a power limitation for NB-IoT standalone BS operating in band 85. This limitation is of course still valid for NB-IoT BS standalone operating in band n85. But as NB-IoT standalone requirements are only specified in TS 36.104, this power limitation doesn’t have to be copied in TS 38.104.

### OBUE

Band n85 shall be added to the list of bands mentioned for Wide Area cat A and cat B option 1 limits for bands below 1GHz, in clauses 6.6.4.2.1 and 6.6.4.2.2:

For BS operating in Bands n5, n8, n12, n13, n14, n18, n26, n28, n29, n71, n85, basic limits are specified in table 6.6.4.2.1-1.

For BS operating in Bands n5, n8, n12, n20, n26, n28, n29, n71, n85, the basic limits are specified in table 6.6.4.2.2.1-1:

### Spurious

Band n85 limits shall also be added to the coexistence spurious emissions limits, table 6.6.5.2.3-1:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| UTRA FDD Band XII or | 729 – 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n12 or n85. |
| E-UTRA Band 12 or NR Band n12 | 699 – 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n12 or n85, since it is already covered by the requirement in clause 6.6.5.2.2. For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). |
| E-UTRA Band 29 or NR Band n29 | 717 – 728 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n29 or n85. |
| E-UTRA Band 85 or NR band n85 | 728 - 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 12, 29 or 85. |
|  | 698 - 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band n85, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band n29, it applies 1 MHz below the Band n29 downlink operating band. |

And Note 5 shall be updated:

NOTE 5:	For NR Band n29 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band.

Band n85 limits shall also be added to the colocation spurious emissions limits, table 6.6.5.2.4-1:

| Type of co-located BS | Frequency range for | Basic limits |  |  | Measurement | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  | co-location requirement | WA BS | MR BS | LA BS | bandwidth |  |
| E-UTRA Band 85 or NR Band n85 | 699 – 716 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |

## Rx requirements

The blocking requirements for co-located BS are now band agnostic.

There is so no impact on Rx requirement.

# Conclusion

In this contribution, we analyzed the BS RF impacts when introducing band n85. The corresponding updates shall be done in TS 38.141-1 as well.

Proposal: Agree with the changes listed in this contribution.

# References

- RP-202830, New WID on introduction of NR band n85, T-Mobile USA, Ericsson