3GPP TSG-RAN WG4 #98e	R4-2102244

Electronic Meeting, January 25 –February 5, 2021

Title: 					Discussions on cell reselection test cases for NR-U

Source: 				Ericsson

Agenda item:	7.1.6.3.2

Document for:	Discussion

# Introduction

In the agreed list of test cases for release 16 NR-U following test cases were proposed for RRC_IDLE state cell reselection requirements [1]:

| Group of requirements | Test cases |  | Requirements section | Agreed | Volunteer | Phase I | Phase II |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | NR-U -> NR-U | intra-frequency | 4.2A | Yes | Ericsson |  |  |
|  |  | inter-frequency |  | Yes |  |  |  |
|  | NR(FR1) -> NR-U |  |  | FFS |  |  |  |
|  | NR-U -> NR(FR1) |  |  | FFS |  |  |  |
|  | NR-U - > E-UTRAN (FDD,TDD) |  |  | FFS |  |  |  |
|  | E-UTRAN (FDD,TDD) -> NR-U |  | TS 36.133 | Yes |  |  |  |

In this contribution we provide views on testing of the reselection requirements for the above test cases.

# Discussion

Scenarios

The NR-U cell reselection requirements for standalone operational scenario were introduced in 4.2A.2.2, 4.2A.2.3, 4.2A.2.4 and 4.2A.2.5 in [2].  These requirements are applicable for following types of cell changes:

- intra-frequency NR cells when subject to CCA on both serving and target cell

- inter-frequency NR cells when subject to CCA on the target cell

- inter-RAT E-UTRAN cells when subject to CCA on the serving cell

It is noted that only NR-U to NR-U and E-UTRAN to NR-U test cases were agreed in [1] and others under discussion. In addition, following types of cell changes are also supported from an operational point of view and therefore RAN4 is proposed to develop corresponding test cases: NR-U to E-UTRAN, NR-U to NR (FR1) and NR (FR1) to NR-U.

- Proposal #1: RAN4 to agree on test cases for following types of cell reselections:

  - NR-U  E-UTRAN,

  - NR-U  NR (FR1)

  - NR (FR1) o NR-U

Based on the agreed test cases and above proposal, a change request containing following types of cell reselections are presented in in [3]:

- A.11.1.1		Cell re-selection with both source and target NR carrier frequencies under CCA

  - Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell

  - Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell

- A.11.1.2		Cell re-selection to NR with serving NR carrier frequency under CCA

  - Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA

- A.11.1.3		Cell re-selection from NR carrier with target NR carrier frequency under CCA

  - Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA

- A.11.1.4		Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA

  - Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA

  - Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA

Test configurations

The test configurations depend on the type cell reselection the UE is performing, e.g. whether it is from NR-U to NR-U or from NR-U to NR, etc. The different types of configurations that are needed to suppose the cell reselection types in proposal 1 are shown in Table 1, Table 2 and Table 3 were used.

For cell change from NR-U to NR-U:

| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| --- | --- |

Table 1 Configuration for cell change from NR-U to NR-U

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table 2 Configuration for cell change from NR-U to NR

| Configuration | Description of a cell without CCA | Description of a cell with CCA |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table 3 Configuration for cell change from NR to NR-U

- Proposal #2: The standalone cell reselection test cases are defined based on the test configurations shown in Table 1, Table 2 and Table 3.

The cell which is subject to CCA shall contain LBT configuration that specifies the probability of LBT failure. Depending on type of cell reselection the source or target cell (or both) need LBT configuration in the test case. Examples of LBT configuration that needs to be defined include:

- DL CCA model

- UL CCA model

- DBT Window Configuration

- DL CCA probability PCCA_DL

- UL CCA probability PCCA_UL

In addition, PDSCH reference measurement channel, RMSI CORESET and dedicated CORESET need to be defined differently and aligned throughout all other test cases.

- Proposal #3: Cell specific test parameters should contain following new or modified parameters to account for the LBT impact:

  - DL CCA model

  - UL CCA model

  - DBT Window Configuration

  - DL CCA probability PCCA_DL

  - UL CCA probability PCCA_UL

  - New RMCs

Test requirements:

The overall cell reselection procedure from legacy NR was reused for NR-U with some modification to account for the CCA impact. The CCA aspect of the core requirements with respect to the legacy requirements are highlighted below.

- cell detection delay (e.g. Tdetect,NR_Intra_CCA) is extended by Mm to account for the CCA failures that could occur during the detection attempts

- Measurement periodicity (e.g. Tmeasure,NR_Intra_CCA) is extended by Mm to account for the CCA failures that could occur during the measurement attempts

- Evaluation period (e.g. Tevaluate,NR_Intra_CCA) is extended by Me to account for the CCA failures that could occur during the evaluation period

Parameters Md, Mm and Me are limited by their corresponding maximum values Md,max, Mm,max and Me,max respectively and are defined as follows:

- Mm,max = [16] for DRX cycle length = 0.32s;

- Mm,max = [8] for DRX cycle length = 0.64s;

- Mm,max = [4] for DRX cycle length = 1.28s;

- Mm,max = [4] for DRX cycle length = 2.56s.

The relation between Mm,max, Me,max and Mm,max is as follows:

- Md,max=[4]*Mm,max, Me,max=[2]*Mm,max

The UE is required to restart the measurements when the number of CCA failures exceeds its maximum values. This is an important aspect of CCA that needs to be verified in the reselection test case.

- Proposal #4: Reselection test shall verify that maximum allowed CCA failures for Md, Mm and Me.

In addition, two measurement attempts are allowed for a cell which is already identified. Upon exceeding the two attempts the UE is required to trigger the cell detection process on any of the configured carriers. This particular requirement is difficult to verify in the reselection test case which is focused on a target cell on a particular carrier.

- Observation #1: Not possible to verify the requirement that UE shall trigger cell detection on all configured carriers after two unsuccessful measurement attempts.

Another new aspect is related to the spacing between two measurements used in the filtering. For NR-U two measurements shall be spaced by at least Tmeasure,NR_Intra_CCA/2. This requirement is also difficult to verify using the cell reselection test, however the requirement can be implicitly verified since the measurement delay is verified.

- Observation #2: Not possible to verify the requirement on minimum spacing between two measurements used in the filtering.

Based on these proposals and observations we provide test case draft. Other aspects of the test may need to be updated depending on the outcome of discussion on configuration aspects and common settings for NR-U test cases.

# Conclusions

In this contribution we discuss the cell reselection testing for NR-U and have made following proposal/observations:

- Observation #1: Not possible to verify the requirement that UE shall trigger cell detection on all configured carriers after two unsuccessful measurement attempts.

- Observation #2: Not possible to verify the requirement on minimum spacing between two measurements used in the filtering.

- Proposal #1: RAN4 to agree on test cases for following types of cell reselections:

  - NR-U  E-UTRAN,

  - NR-U  NR (FR1)

  - NR (FR1) o NR-U

- Proposal #2: The standalone cell reselection test cases are defined based on the test configurations shown in Table 1, Table 2 and Table 3.

- Proposal #3: Cell specific test parameters should contain following new or modified parameters to account for the LBT impact:

  - DL CCA model

  - UL CCA model

  - DBT Window Configuration

  - DL CCA probability PCCA_DL

  - UL CCA probability PCCA_UL

  - New RMCs

- Proposal #4: Reselection test shall verify that maximum allowed CCA failures for Md, Mm and Me.

# References

- R4-2017089, “WF on NR-U RRM performance”, Nokia, Nokia Shanghai Bell

- TS 38.133 V16.5.0

- R4-2102243, “Introduction of NR-U cell reselection tests”, Ericsson