3GPP TSG-RAN WG4 Meeting #98-e	R4- 2102283

Online, 25 Jan – 5 Feb 2020

Source: 	Huawei, HiSilicon

Title:	Consideration on adding PC 1.5 for n77 and n78

Agenda Item:	9.34.1

Document for:	Approval

# 1	Introduction

In RAN#90-e, a WI [1] was agreed to add power class 1.5 to band n77 and n78 in Rel-17. The necessary changes needed in the UE specification TS 38.101-1 are discussed in this paper.

# 2	Discussion

The Power Class 1.5 has been introduced for band n41 in Rel-16. As per the discussions [2] in RAN#90-e, the same assumptions such as 10 dB antenna isolation and dual PA/2Tx architecture should apply for band n77 and n78 PC 1.5. Hence the existing RF requirements for PC 1.5 should be re-used. The changes needed are for adding n77 and n78 to the relevant clauses. More explicitly, the following changes are identified.

********************Begin of Changes *****************

### 6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  |  |  | 23 | ±2 |
| n2 |  |  |  |  |  |  | 23 | ±23 |
| n3 |  |  |  |  |  |  | 23 | ±23 |
| n5 |  |  |  |  |  |  | 23 | ±2 |
| n7 |  |  |  |  |  |  | 23 | ±23 |
| n8 |  |  |  |  |  |  | 23 | ±23 |
| n12 |  |  |  |  |  |  | 23 | ±23 |
| n14 | 31 | +2/-3 |  |  |  |  | 23 | ±23 |
| n18 |  |  |  |  |  |  | 23 | ±2 |
| n20 |  |  |  |  |  |  | 23 | ±23 |
| n25 |  |  |  |  |  |  | 23 | ±23 |
| n26 |  |  |  |  |  |  | 23 | ±23 |
| n28 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  |  |  | 23 | ±2 |
| n34 |  |  |  |  |  |  | 23 | ±2 |
| n38 |  |  |  |  |  |  | 23 | ±2 |
| n39 |  |  |  |  |  |  | 23 | ±2 |
| n40 |  |  |  |  | 26 | +2/-33 | 23 | ±2 |
| n41 |  |  | 295 | _2/-33 | 26 | +2/-33 | 23 | ±23 |
| n47 |  |  |  |  |  |  | 23 | ±2 |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  |  |  | 23 | ±2 |
| n51 |  |  |  |  |  |  | 23 | ±2 |
| n53 |  |  |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  |  |  | 23 | ±2 |
| n66 |  |  |  |  |  |  | 23 | ±2 |
| n70 |  |  |  |  |  |  | 23 | ±2 |
| n71 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n74 |  |  |  |  |  |  | 23 | ±2 |
| n77 |  |  | 295 | _2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n78 |  |  | 295 | _2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  |  |  | 23 | ±2 |
| n81 |  |  |  |  |  |  | 23 | ±2 |
| n82 |  |  |  |  |  |  | 23 | ±2 |
| n83 |  |  |  |  |  |  | 23 | ±2/-2.5 |
| n84 |  |  |  |  |  |  | 23 | ±2 |
| n86 |  |  |  |  |  |  | 23 | ±2 |
| n89 |  |  |  |  |  |  | 23 | ±2 |
| n91 |  |  |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  |  |  | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB NOTE 5: Achieved via dual Tx |  |  |  |  |  |  |  |  |

************************** End of Changes *********************

**************************Begin of Changes ********************

## 6.2D	Transmitter power for UL MIMO

### 6.2D.1	UE maximum output power for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1 ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | +2/-3 |  |  |
| n2 |  |  |  |  | 23 | +2/-31 |  |  |
| n3 |  |  |  |  | 23 | +2/-31 |  |  |
| n7 |  |  |  |  | 23 | +2/-31 |  |  |
| n25 |  |  |  |  | 23 | +2/-31 |  |  |
| n30 |  |  |  |  | 23 | +2/-3 |  |  |
| n34 |  |  |  |  | 23 | +2/-3 |  |  |
| n38 |  |  |  |  | 23 | +2/-3 |  |  |
| n39 |  |  |  |  | 23 | +2/-3 |  |  |
| n40 |  |  |  |  | 23 | +2/-3 |  |  |
| n41 | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n48 |  |  |  |  | 23 | +2/-3 |  |  |
| n66 |  |  |  |  | 23 | +2/-3 |  |  |
| n70 |  |  |  |  | 23 | +2/-3 |  |  |
| n71 |  |  |  |  | 23 | +2/-3 |  |  |
| n77 | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

************************** End of Changes *********************

Proposal 1: Add Power Class 1.5 to band n77 and n78 in Table 6.2.1-1 and Table 6.2D.1-1 as shown above.

Furthermore, the MPR for Power Class 1.5 with dual Tx has been defined in Clause 6.2.2. This should be reused for band n77 and n78. No changes in the spec are needed.

Proposal 2: Reuse the existing MPR requirements in Clause 6.2.2 for band n77 and n78 PC 1.5.

Meanwhile, no additional spectrum emission requirements have been identified so far for band n77 or n78. Hence A-MPR is not needed for band n77 and n78 PC 1.5.

Proposal 3: No A-MPR is needed for band n77 or n78 PC 1.5.

In terms of fulfilling the regulatory SAR requirements, the power reduction mechanism as described in Clause 6.2.4 Configured Transmitter Power (e.g., P-MPR or maxUplinkDutyCycle-PC2-FR1) can be reused. No changes in the spec are needed.

Proposal 4: Reuse the existing power reduction mechanism in Clause 6.2.4 for band n77 and n78 PC 1.5 in order to fulfil the regulatory SAR requirements.

# 3	Conclusion

This paper analyses the changes to the UE spec TS 38.101-1 that are needed for adding PC 1.5 to band n77 and n78. And the proposals are summarised as below.

Proposal 1: Add Power Class 1.5 to band n77 and n78 in Table 6.2.1-1 and Table 6.2D.1-1 as shown above.

Proposal 2: Reuse the existing MPR requirements in Clause 6.2.2 for band n77 and n78 PC 1.5.

Proposal 3: No A-MPR is needed for band n77 or n78 PC 1.5.

Proposal 4: Reuse the existing power reduction mechanism in Clause 6.2.4 for band n77 and n78 PC 1.5 in order to fulfil the regulatory SAR requirements.

# References

[1] RP-202912, New WID on high-power devices operation use cases over Band n77 and n78, RAN#90-e

[2] RP-202905, Email Summary on Introduction of high-power devices operation use cases over Band n77, RAN#90-e