3GPP TSG-WG4 Meeting #98-e	R4-2102386

# Electronic Meeting, Jan. 25-Feb. 5, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0687 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-1: correction of Pi/2 BPSK |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2021-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There was no evaluation of Pi/2 BPSK with new DMRS for intra-band CA in Rel-16. And there is no A-MPR table in clause 6.2A.2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the description of Pi/2 BPSK in clause 6.2A.2.1 for intra-band CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for Pi/2 BPSK are wrongly refered in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change>

#### 6.2A.2.1		UE maximum output power reduction for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.4-1 with contiguous RB allocation is specified in Table 6.2A.2.4-1 for UE power class 3 CA bandwidth classes B and C.

In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 6.2A.2.1-1: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

For CA bandwidth class B and bandwidth class C with contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner and Outer RB allocations:

An RB allocation is contiguous if LCRB1 = 0 or LCRB2 = 0 or (LCRB1  0 and LCRB2  0 and RBStart1 + LCRB1 = NRB1 and RBStart2 = 0), where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a contiguous allocation is an inner allocation if

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High, and NRB_alloc  ≤  ceil(NRB,agg /2),

where

RBStart,Low = max(1, floor(NRB_alloc /2))

RBStart,High = NRB,agg – RBStart,Low – NRB,alloc,

with

NRB_alloc= LCRB1 ∙ 2^µ1 + LCRB2 ∙ 2^µ2

NRB_alloc= (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2,

NRB,agg=NRB1∙2^µ1+ NRB2∙2^µ2.

If LCRB1 =0, RBStart_CA = NRB1∙2^µ1+ RBStart2∙2^µ2,

if LCRB1 > 0, RBStart_CA = RBStart1∙2^µ1.

A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation.

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.5-1 with non-contiguous RB allocation is specified in Table 6.2A.2.4-2 for UE power class 3 CA bandwidth classes B and C.

Table 6.2A.2.1-2: non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 2 | 5.5 | 11.5 | 2.5 | 6 | 13 |
|  | QPSK | 2 | 5.5 |  | 2.5 | 6 |  |
|  | 16QAM | 2.5 | 5.5 |  | 3 | 6 |  |
|  | 64QAM | 4.5 | 6 |  | 5 | 6 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 2.5 | 6.5 | 12 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

For CA bandwidth classes B and C with non-contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner, Outer1 and Outer2 RB allocations:

Non-Contiguous RB allocation is defined as RBStart1 + LCRB1 < NRB1, or RBStart2 > 0, when both uplink CCs are activated and allocated with RB(s), where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High and NRB_alloc ≤  ceil((BWChannel_CA / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2, RBStart_CA = RBStart1∙2^1

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_CA – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of CC1 and CC2 respectively.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High and NRB_alloc ≤  ceil((3 BWChannel_CA / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc , RBStart_CA , BWgap and BWGB,low are as defined for the Inner region.

In contiguous CA, a non-contiguous allocation is an Outer 2 allocation if it is neither an non-contiguous Inner allocation nor an Outer 1 allocation.

## <End of Change>