3GPP TSG-RAN WG4 Meeting #98-e	R4- 2102437

Online, 25 Jan – 5 Feb 2020

Source: 	Huawei, HiSilicon

Title:	A-MPR for LTE CA_NS_04 256QAM PC2

Agenda Item:	5.2

Document for:	Approval

# 1	Introduction

In TS 36.101 v16.8.0 [1], the A-MPR values for LTE CA_NS_04 256QAM PC2 are still FFS. Based on our simulation results, the A-MPR values are proposed to complete the spec.

# 2	Discussion

According to Table 6.2.4A-1 in Clause 6.2.4A of [1], the network signal CA_NS_04 imposes additional SEM requirement in Clause 6.6.2.2A.1 as well as additional spurious emission requirement in Clause 6.6.3.3A.8. However, despite the stricter spectrum emission requirements, the gating factor is still the EVM limit as shown by the simulation results in the Appendix. Hence, the following A-MPR values are proposed for 256QAM, taking into account some implementation margin.

Proposal 1: Modify the A-MPR for CA_NS_04 (power class 2) as in Table 2.1

Table 2.1: Contiguous Allocation A-MPR for CA_NS_04 (power class 2)

| CA Bandwidth Class C | Lower edge cutoff frequency [MHz]5 | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR per modulation [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | QPSK | 16QAM | 64QAM | 256QAM |
| 25 RB / 100 RB | 2513.5 | 0 – 42 | >0 | N/A | ≤5 | ≤5 | ≤5 | 6.5 |
|  |  | 43 – 81 | N/A | >82 | ≤1 | ≤1.5 | ≤1.5 | 6 |
|  |  | 82 – 124 | >0 | N/A | ≤1 | ≤1.5 | ≤1.5 | 5 |
| 50 RB / 100 RB | 2518.4 | 0 – 52 | >0 | N/A | ≤5 | ≤5 | ≤5 | 6.5 |
|  |  | 53 – 94 | N/A | >95 | ≤1 | ≤1.5 | ≤1.5 | 6 |
|  |  | 95 – 149 | >0 | N/A | ≤1 | ≤1.5 | ≤1.5 | 5 |
| 75 RB / 75 RB | 2519.0 | 0 – 54 | >0 | N/A | ≤5 | ≤5 | ≤5 | 6.5 |
|  |  | 55 – 94 | N/A | >95 | ≤2 | ≤2.5 | ≤2.5 | 6 |
|  |  | 95 – 149 | >0 | N/A | ≤1.5 | ≤2 | ≤2 | 5 |
| 75 RB / 100 RB | 2523.4 | 0 – 64 | >0 | N/A | ≤5 | ≤5 | ≤5 | 6.5 |
|  |  | 65 – 114 | N/A | >115 | ≤2 | ≤2.5 | ≤2.5 | 6 |
|  |  | 115 – 174 | >0 | N/A | ≤1 | ≤1.5 | ≤2 | 5 |
| 100 RB / 100 RB | 2528.3 | 0 – 69 | >0 | N/A | ≤5 | ≤5 | ≤5 | 6.5 |
|  |  | 70 – 129 | N/A | >130 | ≤2 | ≤2.5 | ≤2.5 | 6 |
|  |  | 130 – 199 | >0 | N/A | ≤1.5 | ≤1.5 | ≤2 | 5 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe NOTE 5: The A-MPR values in this table shall apply when the lower edge of the aggregated channel bandwidth (Figure 5.6A-1) is less than or equal to the lower edge cutoff frequency specified in this table for the corresponding CA bandwidth combination. When the lower edge of the aggregated channel bandwidth exceeds the lower edge cutoff frequency, then the A-MPR shall be equal to the MPR specified in Table 6.2.3A-1a. |  |  |  |  |  |  |  |  |

# 3	Conclusion

Based on simulation results, the A-MPR values are proposed for CA_NS_04 256QAM.

Proposal 1: Modify the A-MPR for CA_NS_04 (power class 2) as in Table 2.1

# References

[1] 3GPP TS 36.101 v16.8.0

# Appendix

Table A-1: A-MPR Simulation Results for CA_NS_04

| CA Configuration | A-MPR excluding EVM | A-MPR including EVM |
| --- | --- | --- |
| 50+75 RB |  |  |
| 25+100RB |  |  |
| 50+100RB |  |  |
| 75+75RB |  |  |
| 75+100RB |  |  |
| 100+100RB |  |  |