3GPP TSG-RAN WG4 Meeting#98-e	R4- 2102821

Online, 25th Jan – 5th Feb, 2021

Source: 	Qualcomm Incorporated

Title: 	Views on Type II PMI Reporting Tests

Agenda item:	7.16.1.2

Document for:	Discussion

# Introduction

In WF [1], there were some open issues related to test metric and antenna correlation. In this paper, we discuss our views on those pending issues.

# Type II Codebook

In WF [1], there were 3 options for the test point. As link adaptation algorithms try to keep the UE PDSCH BLER closer to 10%, we prefer to use the baseline of 90% test point. Therefore, we propose the following.

Proposal 1: Use 90% test point for defining Type II PMI reporting test cases.

In WF [1], test metric is defined as the throughput ratio of following PMI and Random Type I PMI. We expect similar results for Type II PMI reporting as for enhanced Type II codebook in [2]. Based on those results, we propose the following.

Proposal 2: Use the test metric of throughput ratio between following Type II and random Type I for defining Type II PMI reporting tests and no need to check whether UE reported codebook is not only within Type I codebook set.

Proposal 3: Use XP Medium correlation for defining Type II PMI reporting tests.

# Conclusions

This paper discusses remaining open issues for enhanced Type II PMI reporting. Following has been observed and proposed.

Proposal 1: Use 90% test point for defining Type II PMI reporting test cases.

Proposal 2: Use the test metric of throughput ratio between following Type II and random Type I for defining Type II PMI reporting tests and no need to check whether UE reported codebook is not only within Type I codebook set.

Proposal 3: Use XP Medium correlation for defining Type II PMI reporting tests.

# References

[1] R4-2017681, “Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32”, 3GPP TSG RAN WG4 Meeting #97-e, Online, Nov 2 - Nov 13, 2020.

[2] R4-2100622, “Views on CSI Reporting test cases for eMIMO”, 3GPP TSG RAN WG4 Meeting #98-e, Online, 25th Jan – 5th Feb, 2021.