3GPP TSG-RAN WG4 Meeting # 98-e	R4-2102965

Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda item:			9.18.1

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL

Document for:	Information

Introduction

In the last RAN4#97e meeting, the SAR schemes for UE power class 2 NR inter-band CA and SUL configurations were discussed and a WF of R4-2016851 was approved with the following candidate options for SAR schemes:

- Duty Cycle based solutions

  - Option 1: Report the duty cycle capability per band combination (CTC, Intel, ZTE, Huawei, Apple)

    - Main issue commented by companies：Unequal responses for the SAR effects in different band frequencies.

  - Option 2: Report the duty cycle capabilities per band (CATT, Xiaomi, ZTE, OPPO, vivo, CMCC)

    - Main issue commented by companies: Too many pairs of signalling's, more detailed signalling design and values need to be provided, especially for the reference band.

- Blind scheme solution (Ericsson, Verizon, T-Mobile USA)

  - Further discussion on whether to consider (Scell) power dropping behavior due to power prioritization for UL CA and SUL configurations

- Other solutions/options are not precluded

Based on this alignment, companies further study the SAR solutions for CA and SUL. In this meeting, according to the contributions submitted, this email discussion thread will focus on the following aspects:

- Topic#1: PC2 band-combination requirements for example combos

- Topic#2: PC2 SAR solutions

  - Sub-topic 2-1: For PC2 inter-band CA

  - Sub-topic 2-2: For PC2 SUL configurations

  - Sub-topic 2-3: Release independence issue

- Topic#3: Power configuration

Note that the tables for collecting comments for sub-topic issues are arranged just below each issue.... and the tables for collecting comments for CR/TP are still kept at the original position.

Topic #1: PC2 band-combination requirements for example combos

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/Abstracts |
| --- | --- | --- |
| R4-2101109 | Xiaomi, ZTE Corporation | Observation 1: No need to consider harmonic and IMD issues for PC2 band combination CA_n41-n79. Observation 2: Cross band isolation issue shall be re-evaluated for those PC2 cases that one band can support 26dBm if simultaneous Rx/Tx is supported for CA_n41-n79. Proposal 1: It is proposed to define the MSD requirements due to cross band isolation as shown in table 3 for CA_n41-n79 high power UE. Table 3: the proposed value for MSD requirements due to cross band isolation for high power UE NR Band / Channel bandwidth of the affected DL band. UL band; DL band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB); 25 MHz (dB); 30 MHz (dB); 40 MHz (dB); 50 MHz (dB); 60 MHz (dB); 70 MHz (dB); 80 MHz (dB); 90 MHz (dB); 100 MHz (dB). n41; n79; 3.1; 3.1; 3.1; 3.1; 3.1. n79; n41; 3.5; 3.3; 3.1; 2.6; 2.5; 2.5; 2.4; 2.4; 2.4. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: PC2 band-combination requirements

This sub-topic will discuss band-combination requirements for PC2 inter-band CA, i.e. CA_n41A-n79A

Issue 1-1-1: Requirements for PC2 CA_n41A-n79A

- Proposals

  - It is proposed to define the MSD requirements due to cross band isolation as shown in table 3 for CA_n41-n79 high power UE. (R4-2101109)

Table 3: the proposed value for MSD requirements due to cross band isolation for high power UE

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n41 | n79 |  |  |  |  |  |  | 3.1 | 3.1 | 3.1 |  | 3.1 |  | 3.1 |
| n79 | n41 |  | 3.5 | 3.3 | 3.1 |  |  | 2.6 | 2.5 | 2.5 |  | 2.4 | 2.4 | 2.4 |

- Recommended WF

  - Collect views on the proposed MSD values, if no comments, the contribution R4-2101109 will be recommended as approved.

| Company | Comments on Issue 1-1-1: Requirements for PC2 CA_n41A-n79A |
| --- | --- |
| ZTE | Agree with the proposals. |
| Apple | We would like to request more time to review the proposed values |
| Huawei | Disagree with observation 1. We have a draft CR in last meeting to consider the harmonic MSD in R4-2016483. The harmonic issue should be further studied. |
| Xiaomi | The proposal is about MSD requirements due to cross band isolation for high power UE. For harmonic issue, we are OK to further study. |

Companies views’ collection for 1st round

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: The contribution R4-2101109 is recommended as return to by considering comments and response and to continue the discuss along with the corresponding TP of R4-2102221 in thread 118 which is based on the proposal of this contribution. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP/Contribution number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2101109 | Return to |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Topic #2: PC2 SAR solutions

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/Abstracts |
| --- | --- | --- |
| R4-2100100 | CMCC | Proposal: It is proposed to define the SAR solutions for PC2 inter band CA and SUL configurations based on per band. |
| R4-2101101 | Xiaomi | Observation 1: If dutycycle based solution is used, the P-MPR impact on UE configured maximum output power could be decrease. Proposal 1: Besides the default solution, i.e. UE implementation based solution (P-MPR), the dutycycle based solution can be introduced as a capability for PC2 NR inter-band CA UE meeting SAR issue. Observation 2: For option 2, the most important thing is not about which band is selected as reference band but how many reference points and theirs values are used. Observation 3: Option 2a is not only saving signaling but also be more operable in practice compared to option 2b. Proposal 2: For PC2 inter-band CA case, it is proposed option 2a is adopted Proposal 3: For FDD+TDD case, it is proposed to select FDD as reference band by considering the dutycycle for FDD is not so viable and FDD band usually has better coverage due to low frequency. The reference points can be reused from NSA FDD+TDD case. For TDD+TDD case, it is proposed to select Low band as reference band, but how many reference points shall be used can be FFS. |
| R4-2101102 | Xiaomi | Observation 1: If dutycycle based solution is used, the P-MPR impact on UE configured maximum output power could be decrease. Proposal 1: Besides the default solution, i.e. UE implementation based solution (P-MPR), the dutycycle based solution can be introduced as a capability for PC2 NR inter-band CA UE meeting SAR issue. Proposal 2: For PC2 SUL case, it is proposed to report UL duty cycle capabilities based on the duty cycle in NUL band and how many reference points are used can be FFS. |
| R4-2101117 | China Telecom | Proposal 1: Report one total UL duty cycle capability by considering SARratio factor as expressed by the equation 2. DutyNR, x *( PNR,x/ P26)*SARratioNR, x + DutyNR, y *(PNR, y/ P26)* SARratioNR, y ≤ Duty threshold …(2) Proposal 2: Report the maxUplinkDutyCycle-CA-PC2 as the sequence of maxUplinkDutyCycle[1,2,3,4] for power class 2 case [a,b,c,d] correspondingly. - Proposal 2a: The SARratio factor is proposed by equation 3, but no need to report since it could be inherited from the corresponding single band capability. SARratioNR, x = 50%/DutycycleNR, x SARratioNR, y = 50%/DutycycleNR, y …(3) |
| R4-2101118 | China Telecom | Proposal 1: Report one total UL duty cycle capability by considering SARratio factor as expressed by the equation 2. Proposal 2: Report the maxUplinkDutyCycle-SULcombination-PC2 for power class 2 NR SUL configurations. - Proposal 2a: The SARratio factor is proposed by equation 3, but no need to report since it could be inherited from the corresponding single band capability. |
| R4-2101119 | China Telecom | draft CR to 38.101-1 Introduce SAR solution for UE power class 2 NR inter-band CA with 2UL |
| R4-2101120 | China Telecom | draft CR to 38.101-1 Introduce SAR solution for UE power class 2 NR SUL configurations |
| R4-2101726 | Ericsson | We make the following Observation 1: duty-cycle reporting is not viable for UL CA (and EN-DC) - the ‘actual’ UE output powers on the uplinks also determine the total average output power; the network has limited information about the UE output power on a radio-frame time scale, the PHR is not that frequent, not accounting for any scaling and has limited reporting accuracy - the measurement of the ‘actual’ duty cycle is ambiguous in the time domain; ”certain evaluation period” has been used for TDD HPUE throughout, but is unknown to the scheduler for its evaluation and Proposal 1: duty cycle reporting should not be specified for UL CA PC2; it is not viable. Instead, we propose that Proposal 2: to facilitate SAR compliance for UL CA PC2 and prevent dropping of SCells, UE-specific absolute or relative power limits (P-Max) could be specified modifying the configured maximum output power per serving cell. Proposal 3: the UE-specific power limits could be used in conjunction with the P-MPR method. Proposal 4: the absolute/relative power limits are set up during the RRC reconfiguration (or modification) of the band combination. The limit to be used by the UE is determined by a MAC-CE or a PDCCH message based on a DCI format, which enables fast adaptation to changing radio conditions temporarily enabling/disabling limits. The method is also applicable for SUL. The following should also be considered: Observation 2: the Pcmax,f,c per serving cell and thus the PHR (always per cell) would be modified by the P-MPR method, but not by any power prioritization for concurrent transmissions according to 38.213. Observation 3: the PCMAX would be modified by the P-MPR method for SAR compliance (long time scale) and thus the power prioritization for concurrent transmissions (slot time scale) according to 38.213. |
| R4-2101752 | OPPO | Observation 1: Reporting of combined Band X +Band Y duty cycle capability is a possible way for inter-band UL CA HPUE SAR issues. Proposal 1: It is proposed to consider reporting a group of combined maxUplinkdutycycle capabilities for inter-band UL CA HPUE SAR issue. |
| R4-2102190 | ZTE Corporation | Proposal 1. For duty cycle based solutions, report both total duty cycle capability and duty cycle of PCell. If only Option 1 and Option 2 in the WF are considered, for sake of progress, Option 2 is also acceptable for us. |
| R4-2102191 | ZTE Corporation | Proposal 1. For duty cycle based solutions, report both total duty cycle capability and duty cycle of PCell. Proposal 2. Introduce maximum output power table for both PC3 and PC2 SUL in TS38.101-1. For proposal 1, if only Option 1 and Option 2 in the WF are considered, for sake of progress, Option 2 is also acceptable for us. |
| R4-2102287 | Huawei, HiSilicon | Proposal 1: It is proposed to report the per-BC total UL duty cycle capability as well as SARratio factor for NR HPUE UL CA band combinations. |
| R4-2102289 | Huawei, HiSilicon | Proposal 1: It is proposed to report the per-BC total UL duty cycle capability as well as SARratio factor on the PC2 NR band for NR SUL HPUE band combinations. Proposal 2: PC2 HPUE only falls back maximum output power on TDD band but not SUL band when operating under SUL-TDD band combinations. Proposal 3: The condition for UE power fallbacks is met when either 1) the network configures 23dbm or less Pmax or 2) the network schedules too many UL resources, under SUL-TDD band combinations operating with 26dbm MOP. |
| R4-2102712 | vivo | Proposal 1: The solution based on option 2 with detail signaling: - Select PCell band as the reference band, UE report duty cycle capability based on PCell band. - Select 2 reference points on the reference band (Pcell band); - Two pair of reference points are defined: [40%/70%], [20%/35%], UE chooses one pair reference points to report duty cycle. The signalling can be as the following table: UE maxUplinkDutyCycle signaling; Parameter (for another cell). ReferenceDutyCycle70and40; {maxUplinkDutyCycle1, maxUplinkDutyCycle2}. ReferenceDutyCycle35and20; {maxUplinkDutyCycle1, maxUplinkDutyCycle2}. - When the actual uplink transmission time exceed the UE capability, the overall power class fallback and the power reduction on each band and each channel follows the RAN1 power allocation prioritizing order. Observation: The UE would actually only report one of the two sets of parameters after RACH, this means the signalling overhead is in fact aligned with FDD-TDD EN-DC. Proposal 2: Further analysis for removing upper power limits for PC2 UE with the inter-band NR CA is proposed. |
| R4-2101121 | China Telecom | Proposal 1: It is proposed to consider the example provided in this contribution when specifying into 38.307. |
| R4-2101122 | China Telecom | CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R15) |
| R4-2101123 | China Telecom | CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R16) |
| R4-2101124 | China Telecom | CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R17) |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1: For PC2 inter-band CA

This sub-topic will discuss SAR schemes/solutions for PC2 inter-band CA.

Issue 2-1-1: SAR schemes for PC2 inter-band CA

- Proposals

  - Duty Cycle based solutions

    - Option 1: Report the duty cycle capability per band combination

    - Option 2: Report the duty cycle capabilities per band

  - “blind scheme” solution

    - To facilitate SAR compliance for UL CA PC2 and prevent dropping of SCells, UE-specific absolute or relative power limits (P-Max) could be specified modifying the configured maximum output power per serving cell.

- Recommended WF

  - Try to merge the options for capabilities reporting for duty cycle solution

  - Discussion on “blind scheme” solution

| Company | Comments for Issue 2-1-1: SAR schemes for PC2 inter-band CA |
| --- | --- |
| China Telecom | Wonder if any update for supporting option 1 or option 2 by comparing to the standpoint of last meeting? as we have addressed the main concern of option 1 in the contribution R4-2101117. |
| OPPO | Option 1. |
| Vivo | For the potential SAR test method, we propose 2 reference points for option2, it can reduce the signalling and simplify SAR test. The duty cycle limits in capability report is the worst case in the SAR test. If the actual uplink transmission exceeds the limits, UE power class will fallback. For per band SAR test, only the duty cycle limits need to be verified, and the totally SAR effect is the combination of these duty cycle limits’ results. About the option1 in R4-2101117(China Telecom), it is much more complete compared to last meeting. Still, there is a new solution for inter-band CA based on SARratio, compared to ENDC case, which may bring complexity. And also, we would like some further clarification on the difference between signalling n100 and full_duty. |
| Xiaomi | Prefer option 2 |
| ZTE | Option 2. SARratio factor become more complexity and it may not easy to determine the exact value for SARratio factor since SAR test is very complicated and affected by various factors such as implementation where different antenna configurations for one band. |
| Ericsson | Neither Option 1 nor Option 2. 1. Duty-cycle reporting should not be specified for UL CA PC2; it is not viable. What is the network supposed to do when the measurement of the ‘actual’ duty cycle that is ambiguous in the time domain; ”certain evaluation period” has been used for TDD HPUE throughout, but is unknown to the scheduler for any evaluation For the UE, the ‘actual’ UE output powers on the uplinks also determine the total average output power, e.g. there is no need to fall back to PC3 if the output power is low on the FDD even if the duty cycle is 100%. The network has limited information about the UE output power on a radio-frame time scale, the PHR is not that frequent, not accounting for any scaling and has limited reporting accuracy Duty cycle reporting may be useful for a single UL CC to indicate that the UE is capable of exceeding the default 50% for TDD PC2, then the total UE power is not depending on the power and time-domain behaviour of another UL. 2. We agree to listing the candidate solutions in a WF as proposed by the moderator. We propose that the blind scheme is used in conjunction with the proprietary P-MPR method. SAR compliance for UL CA PC2 can be facilitated and dropping of SCells prevented by UE-specific absolute or relative power limits (P-Max) could be specified modifying the configured maximum output power per serving cell. We also propose that the WF lists include consideration of impact on the PHR for the different schemes. Regarding the “blind scheme” we agree to use this notion for the sake of “backwards compatibility” but would like to point out that the scheme is not blind! The UE behaviour is governed by network configuration. |
| Huawei | Support option 1. Note that the “blind scheme” was already excluded from the selected SAR solution for FDD-TDD EN-DC HPUE in R16. For the sake of progress, we should focus on duty cycle based solutions. |

Sub-topic 2-2: For PC2 SUL configurations

This sub-topic will discuss SAR schemes/solutions for PC2 SUL configurations.

Issue 2-2-1: SAR schemes for PC2 SUL configurations

- Proposals

  - Duty Cycle based solutions

    - Option 1: Report the duty cycle capability per band combination

    - Option 2: Report the duty cycle capabilities per band

  - “blind scheme” solution

    - To facilitate SAR compliance for UL CA PC2 and prevent dropping of SCells, UE-specific absolute or relative power limits (P-Max) could be specified modifying the configured maximum output power per serving cell.

- Recommended WF

  - Try to merge the options for capabilities reporting for duty cycle solution

  - Discussion on “blind scheme” solution

| Company | Comments for Issue 2-2-1: SAR schemes for PC2 SUL configurations |
| --- | --- |
| OPPO | Option 1 |
| ZTE | Same with PC2 inter-band NR CA. Option 2. |
| Huawei | Support option 1. |
| Xiaomi | Prefer option 2 |

Sub-topic 2-3: Release independence issue

Issue 2-3-1: Release independence issue for NR PC2 inter-band CA and SUL configurations

- Proposals

  - It is proposed to agree the CRs R4-2101122, R4-2101123, R4-2101124 for 38.307.

- Recommended WF

  - Collect the views on the release independence CRs R4-2101122, R4-2101123, R4-2101124. If no comments, the CRs will be recommended as agreed.

| Company | Comments for Issue 2-3-1: Release independence issue for NR PC2 inter-band CA and SUL configurations |
| --- | --- |
| Nokia | Spec structure shall have a consistency. The proposed structure is not aligned with 8.1.1 Intraband EN-DC. We also do not understand why every single CR is submitted as Cat B? What is the difference between the three? |
| China Telecom | Ok, we can change the order and numbering of the table to align with EN-DC, is that correct understanding? The three Cat B CR’s are for R17, 16 and 15. We just follow the way from TDD EN-DC as shown in agreed R4-1913209 and R4-1913210, which are two Cat B CR’s for R16 and R15. |

Companies views’ collection for 1st round

CRs/TPs comments collection

The following two draft CRs will depend on the progress of SAR schemes discussion.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2101122 | Ericsson: not agreed, premature. |
| R4-2101123 | Ericsson: not agreed, premature. |
| R4-2101124 | Ericsson: not agreed, premature. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Candidate options: - Duty Cycle based solutions - Option 1: Report the duty cycle capability per band combination - Option 2: Report the duty cycle capabilities per band - “blind scheme” solution - To facilitate SAR compliance for UL CA PC2 and prevent dropping of SCells, UE-specific absolute or relative power limits (P-Max) could be specified modifying the configured maximum output power per serving cell. Recommendations for 2nd round: Regarding the release independence CR’s, the CR’s for rel-15 and rel-16 are not needed according to the GTW discussion for the rules. The rel-17 CR of R4-2101124 is recommended as “revised to” to capture the comments. A way forward is recommended to capture the SAR solutions and release independence CR. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| 1 | WF on SAR solutions for PC2 NR inter-band CA and SUL configurations | China Telecom |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2101122 | Not pursued |
| R4-2101123 | Not pursued |
| R4-2101124 | Revised to |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #3: Power configuration

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/Abstracts |
| --- | --- | --- |
| R4-2100372 | CATT | Observation 1: The impact to current PC3 definition needs to be clarified. Observation 2: How to interpret the power class for inter-band UL CA needs to be clarified. Observation 3: The spec impacts need more discussion such as Pcmax_H definition and Power class report. |
| R4-2102414 | Qualcomm | The specifications should allow for this possibility by either removing artificial constraints on maximum configured output power or by definition of a new power class to reflect the hardware configuration. The pros and cons of each of these two approaches is listed in this contribution with a proposal that RAN4 should agree which is the most appropriate to follow. |
| R4-2102712 | vivo | Proposal 2: Further analysis for removing upper power limits for PC2 UE with the inter-band NR CA is proposed. |
| R4-2101749 | OPPO | Power class Observation 1: Remove Ppowerclass from Pcmax_h, will allow UE transmit power higher than 28dBm which is not compatible with current power class definition. Observation 2: Remove power class upper limit will make power levels overlapping among different power classes, and the interpretation of power classes need to be clarified. Observation 3: If PC2 UE is allowed to transmit at 29dBm with 2T, but only 26dBm with 1T, then whether PC2 UE is also allowed to transmit 26dBm with 2T but only 23dBm with 1T? Conformance tests Observation 4: If removing Ppowerclass also is planned for CA/DC PC3, SA PC3/PC2 and NSA PC3/PC2, then the impacts to the already existing systems/specifications are not trivial and need to be taken into account carefully. Power enhance approaches Observation 5: Current 3dB power step for PC1.5/2/3 is already a trade-off between flexibility and complexity, defining more power classes among them is not a good approach. Observation 6: The room for power enhancement by removing the upper limit is only around 1.5dB considering the PA reliability in massive production and also the already power boosting to compensate RFFE IL and antenna efficiency loss. Observation 7: Power consumption and thermal issues already been challenging for 2T PC2 UEs, transmitting even higher power is not a good choice from UE performance perspective. Observation 8: No matter removing the upper limits or introducing new power class, the benefits seem not big enough but potential impacts are big. Observation 9: Another alternative is to introduce a 2dB or 3dB power boosting UE capability similar as the capability introduced in switched UL transmission (or super UL) topic while keeping PC2 definitions. Proposal 1: It is proposed to do more analysis on the impacts and achieve common understanding before removing upper limits of Pcmax. Proposal 2: Consider introducing 2dB or 3dB power boosting UE capability while keeping PC2 definitions to allow UE transmit higher power levels. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1: Power configuration

This sub-topic will discuss power configuration for UE equipped with two PA’s.

Issue 3-1-1: Increasing UE maximum power for UE equipped with two PA’s

- Proposals on increasing UE maximum power for UE equipped with two PA’s

  - The specifications should allow higher UE Tx power by

    - Option1: Removing artificial constraints on maximum configured output power

    - Option2: Definition of a new power class to reflect the hardware configuration.

- Recommended WF

  - Collect views on the proposals.

| Company | Comments for Issue 3-1-1: Increasing UE maximum power for PC2 inter-band CA |
| --- | --- |
| Verizon | Sub-topic 3-1: The limited TDD link budget in NR seems a generic concern in the number of CA and DC operations. And, it is possible to maximize output power of the UE by dual Tx paths now. Thus, RAN4 should allow this feature, and we like support the idea. We slightly prefer the method in modifying PCMAX_H because it is simpler. RAN4 should study two approaches further and finalize a solution in the most appropriate one. |
| Nokia | In principle, we support the idea to allow UEs to transmit up to UE’s real maximum power. We, however, do not support both provided options at this moment. It is too early to agree with the solution. Firstly, we would like to agree with the concept to leverage UE’s real ability to full advantage and to make the UE transmit at higher power. And then, we study this idea thoroughly in future meetings to find out the best solution. For option 1, as mentioned by CATT, we need more study on what removing the upper limit means and if there are alternatives or not. Still UE has to a signal PC for a CA? If not, existing network considers the CA configuration as PC3 as default. If the UE signals it, network shall ignore the signaled PC etc. For option 2, it is better to avoid this as much as possible. Specifications become full of PCs… |
| OPPO | Agree with Nokia, the concepts and impacts have not been discussed thoroughly, before that it cannot be agreed to decide how to increase the power. In our paper we discussed the impact to power class and trying to understand better on the meaning of removing the higher limit. And the observations reproduced here for information. Observation 1: Remove Ppowerclass from Pcmax_h, will allow UE transmit power higher than 28dBm which is not compatible with current power class definition. Observation 2: Remove power class upper limit will make power levels overlapping among different power classes, and the interpretation of power classes need to be clarified. Observation 3: If PC2 UE is allowed to transmit at 29dBm with 2T, but only 26dBm with 1T, then whether PC2 UE is also allowed to transmit 26dBm with 2T but only 23dBm with 1T? Besides, actually in our paper we suggest to keep the current spec structure unchanged but introduce a power boosting capability to allow UE transmit higher powers. |
| Vivo | As mentioned in QC’s paper, if removing Pmax, no update for the MPR, A-MPR may not appropriate. For higher maximum output power, new MPR and A-MPR may be needed. An “Option 3: no update for power configuration” can also be considered in the current stage. |
| ZTE | Higher maximum output power may introduce additional RF requirements discussion considering the SAR and regulatory requirements, also it will cause more discussions for the new PC, not only for the single band operation, but also for band combination. We tent to agree that no updating for the current power configuration. |
| Ericsson | We are open to considering a total CA power exceeding 26 dBm. However, RAN4 must specify a PCMAX for each CA configuration for this governs the power prioritization specified in 38.213 clause 7.5. The power prioritization rules apply when the total configured output power PCMAX is exceeded. For UL PC2 then the PCMAX would be upper bounded by 26 dBm. For concurrent transmissions of the CCs, the total SCell power would be capped at 23 dBm or the SCell power reduced for a PCell transmission at 23 dBm that is of higher priority (clause 9 of 38.213). If the priority of the transmissions is the same, the PCell power would be prioritized over the SCell but the actual scaling is not specified. Moreover, the UE (can) indicate a power class for the band combination. If absent, then the default power class applies for the band combination (PC3 at present). |
| Huawei | The motivation of removing the upper limit for Pcmax is understandable, but it could cause unexpected consequences. Regulatory requirements may be violated, as the expected output power is based on the advertised power class. The measurement is also based on the reported power class, but without the upper limit, the max output power for inter-band combination is unpredictable. Currently, the SAR solution discussed under this agenda also considers the duty cycle capability solution, in which the capability is derived upon certain power class, however, removing the upper limit of power class may invalidate the solution. So many issues exist for the proposed changes for the requirement. It is also worth noting that this issue is not included in the WI objective. |
| Apple | We prefer Option 2; however, in this case we don’t need to consider a new power class; rather, we need to define PC1.5 requirements for the appropriate band based on operator request. In our understanding, the constraints on UE output power are informed by regulatory and conformance limitations, and removing such constraints, as proposed in Option 1, would introduce ambiguity to the entire power class framework in the specification. As Verizon has commented, there is operator concern about uplink link budgets; we would be interested in understanding whether existing PC1.5 work is already addressing this or if there are gaps relative to operator needs. |
| Skyworks | To enable the higher power upper limit some other aspects that must be understood like how the max duty cycle may work. for PC2+PC2 case we can reuse PC1.5 but what about PC2+PC3? Also in the NRU+NR CA/DC we have PC3+PC5 or PC2+PC5…we should try to find a generic solution |

Companies views’ collection for 1st round

CRs/TPs comments collection

The following two draft CRs will depend on the progress of SAR schemes discussion.

| CR/TP number | Comments collection |
| --- | --- |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic# | Tentative agreements: No consensus on any of the options Candidate options: Recommendations for 2nd round: Continue discuss on 2nd round, and a WF is recommended to be assigned. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on increasing UE maximum power for UE equipped with two PA’s | [Qualcomm] |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)