3GPP TSG-RAN WG4 Meeting #98-e                               R4-2103061

Online, 25th Jan-5th Feb, 2021

Agenda item:	9.7.2

Source:	KDDI

Title:	TP for TR 37.717-11-21: DC_1-3-18_n41-n77

Document for:	Approval

1	Introduction

This contribution is a text proposal for TR 37.717-11-21 to include DC_1-3-18_n41-n77 and according to the request in [1].

2.  Reference

- RP-201477, New WID on Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL).

3.  Text Proposal

[Start of text Proposal]

8.X	DC_1-3-18_n41-n77

8.X.1	Operating bands for DC

Table 8.X.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n41-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

8.X.2	Channel bandwidths per operating band for DC

Table 8.X.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
| DC_1A-3A-18A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 355 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.X.3	Co-existence studies

Co-existence studies of DC_1A-3A-18A_n41A-n77A and protected bands are already covered in the constituent fall-back modes.

8.X.4	∆TIB and ∆RIB values

For DC_1-3-18_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n41-n77 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 18 | 0.3 |
|  | n41 | 0.31 |
|  |  | 0.82 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n41-n77 | 1 | 0.0 |
|  | 3 | 0.5 |
|  | 18 | 0 |
|  | n41 | 01 |
|  |  | 0.52 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

8.X.5	MSD

As mentioned in 8.X.3, there is no need to specify additional MSD requirement for this UL DC configuration.

[End of text Proposal]