3GPP TSG-RAN WG4 Meeting # 98-e 												        R4-2103462

Electronic Meeting, 25 Jan. - 5 Feb., 2021

Agenda item:			7.15.1

Source:	Moderator (CMCC)

Title:	Email discussion summary for [98e][223] NR_HST_RRM

Document for:	Information

Introduction

This email discussion focuses on RRM for Rel-16 NR HST, and in particular the agenda items are:

7.15.1	RRM requirements maintenance

The targets of email discussion for 1st round and 2nd round are:

- 1st round: provide comments on the CRs.

- 2nd round: further discussion on CRs and strive to agree on CRs

Topic #1: RRM core requirements maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100237 | Apple | Observation 1: HST scaling factor M2 is outside of CSSF. Therefore, the measurement on SCC is also enhanced in R16. Proposal 1: further clarification needs to be introduced to clarify that R16 enhancement only applies when there is no SCC configured. |
| R4-2100238 | Apple | CR on HST core part maintenance |
| R4-2100239 | Apple | CR on HST core part maintenance (R17) |
| R4-2101013 | Apple | CR on HST core part maintenance (R17) |
| R4-2100849 | CMCC | CR on HST RRM requirements in connected mode |
| R4-2100850 | CMCC | CR on HST RRM requirements in connected mode |
| R4-2101844 | Huawei, HiSilicon | CR on Correction on inter-RAT measurement in high speed scenario |
| R4-2101845 | Huawei, HiSilicon | CR on Correction on inter-RAT measurement in high speed scenario |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Companies are encouraged to provide comments to the CRs.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| CRs related to the IE update |  |
| R4-2100849 (CMCC) | Apple: the change looks good. |
|  | Huawei: ok |
|  | Ericsson: ok |
|  | CATT: fine with the correction. Only one minor comment: the first half of Note 1 in Table 9.2.6.2-3 needs update for RAN2 IE name. |
|  | MediaTek: ok |
|  | CMCC: we are OK with CATT’s comment, and will update the CR. |
|  | Nokia: The CR is OK, but it seems the proposed changes are similar to R4-2100238 except that there is an additional note in R4-2100238 to clarify the applicability of the requirement to SCells? |
| R4-2100850 (CMCC) Mirror CR | Company A |
|  | Company B |
| R4-2101844 (Huawei, HiSilicon) | Apple: the change looks good. |
|  | Ericsson: The correction is good. |
|  | CATT: ok |
|  | MediaTek: ok |
|  | CMCC: OK |
|  | Nokia: The CR is OK. |
| R4-2101845 (Huawei, HiSilicon) Mirror CR | Company A |
|  | Company B |
| CRs related to the active SCell in HST |  |
| R4-2100238 (Apple) | Apple: IE update related content can be removed and merged into other CRs. |
|  | Ericsson: The clarification is good. |
|  | CATT: ok |
|  | MediaTek: ok |
|  | CMCC: we are OK with the clarification. But for the wording, we have another suggestion. In our view, even for CA scenario, the Rel-16 HST requirements can be applied to the PCell. But the current wording in the CR “NOTE 3: Requirement doesn’t apply if UE is configured with highSpeedMeasFlag-r16 and at least one SCell.”, seems that requirement doesn’t apply to all the CCs including PCell for CA scenario. Our suggestion is that Note 3 can be updated as “When highSpeedMeasFlag-r16 is configured, the requirements apply only to measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell”, which is also the approach we used for LTE HST scenario. |
|  | Nokia: See the comments for R4-2100849. Regarding Note 3, the proposed wording from CMCC is clearer. |
| R4-2100239 (Apple) Mirror CR | Company A |
|  | Company B |
| R4-2101013 (Apple) Mirror CR | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2100849 (CMCC) | to be revised |
| R4-2100850 (CMCC) Mirror CR | Return to |
| R4-2101844 (Huawei, HiSilicon) | agreeable |
| R4-2101845 (Huawei, HiSilicon) Mirror CR | agreeable |
| R4-2100238 (Apple) | to be revised |
| R4-2100239 (Apple) Mirror CR | After further check with Apple, the release of this mirror CR is not correct. This CR can be withdrawn. |
| R4-2101013 (Apple) Mirror CR | Return to |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #2: RRM performance part maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100484 | CATT | CR on correction to cell reselection test case for HST |
| R4-2100485 | CATT | CR on correction to cell reselection test case for HST |
| R4-2101846 | Huawei, HiSilicon | CR on correction on test cases for inter-RAT cell identification in connected mode for HST |
| R4-2101847 | Huawei, HiSilicon | CR on correction on test cases for inter-RAT cell identification in connected mode for HST |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Companies are encouraged to provide comments to the CRs.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 2-1: Sub topic 2-2: …. Others: |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2100484 (CATT) | Apple: change is OK. |
|  | Huawei: ok |
|  | Ericsson: OK |
|  | MediaTek: ok |
|  | CMCC: OK |
|  | Nokia: The CR is OK. |
| R4-2100485 (CATT) Mirror CR | Company A |
|  | Company B |
| R4-2101846 (Huawei, HiSilicon) | Apple: change is OK. |
|  | Ericsson: OK |
|  | CATT: ok |
|  | MediaTek: ok |
|  | CMCC: OK |
|  | Nokia: The CR is OK. |
| R4-2101847 (Huawei, HiSilicon) Mirror CR | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2100484 (CATT) | agreeable |
| R4-2100485 (CATT) Mirror CR | agreeable |
| R4-2101846 (Huawei, HiSilicon) | agreeable |
| R4-2101847 (Huawei, HiSilicon) Mirror CR | agreeable |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |