3GPP TSG-RAN WG4 Meeting #97-e	R4-2103495

# Electronic Meeting, Nov 2 – 13, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1653 | rev | 1 | Current version: | 15.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on test cases of Inter-frequency Measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-12-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In test cases of Inter-f Measurements in FR2, there are multiple mistakes in Noc, , for Cell 2/Cell 3 in NSA and Cell 1/Cell 2 in SA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | a) Remove Noc, and correct BB, Io values for inter-frequency measurement test cases. b) Remove redundant Noc-related parameters, as no noise is applied c) Update Es/Iot to baseband value and correct Io in A.5.x test cases d) Specify AWGN propagation per frequency e) Update Test case Notes and remove A3 offset [ ] in Table A.5.6.2.1.1-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.6.2.1; A.5.6.2.3, A.7.6.2.1; A.7.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1 ------------------------------------------------------------

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used

A.5.6.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR2 on NR RF channel 1 and NR cell 3 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.5.6.2.1.1-1, A.5.6.2.1.1-2, and A.5.6.2.1.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.5.6.2.1.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #13 as defined in Table A.5.6.2.1.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.2-1. Supported test configurations are shown in table A.5.6.2.1.1-1.

Table A.5.6.2.1.1-1 EN-DC event triggered reporting tests without SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | LTE FDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.5.6.2.1.1-2: General test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  | One E-UTRAN TDD carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  | Two FR2 NR carrier frequencies are used. |
| Active cell |  | Config 1,2 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2 | 0 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 39 | 39 |  |
| SMTC-SSB parameters |  | Config 1,2 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1,2 | -30 |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |
| CP length |  | Config 1,2 | Normal |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2 | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2 | 3 s |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2 | 5 |  |  |
| T2 | s | Config 1,2 | 5.2 for PC1; 3.5 for other PC | 5.2 for PC1; 3.5 for other PC |  |

Table A.5.6.2.1.1-3: Cell specific test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |  |
| AoA setup |  | Config 1,2 | Setup 3 as specified in clause A.3.15 |  |  |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |  |  |
| Assumption for UE beamsNote 7 |  | Config 1,2 | Rough |  | Rough |  |  |  |
| NR RF Channel Number |  | Config 1,2 | 1 |  | 2 |  |  |  |
| Duplex mode |  | Config 1,2 | TDD |  | TDD |  |  |  |
| BWchannel | MHz | Config 1,2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |  |
| BWP BW | MHz | Config 1,2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |  |
| TDD configuration |  | Config 1,2 | TDDConf.3.1 |  | TDDConf.3.1 |  |  |  |
| Initial DL BWP |  | Config 1,2 | DLBWP.0.1 |  | NA |  |  |  |
| Initial UL BWP |  | Config 1,2 | ULBWP.0.1 |  | NA |  |  |  |
| Dedicated DL BWP |  | Config 1,2 | DLBWP.1.1 |  | NA |  |  |  |
| Dedicated UL BWP |  | Config 1,2 | ULBWP.1.1 |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | Config 1,2 | OP.1 |  | OP.1 |  |  |  |
| TRS configuration |  | Config 1,2 | TRS.2.1 TDD |  | NA |  |  |  |
| TCI configuration |  | Config 1,2 | CSI-RS.Config.0 |  | NA |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,2 | SR.3.1 TDD |  | - |  |  |  |
| CORESET Reference Channel |  | Config 1,2 | CR.3.1 TDD |  | - |  |  |  |
| SMTC configuration defined in A.3.11 |  | Config 1,2 | SMTC.1 |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2 | 120 |  | 120 |  |  |  |
| EPRE ratio of PSS to SSS |  | Config 1,2 | 0 |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Ês | dBm/SCS | Config 1,2 | -87 | -87 | -Infinity |  |  | -87 |
| SSB-RP Note 3 | dBm/SCS Note5 | Config 1,2 | -87 | -87 | -Infinity |  | -87 |  |
| BB Note 8 | dB | Config 1,2 | 1.89 | 1.89 | -Infinity |  | 1.89 |  |
| Io Note3 | dBm/95.04 MHz Note5 | Config 1,2 | -58.01 | -58.01 | -Infinity |  | -58.01 |  |
| Propagation Condition |  | Config 1,2 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB-RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 6: As observed with 0dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

A.5.6.2.1.2	Test Requirements

In test 1 with per-UE gap and in test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

In test 1 and 2 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

------------------------------------------------------------- End of change 1 ------------------------------------------------------------

----------------------------------------------------- Beginning of Change 2 ------------------------------------------------------------

#### A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used

##### A.5.6.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR2 on NR RF channel 1 and NR cell 3 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.5.6.2.3.1-1, A.5.6.2.3.1-2, and A.5.6.2.3.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.5.6.2.3.1-1 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #13 as defined in Table A.5.6.2.3.1-1 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.2-1. Supported test configurations are shown in table A.5.6.2.3.1-1.

Table A.5.6.2.3.1-1 EN-DC event triggered reporting tests with SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | LTE FDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.5.6.2.3.1-2: General test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  | One E-UTRAN TDD carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  | Two FR2 NR carrier frequencies are used. |
| Active cell |  | Config 1,2 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2 | 0 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 39 | 39 |  |
| SMTC-SSB parameters |  | Config 1,2 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1,2 | -30 |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |
| CP length |  | Config 1,2 | Normal |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2 | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2 | 3 s |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2 | 5 |  |  |
| T2 | s | Config 1,2 | 7 for PC1; 4.5 for other PC | 7 for PC1; 4.5 for other PC |  |

Table A.5.6.2.3.1-3: Cell specific test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T1 |  | T2 |  |
| AoA setup |  | Config 1,2 | Setup 3 as specified in clause A.3.15 |  |  |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |  |  |
| Assumption for UE beamsNote 7 |  | Config 1,2 | Rough |  |  | Rough |  |  |  |
| NR RF Channel Number |  | Config 1,2 | 1 |  |  | 2 |  |  |  |
| Duplex mode |  | Config 1,2 | TDD |  |  | TDD |  |  |  |
| BWchannel | MHz | Config 1,2 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |  |
| BWP BW | MHz | Config 1,2 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |  |
| TDD configuration |  | Config 1,2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |  |
| Initial DL BWP |  | Config 1,2 | DLBWP.0.1 |  |  | NA |  |  |  |
| Initial UL BWP |  | Config 1,2 | DLBWP.0.1 |  |  | N/A |  |  |  |
| Dedicated DL BWP |  | Config 1,2 | DLBWP.1.1 |  |  | NA |  |  |  |
| Dedicated UL BWP |  | Config 1,2 | ULBWP.1.1 |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | Config 1,2 | OP.1 |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,2 | SR.3.1 TDD |  |  | - |  |  |  |
| CORESET Reference Channel |  | Config 1,2 | CR.3.1 TDD |  |  | - |  |  |  |
| TRS configuration |  | Config 1,2 | TRS.2.1 TDD |  |  | NA |  |  |  |
| TCI configuration |  | Config 1,2 | CSI-RS.Config.0 |  |  | NA |  |  |  |
| SMTC configuration defined in A.3.11 |  | Config 1,2 | SMTC.1 |  |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2 | 120 |  |  | 120 |  |  |  |
| EPRE ratio of PSS to SSS |  | Config 1,2 | 0 |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Ês | dBm/SCS | Config 1 | -87 |  | -87 | -Infinity |  |  | -87 |
| SSB-RP Note 3 | dBm/SCS Note5 | Config 1,2 | -87 | -87 |  | -Infinity |  | -87 |  |
| BB Note 8 | dB | Config 1,2 | 1.89 | 1.89 |  | -Infinity |  | 1.89 |  |
| Io Note3 | dBm/95.04 MHz Note5 | Config 1,2 | -58.01 | -58.01 |  | -Infinity |  | -58.01 |  |
| Propagation Condition |  | Config 1,2 | AWGN |  |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB-RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 6: As observed with 0dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |  |

##### A.5.6.2.3.2	Test Requirements

In test 1 with per-UE gap and in test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

6720 for UE supporting power class 1, or

4160 for UE supporting other power class.

In test 1 and 2 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

------------------------------------------------------------- End of change 2 ------------------------------------------------------------

----------------------------------------------------- Beginning of Change 3 ------------------------------------------------------------

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)

A.7.6.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.7.6.2.1.1-1, A.7.6.2.1.1-2, and A.7.6.2.1.1-3.

Measurement gap pattern configuration # 13 as defined in Table A.7.6.2.1.1-2 is provided for UE that does not support per-FR gap and for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.2.1.1-1.

Table A.7.6.2.1.1-1 SA event triggered reporting tests without SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. |  |

Table A.7.6.2.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -30 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 5.2 for PC1; 3.5 for other PC |  |

Table A.7.6.2.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |  |
| AoA setup |  |  | Config 1 | Setup 3 as specified in clause A.3.15 |  |  |  |  |  |
|  |  |  |  | AoA1 |  | AoA2 |  |  |  |
| Beam AssumptionNote 7 |  |  | 1,2 | Rough |  | Rough |  |  |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |  |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |  |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |  |  |
| TCI configuration |  |  | Config 1 | CSI-RS.Config.0 |  | N/A |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1 | -87 | -87 | -Infinity |  |  | -87 |
| SSB-RP Note 3 |  | dBm/SCS Note5 | Config 1 | -87 | -87 | -Infinity |  | -87 |  |
| BB Note 8 |  | dB | Config 1 | 1.89 | 1.89 | -Infinity |  | 1.89 |  |
| Io Note3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.01 | -58.01 | -Infinity |  | -58.01 |  |
| Propagation Condition |  |  | Config 1 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB-RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |  |

A.7.6.2.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

The UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

------------------------------------------------------------- End of change 3 ------------------------------------------------------------

----------------------------------------------------- Beginning of Change 4 ------------------------------------------------------------

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)

A.7.6.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.7.6.2.3.1-1, A.7.6.2.3.1-2, and A.7.6.2.3.1-3.

Measurement gap pattern configuration # 13 as defined in Table A.7.6.2.3.1-2 is provided for UE that does not support per-FR gap and for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.2.3.1-1.

Table A.7.6.2.3.1-1: SA event triggered reporting tests with SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. |  |

Table A.7.6.2.3.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -30 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 7 for PC1; 4.5 for other PC |  |

Table A.7.6.2.3.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |  |
| AoA setup |  |  | Config 1 | Setup 3 as specified in clause A.3.15 |  |  |  |  |  |
|  |  |  |  | AoA1 |  | AoA2 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |  |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |  |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |  |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |  |  |
| TCI configuration |  |  | Config 1 | CSI-RS.Config.0 |  | N/A |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1 | -87 | -87 | -Infinity |  |  | -87 |
| SSB-RP Note 3 |  | dBm/SCS Note5 | Config 1 | -87 | -87 | -Infinity |  | -87 |  |
| BB Note 8 |  | dB | Config 1 | 1.89 | 1.89 | -Infinity |  | 1.89 |  |
| Io Note3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.01 | -58.01 | -Infinity |  | -58.01 |  |
| Propagation Condition |  |  | Config 1 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB-RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |  |

A.7.6.2.3.2	Test Requirements

In test 1 with per-UE gap and in test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

6720 for UE supporting power class 1, or

4160 for UE supporting other power class.

The UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

------------------------------------------------------------- End of change 4 ------------------------------------------------------------