3GPP TSG-RAN WG4 Meeting # 99-e                                                              R4-2107697

Electronic Meeting, 19– 27 April, 2021

Source:	Huawei, HiSilicon, KDDI

Title:	TP for TR 38.717-02-01 CA_n28A-n74A

Agenda item:	8.10.2

Document for:	Approval

1 Background

This contribution provides text proposal on the NR CA band combination CA_n28A-n74A as defined in New WID on NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2) RP-202199 [1].

2 Discussion

This inter-band ENDC combination is subjected to the 2nd, 4th and 5th order IM issues. We use main/diversity antenna topology as RF front-end architecture in our analysis, as shown in figure 1. The component linearity assumptions are listed in table 1, and the component attenuation and isolation values are listed in table 2.

Table 1 General linearity parameters

| Component | IP2(dBm) | IP4(dBm) | IP5(dBm) |
| --- | --- | --- | --- |
| Antenna switch | 112 | 56 | 53 |
| Diplexer | 115 | 55 | 53 |
| Duplexer | 115 | 55 | 53 |
| PA forward mixing | 28 | 30 | 28 |
| PA reverse mixing | 40 | 30 | 40 |
| LNA | 5 | -6 | -10 |

Figure 1 main/diversity antenna topology

Table 2 Attenuation and isolation values

| Attenuation and Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna ISO | 10 | Main antenna to diversity antenna |
| PA gain | 25 |  |
| PCB isolation Paout-Pain | 60 | PCB isolation (PA forward mixing) |
| n28 Tx attenuation at Band n74 Rx | 32 |  |
| n74 Tx attenuation at n28 RX | 30 |  |

Table 3 and Table 4 summarize the MSD analysis results for CA_n28-n74.

Table 3 MSD calculation for victim Rx of band 28

| Component | IMD2(dBm) |  | IMD4(dBm) |  |
| --- | --- | --- | --- | --- |
|  | Main_@Antenna | Div_@Antenna | Main_@Antenna | Div_@Antenna |
| n28 PA Forward mixing interference | -62.4 |  | -75.8 |  |
| n74 PA Forward mixing interference | -76.8 |  | -171.9 |  |
| n28 PA Reserve mixing interference | -91.9 |  | -93.9 |  |
| n74 PA Reserve mixing interference | -113.1 |  | -188.5 |  |
| n28 duplexer | -119.4 |  | -126.4 |  |
| B28 LNA via filters | -70.8 |  | -89.2 |  |
| B28 LNA via PCB coupling | -79.4 |  | -147.8 |  |
| Diplexer (L-H) | -73.4 | -94.2.2 | -81.8 | -124.2 |
| Antenna switch | -79.6 | -92.1 | -92.6 | -129.7 |
| interference From main path |  | -71.2 |  | -84.6 |
| Total | -61.2 | -71.02 | -74.6 | -84.6 |

Table 4 MSD requirements for band n28 Rx with UL band n28 and n74

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28-n74 | n28 | 705.5 | 5 | 25 | 760.5 | 24.6 | IMD2 |
|  | n74 | 1466 | 5 | 25 | 1514 | N/A | N/A |
| CA_n28-n74 | n28 | 743 | 5 | 25 | 798 | 11.3 | IMD4 |
|  | n74 | 1431 | 5 | 25 | 1479 | N/A | N/A |

Table 5 and Table 6 summarize the MSD analysis results for CA_n28-n74.

Table 5 MSD calculation for victim Rx of band 74

| Component | IMD4(dBm) |  |
| --- | --- | --- |
|  | Main_@Antenna | Div_@Antenna |
| n74 PA Forward mixing interference | -94.6 |  |
| B28 PA Forward mixing interference | -169.2 |  |
| n74 PA Reserve mixing interference | -71.6 |  |
| B28 PA Reserve mixing interference | -74 |  |
| n74 duplexer | -135.3 |  |
| n74 LNA via filters | -110.6 |  |
| n74 LNA via PCB coupling | -27 |  |
| Diplexer (L-H) | -83 | -124.5 |
| Antenna switch | -92.5 | -128.5 |
| interference From main path |  | -81.04 |
| Total | -71.04 | -81.04 |

Table 6 MSD requirements for CA_n28-n74

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28-n74 | n28 | 709 | 5 | 25 | 764 | N/A | N/A |
|  | n74 | 1466 | 5 | 25 | 1514 | 14.6 | IMD4 |

Text Proposal

---Start of changes---

6.X	CA_n28-n74

6.X.1	Common for 1 band UL and 2 bands UL CA

6.X.1.1 Operating bands for CA

Table 6.X.1.1-1: CA band combination CA_n28A-n74A

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n74 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n74 | 1427 MHz | – | 1470 MHz | 1475 MHz | – | 1518 MHz | FDD |

6.X.1.2	Channel bandwidths per operating band for CA

Table 6.X.1.2-1: Supported bandwidths per CA band combination CA_n28A-n74A

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n28A-n74A | - | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n74 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

6.X.1.3	UE Co-existence studies

Table 6.X.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n28-n74.

Table 6.X.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2854 | 2940 | 4281 | 4410 | 5708 | 5880 |

Based on above table, 2nd harmonic of band n28 UL fall into band n74 DL.

Table 6.X.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2950 | 3036 | 4425 | 4554 | 5900 | 6072 |

Based on above table, there is no harmonic mixing issue.

6.X.1.4	∆TIB and ∆RIB values

For CA_n28-n74 , the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TS 38.101-3 DC_11-n28 and TS 36.101 CA_11-28, CA_21-28 relaxation values.

Table 6.X.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28-n74 | n28 | 0.6 |
|  | n74 | 0.4 |

Table 6.X.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n28-n74 | n28 | 0.2 |
|  | n74 | 0 |

6.X.1.5	REFSENs requirements

Regarding MSD due to harmonics of n28 UL fall into n74 DL, the exceptions are needed. The reference sensitivity exceptions are proposed in Table 6.X.1.5-1 with uplink configuration specified in Table 6.X.1.5-2.

Table 6.X.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n28 | n741,2 | 23.1 | 19.8 | 18 | 16.8 |  |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n48, CA_n25-n78, CA_n28-n74, CA_n48-n66, CA_n66-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.X.1.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n28 | n74 | 12 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |

6.X.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirement for CA_n28-n74.

6.X.2	Specific for 2 bands UL CA

6.X.2.1	Maximum output power for inter-band CA

Table 6.X.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n28A-n74A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.X.2.2	UE co-existence studies

Table 6.X.2.1-1 lists Band n28 +Band n74 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.X.2.2-1: Band n28 and Band n74 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 703 | 748 | 1427 | 1470 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 679 | 767 | 2130 | 2218 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 64 | 69 | 2106 | 2237 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 2833 | 2966 | 3557 | 3688 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 639 | 817 | 3533 | 3707 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 3536 | 3714 | 4984 | 5158 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 1358 | 1534 | 4260 | 4436 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 4960 | 5177 | 1342 | 1565 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 6411 | 6628 | 4239 | 4462 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 2785 | 3004 | 610 | 831 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 5687 | 5906 | 4963 | 5184 |

Based on above, 2nd,4th,5th IMD of n28 UL and n74 UL fall into n28 DL, and 4th,5th IMD of n28 UL and n74 UL fall into n74 DL.

Table 6.X.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.X.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n28-n74 | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 26, 31, 34, 38, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 15 |
|  | E-UTRA Band 4, 42, 43, 52, 65, 66 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20, 2 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 21, 2 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4, 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 20: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. NOTE 21: Applicable for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for 10 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth |  |  |  |  |  |  |  |

6.X.2.3	REFSENS requirements

Based on Table 6.X.2.2-1, MSD due to IMD is specified below:

Table 6.X.2.3-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28A-n74A | n28 | 705.5 | 5 | 25 | 760.5 | 24.6 | FDD | IMD2 |
|  | n74 | 1466 | 5 | 25 | 1514 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 11.3 | FDD | IMD4X |
|  | n74 | 1431 | 5 | 25 | 1479 | N/A | FDD | N/A |
|  | n28 | 709 | 5 | 25 | 764 | N/A | FDD | N/A |
|  | n74 | 1466 | 5 | 25 | 1514 | 14.6 | FDD | IMD4 |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | FDD | N/A |
|  | n74 | 1450.4 | 5 | 25 | 1498.4 | 2.5 | FDD | IMD5 |
| NOTE X: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

---End of changes---

Reference

[1]	RP-210495, “Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation