# 3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2107780

Electronic Meeting, May 19th - 27th, 2021

Title:	[Draft] Further Reply LS on power control for NR-DC

Response to:	(R1-2104018) Further Reply LS on power control for NR-DC

Release:	Rel-16

Work Item:	LTE_NR_DC_CA_enh-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name: Sanjun Feng

E-mail Address: fengsanjun@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN1 for the Further Reply LS on power control for NR-DC.

Regarding the feasibility of independent power control for the mentioned two cases:

1) uplink CCs of MCG and uplink CCs of SCG are in different frequency bands in FR2.

2) uplink CCs of MCG and uplink CCs of SCG are in the same frequency band in FR2.

RAN4 has been discussed the power limitation for FR2 in R17 WI for Inter-band UL CA for different frequency group (i.e., 28GHz + 40GHz) and just agreed that both the max TRP and max EIRP limitation are per-band. Based on this agreement, for case 1, no common power limitation exists and no power sharing would be needed, and the UE performs transmission power control independently.

For case 2, there is max TRP and max EIRP limitation per band as specified in section 6.2A.4 of TS 38.101-2, thus power sharing is needed in case the maximum power limitation was reached, this can be interpreted as UE not being able to perform independent transmission power control.

In addition, RAN4 do not plan to discuss p-NR-FR2 or p-UE-FR2 in Rel-17.

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to take the above information into account.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #100-e	23 - 27 Aug 2021         E-meeting

TSG-RAN WG4 Meeting #101-e	1 - 12 Nov 2021         E-meeting