3GPP TSG-RAN WG4 Meeting # 99-e 												R4-2107925

Electronic Meeting, 19th – 27th May, 2021						(Revision of R4-2107641)

Agenda item:			8.7, 11.8

Source:	Moderator (Ligado Networks)

Title:	Email discussion summary for [99-e][115] NR_LTE_band_n24

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Background:

Agenda Items 8.7 is related to the recently introduced NR bands, n24.  Agenda Item 11.8 is related to the recent modifications to LTE Band 24 due to regulatory updates.

CRs for both WIs were approved at RAN#91e meeting.  At the RAN4#98-bis-e meeting, further progress was made via the WF R4-2105371.

Scope

To review and agree to the submitted CRs based on the WF agreements.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

Discussion structure:

Topic 1: Review and agree to submitted CRs

- 2nd round: TBA

# Topic #1: Review and agree to submitted CRs

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

Moderator’s note: Cat A CRs are not listed.

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2108986 | Ligado Networks | Summary of change to 38.101-1: Finalization of A-MPR regions and values, assignment of different NS value to avoid conflict with one assigned for V2X bands and update to the notes as agreed to in WF tdoc R4-2105371 |
| R4-2108987 | Ligado Networks | Summary of change to 36.101: Finalization of A-MPR regions and values, assignment of different NS value to avoid conflict with one assigned for V2X bands and update to the notes as agreed to in WF tdoc R4-2105371 |

## Companies views’ collection for 1st round

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2108986 | Nokia: You have changes on changes i.e. you added and then deleted a sentence. This is not ok. CR cannot be implemented by MCC. Take clean spec as a baseline. |
|  | Qualcomm: The MOP is specified as 23 dBm +2/-2, but when I checked the Tx filter IL, I found it to be 5 dB. With such large TxIL, it is not feasible to achieve this MOP. Was there other filter data that showed acceptable TxIL to enable MOP? |
|  | Ligado: The average max Tx IL of the filter data from 4 vendors submitted in R4-2014466 was 3.7 dB. |
|  | Huawei: We think that 3.7dB is OK according to proposed contribution. As for Nokia’s comments, it is better to revised the specs one time and one author for further processing. |
|  | Skyworks: filter data was looked at from different vendors and at the time the worst case filter insertion losses was 3dB which is comparable with other bands. |
| R4-2108987 | Nokia: You have changes on changes i.e. you added and then deleted a sentence. This is not ok. CR cannot be implemented by MCC. Take clean spec as a baseline. |
|  | Qualcomm: The A-MPR for region A is 3 dB but for the same region, SCS for NR, the A-MPR is 14 dB. Why is the A-MPR for LTE 11 dB lower than for NR? |
|  | Ligado: This is based on the simulation results and proposal presented in R4-2100134 and R4-2100135 at the RAN4#98e meeting. R4-2100545 describes the reasoning of lower A-MPR for LTE compared to NR. |
|  | Huawei: Do we only change 36.101(Rel-10)? For Rel-11 to Rel-17 of LTE, are there other needed CRs whose category is ‘A’? |
|  | Skyworks: the critical point is for requirement just at the band edge, for 10MHz LTE SU is 50RB vs 52RB for NR thus the reduced AMPR thanks to larger guard band |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1.1 | Moderator summary - Clarification on max Tx IL of the duplexer, unrelated to the technical changes in the CR, was requested and was provided. - Clarification was requested on LTE A-MPR values in CR R4-2108987 and was provided. Tentative agreements: Technical changes proposed in CR R4-2108986 seem to be agreeable. Except for LTE A-MPR Region A, technical changes proposed in CR R4-2108987 seem to be agreeable. Candidate options: Recommendations for 2nd round: - Address the change over change issue raised by Nokia and Huawei in revised CRs. The meeting date on the cover page needs to be also updated in the revised CRs. - Continue to review LTE A-MPR Region A value in Round 2. - Based on filter data provided, review MOP tolerances in 36.101 and 38.101-1 and determine if updates are necessary. Any agreement resulting from the discussion should be incorporated in the revised CRs. - Cat A CRs for 36.101 Rel 11-17 to be uploaded only after the revised CR for Rel-10 is agreed to. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2108986 | To be revised |
| R4-2108987 | To be revised |

## Discussion on 2nd round

### Sub-topic 2-1

Sub-topic description: Discuss MOP tolerances specified for band 24 and n24 in 36.101 and 38.101-1 and determine if further updates are necessary.  Given that both LTE_band24_mod and NR_band_n24 are targeted to be completed in June 2021, any agreements from the discussion should be incorporated in the revisions to R4-2108986 and R4-2108987.

### Sub-topic 2-2

Sub-topic description: Discuss LTE A-MPR value for Region A and determine if further updates are necessary to the Region A A-MPR in the revision of R4-2108987.

### Sub-topic 2-3

Sub-topic description: If updates to MOP tolerances are determined to be needed for n24, should the revision of R4-2108986 also incorporate similar updates for SUL band n99 defined for the UL of n24?

### Sub-topic 2-4

Sub-topic description: Review the revised CRs and confirm that the change over change issues have been addressed.

## Companies views’ collection for 2nd round

Sub topic 2-1

| Company | Comments |
| --- | --- |
| Nokia | Normal MOP tolerances can be used also for this band |
| Qualcomm | Given the excessive Tx IL of the filter, we suggest setting the MOP tolerance to +2/-3 and applying dTC to this band. |

Sub topic 2-2

| Company | Comments |
| --- | --- |
| Nokia | LTE A-MPR is based on simulations, as Skyworks pointed out LTE has smaller SU |
| Qualcomm | For LTE A-MPR we suggest the following. A new Region A should be created for 10 MHz and the existing regions shifted and reindexed starting from B. Channel bandwidth confined to 1627.5- 1637.5MHz. Channel bandwidth; Carrier centre frequency (FC) (MHz); Parameters; Region A; Region B; Region C; Region D; Region E; Region F. 5 MHz; 1630.0, 1630.3; RBstart; N/A; ≤ 8; ≤ 8; N/A; N/A; N/A; N/A. LCRB [RBs]; N/A; ≤ 8; > 8; N/A; N/A; N/A; N/A. A-MPR [dB]; N/A; [8]; 2; N/A; N/A; N/A; N/A. 1635.0; No A-MPR needed. 1649.0. 1654.0. 10 MHz; 1632.5; RBstart; ≤ 5; ≤ 18; ≤ 18; ≥ 35; ≥ 35; ≥ 40; ≥ 40. LCRB [RBs]; ≤ 5; ≤ 12; > 12; ≤ 7; > 7; ≤ 7; > 7. A-MPR [dB]; [7]; 5; 7; 4; 2; 5; 3. 1651.5; No A-MPR needed. |

Sub topic 2-3

| Company | Comments |
| --- | --- |
| Qualcomm | Yes |

Sub topic 2-4

| CR/TP number | Comments collection |
| --- | --- |
| R4-217800 (Rev of R4-2108986) | Nokia: We support this CR |
|  | Qualcomm: Revision needed |
| R4-217801 (Rev of R4-2108987 |  |
|  | Nokia: We support this CR |
|  | Qualcomm: Revision needed |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2108986 | CR for updates related to n24 in 38.101-1 | Ligado Networks | Revised | Revised to R4-2107800 |
| R4-2108987 | CR for updates related to LTE band 24 in 36.101 (Rel-10) | Ligado Networks | Revised | Revised to R4-2107801 |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| Revised to R4-2107800 | CR for updates related to n24 in 38.101-1 | Ligado Networks | Agreeable | Change section for suggested MOP tolerance updates were added. Changes were made for both n24/n99. Change over change issues identified in Round 1 were resolved |
| Revised to R4-2107801 | CR for updates related to LTE band 24 in 36.101 (Rel-10) | Ligado Networks | Agreeable | Change section for suggested MOP tolerance updates were added; A-MPR region/values were updated with suggested values; change over change issues identified in Round 1 were resolved Cat A CRs to be uploaded towards the end of meeting. |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents