3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2107931

Electronic Meeting, 19– 27 May, 2021

Agenda item:			8.28.2, 8.28.3

Source:	Moderator (Huawei)

Title:	Email discussion summary for [99-e][121] NR_FR1_35MHz_45MHz_BW

Document for:	Information

Introduction

The scope of this email discussion is to discuss the contributions submitted at agenda 8.28.2 and 8.28.3 on introduction of channel bandwidths 35MHz and 45MHz for NR. The following topics are discussed in the email discussion.

- Topic#1: UE RF requirements

- Topic#2: BS CRs

Topic #1: UE RF requirements

Companies’ contributions summary

UE REFSENS

| T-doc number | Company | Proposal summary |
| --- | --- | --- |
| R4-2110657 | Huawei, HiSilicon | On asymmetric Uplink /Downlink scenarios, Proposal 1: The following note is added for the REFSENS for band n8/n71 35MHz, and n2/n25 45MHz NOTE: the requirement is met for a default TX–RX frequency separation specified in Table 5.4.4-1. Proposal 2: The widest channel bandwidth is applicable only to downlink should be clarified in Table Table 5.3.5-1 with NOTE3. NOTE 3: This UE channel bandwidth is applicable only to downlink. |
| R4-2111471 | Qualcomm Incorporated | On 35MHz and 45MHz REFSENS Observation 1: The “best” option and unrestricted option provide similar REFSENS. Proposal 1: Consider 25MHz, “best” 40MHzUL/45MHz DL, or unrestricted 45MHz UL BW option for n2 REFSENS. Proposal 2: Consider 25MHz, “best” 40MHzUL/45MHz DL, or unrestricted 45MHz UL BW option for n25 REFSENS. Proposal 3: Consider 35MHz, “best” 20MHzUL/35MHz DL, or unrestricted 35MHz UL BW option for n71 REFSENS. Proposal 4: Consider 35MHz, “best” 20MHzUL/35MHz DL, or unrestricted 35MHz UL BW option for n71 REFSENS. |
| R4-2111530 | Skyworks Solutions Inc. | On remaining n8 n71 Asymmetric Uplink Downlink Requirements Proposal 1: Adopt changes highlighted in yellow in Table 2 and Table 3 for introduction of n8 and n71 asymmetric UL 20MHz/DL 35MHz operation. Table 2: Proposal for Table 5.3.5-1 - Channel bandwidths for NR band n8 and n71. NR band / SCS / UE Channel bandwidth. NR Band; SCS kHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 35 MHz; 40 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. n8; 15; Yes; Yes; Yes; Yes; Yes3,4. 30; Yes; Yes; Yes; Yes3,4. 60. n71; 15; Yes; Yes; Yes; Yes; Yes3,4. 30; Yes; Yes; Yes; Yes3,4. 60. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. N. Table 3: Proposal “Table 5.3.6-1 FDD asymmetric UL and DL channel bandwidth combinations” for n8 and n71. NR Band; Channel bandwidths for UL (MHz); Channel bandwidths for DL (MHz); Asymmetric channel bandwidth combination set. n8; 20; 35; 0. n71; 5; 10; 0. 10; 15. 15; 20. 5; 10; 1. 10; 15. 15; 20. 20; 35. NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4.. Proposal 2: Include the word “maximum” in the following text in sub-clause 5.3.6, as highlighted in yellow: The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth. In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in Table 5.4.4-1) as follows: ΔFTX-RX = ¦ (BWDL – BWUL)/2 ¦ Proposal 3: Add text highlighted in yellow to sub-clause 3.2 ΔFTX-RX Maximum deviation to the Tx-Rx carrier center frequency separation for asymmetric uplink/downlink channel bandwidth operation |

REFSENS table split and simplication

| T-doc number | Company | Proposal summary |
| --- | --- | --- |
| R4-2111471 | Qualcomm Incorporated | On REFSENS simplification Proposal 5: Keep REFSENS simplification for TDD bands only. |
| R4-2109418 | ZTE Wistron Telecom AB | A unified equation for specifying REFSENS for both TDD and FDD bands REFSENS=-100+10∙log10NRB∙SCSkHz∙124500+∆R |
| R4-2109419 | ZTE Wistron Telecom AB | Draft CR for specifying REFSENS based on a unified equation method |
| R4-2110160 | Apple | CR for TS 38.101-1: FR1 REFSENS table split and simplication |
| R4-2111149 | Ericsson | REFSENS table structure, band groups Observation 1: Changing the REFSENS table structure in TS38.101-1 will have a large impact on the corresponding RAN5 test specification TS38.521-1. Observation 2: With a formula based REFSENS requirement table RAN5 will most probably anyway need to calculate and present the actual values as part of the test case in question. Observation 3: An Operating band grouping concept can be introduced to define the REFSENS requirement values that is more readable than a formula-based approach. Proposal 1: Send an LS to RAN5 informing them on the split of REFSENS tables for FDD and TDD and ask for their feedback on formula based TDD table vs. band grouping concept |
| R4-2111538 | Skyworks Solutions Inc. | FR1 FDD SDL REFSENS Table Simplification Proposal: REFSENS Table is split into two groups: with MSD/without MSD |

38.101-1 CR

| T-doc number | Company | Proposal summary |
| --- | --- | --- |
| R4-2111150 | Ericsson | Draft CR to add 35 and 45MHz BW to TS38.101-1 Withdrawn |

Open issues summary

Sub-topic 1-1

Issue 1-1:  REFSENS for Asymmetric Uplink Downlink

- Proposals summary

Option 1:

- To add a new entry in "Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations"

- To add a note in Table 5.3.5-1 to state that UE channel bandwidth is applicable only to downlink

- To include the word “maximum” deviation definition to ΔFTX-RX

Option 2:

- To state that the REFSENS requirement is met for a default specific duplex spacing in the general REFSENS section

- To add a note in Table 5.3.5-1 to state that UE channel bandwidth is applicable only to downlink

Option 3:

- Consider “best” option with asymmetric Uplink Downlink, or unrestricted UL BW option.

- Recommended WF

  - Discussion on the options and check whether consensus can be reached at 1st round discussion.

Sub-topic 1-2

Issue 1-2:  REFSENS table split and simplication

- Proposals summary

- On REFSENS table split

- Option 1: one table for TDD bands and the other table for FDD bands

- Option 2: one table for FDD bands with MSD and the other table for FR1 bands without MSD

- Option 3: one table for both TDD and FDD bands

- On REFSENS simplication

  - Option 1: equation-based REFSENS representation

  - Option 2: band grouping

  - Option 3: Ask for RAN5 feedback

- Recommended WF

  - Discussion on the options and check whether consensus can be reached at 1st round discussion.

Sub-topic 1-3

Issue 1-3: CR to 38.101-1

There is no big CR submitted for this meeting. We need one company to draft the big CR for the WI

- Recommended WF

  - Welcome the volunteer to draft big CR to 38.101-1.

Companies views’ collection for 1st round

Open issues

Sub-topic 1-1: REFSENS for Asymmetric Uplink Downlink

| Company | Comments |
| --- | --- |
| Huawei | Option 2 Unrestricted UL BW option is not preferred since it will need also UL A-MPR for some cases |
| ZTE | Option 2. In some sense the current specs already cover the case of asymmetric UL/DL channel bandwidth as long as the UL channel bandwidth is no larger than the DL channel bandwidth in TS 38.101-1: The reference receive sensitivity (REFSENS) requirements specified in Table 7.3.2-1 and Table 7.3.2-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3. |
| Skyworks | Option 1. The argument proposed in R4-2110657 for not proposing option 1 is not entirely true. The bands which are specified in the asymmetric channel bandwidth table 5.3.6-1 are not always bands which have asymmetric UL and DL spectrum. For example, band n71 does not have that UL/DL asymmetric band definition, and yet it is specified in Table 5.3.6-1. Retracing the early history of Release 15, it is our understanding that parameter DeltaFtx-rx was specifically introduced to account for cases where the UL/DL CBW are asymmetric. This parameter was introduced to handle all combinations of asymmetry (i.e. UL CBW > DL CBW or DL CBW>UL CBW), without any restriction on the UL/DL band spectrum definition. This is why we believe option 1 is the right way to capture the introduction of asymmetric UL/DL CBW operation for bands n8 and n71 UL 20/DL35 MHz. Also note that it is likely we have more cases of UL/DL asymmetric CBW coming, so we think option 1 is the future proof way that takes care of such new requests. Option 1 also has the benefit of being more explicit than using option 2 footnote approach. |
| MediaTek | Option 1 make sense to us. We are open to option2. Regarding option3, if there is unrestricted UL BW option(35MHz), then impact for UL A-MPR and coexistence may need further discussion. |
| Qualcomm | There is a typo in the proposals. The word “25MHz” should not be there. My apologies. WE only suggest option 3 as to not restrict spectrum from an operator point of view. In future, UE devices performance could get better. Resource blocks can be positioned to get the same or similar REFSENS and AMPR can be specified even if work is required. But if there is no agreement, then our preference is the “best” option for asymmetric case to align with 1UL/DLCA. |

Sub-topic 1-2: REFSENS table split and simplication

| Company | Comments |
| --- | --- |
| Huawei | On REFSENS table split We are ok to Option 1 or Option 2 On REFSENS simplication We suport Option 1 or keep existing approach |
| ZTE | On REFSENS table split: As proponent of Option 3 (a unified equation for both TDD and FDD bands), we think this simplification can solve the displaying issue of the current specs on one hand, and on the other hand also keep the same table so to avoid impacts on the corresponding RAN5 test specification TS38.521-1 which is observed in Ericsson’s paper. On REFSENS simplication: Option 1 equation-based approach. |
| Skyworks | On REFSENS table split: Option 2 as an extension of option 1. What we mean here is that, if agreed, our equation-based option 2 proposal should be read like an extension of option 1 as: - a direct extension of CR R4-2110160 Table 7.3.2-1b for TDD bands: our proposal is to add to this table of TDD band equation based REFSENS levels, all FDD bands to which the same rule works well – and adopt our equation proposals. - a direct reduction/shrink of CR R4-2110160 Table 7.3.2-1a: our proposal is to keep in this table only the FDD bands for which MSD occurs. For Option 3: this seems to produce two tables for TDD bands where CR R4-2110160 manages to achieve this result in a single compact table. On REFSENS simplification: Option 1. Question for clarification on R4-2109418: Thank you for the unified equation proposal. How is the MSD taken into account for bands like n12 at CBW 15MHz? It seems the proposed equation only accounts for RB scaling and so Tx excess noise or C-IM related MSD does not seem to be accounted for. |
| Ericsson | On REFSENS table split: We are fine with both Option 1 or Option 2 On REFSENS simplification: Option 2 and Option 3 (obviously since it’s our suggestions) However not against Option 1, but readability and RAN5 impact needs to be addressed. We do not see this REFSENS table split/simplification tightly connected to this W.I (even if the idea occurred here) This Spec improvement can also be done as maintenance, so await a RAN5 LS response should not affect the completion of this WI. |
| Qualcomm | On REFSENS table split: Option 1 for TDD bands only. On REFSENS simplification: Option 1: Equation based for TDD bands only. |
| Apple | Option 1 is our preference. The FDD band table also includes SDL bands. For TDD bands, use equation-based representation for REFSENS. We also appreciate both ZTE’s and Skyworks’s proposals with the intent to further simplify the table format and improve future spec. editability. For the unified equation proposal, we are not sure if the REFSENS offset can be a constant value for all channel BWs for FDD bands. Since the current REFSENS table in FR1 specifications is already showing the readability issue as highlighted in WF R4-2105365, if Option 1 can be acceptable, we suggest to agree the CR based on Option 1 in this meeting as it can greatly ease the CR efforts for introducing 35MHz and 45MHz requirements into 38.101-1 where REFSENS is one of the most concerned requirements. |

Sub-topic 1-3:  CR to 38.101-1

| Company | Comments |
| --- | --- |
| Moderator | One volunteer is needed |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 1-1: REFSENS for Asymmetric Uplink Downlink | Status: Based on 1st round discussion, companies’ view is still diverse Option 1: Skyworks, MediaTek Option 2: Huawei, ZTE Option 3 (“best case” option): Qualcomm Recommendations for 2nd round: further discussion on the options above and capture the agreements in WF1. |
| Sub-topic 1-2: REFSENS table split and simplication | Status: Based on 1st round discussion, companies’ view is still diverse on the table split. For REFSENS simplication, it seems option 1 is agreeable meanwhile RAN5 impact needs to be further discussed. - On REFSENS table split - Option 1: Huawei, Ericsson, Qualcomm, Apple - Option 2: Huawei, Skyworks, Ericsson - Option 3: ZTE - On REFSENS simplication - Option 1: Huawei, ZTE, Skyworks, Qualcomm, Apple - Option 2: Ericsson - Option 3: Ericsson Tentative agreements: - On REFSENS simplication - Option 1: equation-based REFSENS representation Recommendations for 2nd round: further discussion on the options for REFSENS table split and discussion on the impact to RAN5 for REFSENS simplication, and capture the agreements in WF2. |

Discussion on 2nd round (if applicable)

Companies views’ collection for 2nd round

WF1：R4-2107819 WF on REFSENS for Asymmetric Uplink Downlink，Huawei

| Company | Comments |
| --- | --- |
| Skyworks | We support this WF and would like to co-sign. Our preference is for option 1. We can offer to bring CRs at next meeting for the agreed changes (slide 2). |
| Huawei | Option 1 is also ok to us since it is more explicit and future proof |
| MediaTek | We support the WF. Option1 is our preference. |
| Apple | Option is also our preference. |
| Qualcomm | Option 3 was our preference, but I would like to see the “middle” options averaged and the “best” options averaged. We will be flexible based on the outcome as well as how we define the requirement in the spec. For the WF, don’t we need some note on how the REFSENS is specified such as the location of the TX channel BW in relation to the RX channel BW? |

WF2：R4-2107820 WF on REFSENS table split and simplication, Apple

| Company | Comments |
| --- | --- |
| Skyworks | We support this WF and would like to co-sign. Our preference is for option 2. We may come up at next meeting with suggestions to further compress table size while retaining the same level of accuracy as option 1 and option 2. Would it be acceptable to include say, an option 4 that would welcome other proposals? For option 3, we would like to understand how is receiver self-desense due to C-IM or Tx excess noise accounted for? Our limited understanding of option 3 is that only RB scaling is accounted for. We would like to kindly ask for clarification on this option. |
| Huawei | We support option 1 or 2 and slightly prefer option 1 |
| T-Mobile USA | The columns in Table 7.3.2-1are twice as wide as they need to be. The columns could be narrowed to allow for room for 35 and 45 MHz. via an editorial change. We have uploaded an example to the inbox. |
| Apple | For Option 3, we also made similar comments as Skyworks in first round discussions. For the editorial approach as suggested by T-Mobile USA, it may still work based on the current specifications. The concern would be we will soon add 35MHz and 45MHz into the specifications which will need to further compress the column width. If in future we have more new channel BWs to be introduced (such as 55 MHz, 65 MHz, etc., though we do not encourage to introduce more new channel BWs), the exiting table format may not be able to accommodate them without impacting the spec. readability. That is the reason this table simplification exercise was motivated. |
| Qualcomm | Option 2 may not be that simple, but can be studied further. The same or proportional offset may not be easily derived over BW when another distortion mechanism kicks in. Equation based REFSENS for all bands are fine up to certain limited BW in the FDD bands. A 2nd table for FDD bands above a certain BW that requires MSD. Perhaps we can split 2 tables this way. Option 5??? The sky is the limit 😊. The boundary condition for such a table would be 5th order distortion for those suspect FDD bands. |
| Ericsson | We are fine with the WF, will further communicate with RAN5 internally for the next meeting. T-Mobile USA comment is also valid another easy editorial approach would be to split the table in tow one with BW’s up to 50MHz and another from 50MHz to 100MHz. Essentially creating two tables halw as wide as the existing one. |
| Skyworks | We are fine with the WF, thank you for adding us as co-authors. |

Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| WF R4-2107819 | Based on the 2nd round discussion, the WF captured companies’ comments. To be approved |
| WF R4-2107820 | Based on the 2nd round discussion, the WF captured companies’ comments. To be approved |

Topic #2: BS CRs

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2110485 | Ericsson | CR to TS 37.105: Intoduction of 35 MHz and 45 MHz |
| R4-2110486 | Ericsson | CR to TS 38.141-1: Introduction of CBWs 35 MHz and 45 MHz |
| R4-2110598 | ZTE Corporation | CR to TS 38.141-2: Introduction of 35MHz and 45MHz |
| R4-2110599 | ZTE Corporation | CR to TS 37.145-1: introduction of 35MHz and 45MHz |
| R4-2110659 | Huawei, HiSilicon | CR for TS 37.141: introduction of channel bandwidths 35MHz and 45MHz |
| R4-2110660 | Huawei, HiSilicon | CR for TS 37.145-2: introduction of channel bandwidths 35MHz and 45MHz |
| R4-2110747 | Huawei, HiSilicon | CR for TS 38.104: introduction of channel bandwidths 35MHz and 45MHz |
| R4-2111222 | Nokia, Nokia Shanghai Bell | CR to 37.104: Introduction of requirements for 35 and 45MHz channel bandwidths |

Companies views’ collection for 1st round

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2110485 |  |
| R4-2110486 |  |
| R4-2110598 |  |
| R4-2110599 |  |
| R4-2110659 |  |
| R4-2110660 |  |
| R4-2110747 |  |
| R4-2111222 |  |

Summary for 1st round

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| BS CRs | Status: The CRs are based endorsed draft CR and no comments received in 1st round, which should be agreeable. However, there are still some open issue on UE RF requirements and no UE big CR is submitted for the meeting. Recommendations for 2nd round: Return to all BS CRs |

Discussion on 2nd round (if applicable)

| CR/TP number | Comments collection |
| --- | --- |
| R4-2110485 | Huawei: we propose to postpone all BS CRs since there are a few UE RF requirements cannot be finalized |
| R4-2110486 | Huawei: we propose to postpone all BS CRs since there are a few UE RF requirements cannot be finalized |
| R4-2110598 | Huawei: we propose to postpone all BS CRs since there are a few UE RF requirements cannot be finalized |
| R4-2110599 | Huawei: we propose to postpone all BS CRs since there are a few UE RF requirements cannot be finalized |
| R4-2110659 | Huawei: we propose to postpone all BS CRs since there are a few UE RF requirements cannot be finalized |
| R4-2110660 | Huawei: we propose to postpone all BS CRs since there are a few UE RF requirements cannot be finalized |
| R4-2110747 | Huawei: we propose to postpone all BS CRs since there are a few UE RF requirements cannot be finalized |
| R4-2111222 | Huawei: we propose to postpone all BS CRs since there are a few UE RF requirements cannot be finalized |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| BS CRs | Since the UE CR cannot be finalized in this meeting, moderator suggest to postpone all BS CRs. |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on REFSENS for Asymmetric Uplink Downlink | Huawei | WF1 |
| WF on REFSENS table split and simplication | Apple | WF2 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2110160 | CR for TS 38.101-1: FR1 REFSENS table split and simplication | Apple | not pursued |  |
| R4-2110657 | On asymmetric Uplink /Downlink scenarios | Huawei, HiSilicon | Noted |  |
| R4-2111149 | REFSENS table structure, band groups | Ericsson | Noted |  |
| R4-2111150 | Draft CR to add 35 and 45MHz BW to TS38.101-1 | Ericsson | Withdrawn |  |
| R4-2111471 | 35MHz and 45MHz REFSENS | Qualcomm Incorporated | Noted |  |
| R4-2111530 | Remaining n8 n71 Asymmetric Uplink Downlink Requirements | Skyworks Solutions Inc. | Noted |  |
| R4-2111538 | FR1 FDD SDL REFSENS Table Simplification | Skyworks Solutions Inc. | Noted |  |
| R4-2109418 | A unified equation for specifying REFSENS for both TDD and FDD bands | ZTE Wistron Telecom AB | Noted |  |
| R4-2109419 | Draft CR for specifying REFSENS based on a unified equation method | ZTE Wistron Telecom AB | Noted |  |
| R4-2110485 | CR to TS 37.105: Intoduction of 35 MHz and 45 MHz | Ericsson | Return to |  |
| R4-2110486 | CR to TS 38.141-1: Introduction of CBWs 35 MHz and 45 MHz | Ericsson | Return to |  |
| R4-2110598 | CR to TS 38.141-2: Introduction of 35MHz and 45MHz | ZTE Corporation | Return to |  |
| R4-2110599 | CR to TS 37.145-1: introduction of 35MHz and 45MHz | ZTE Corporation | Return to |  |
| R4-2110659 | CR for TS 37.141: introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | Return to |  |
| R4-2110660 | CR for TS 37.145-2: introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | Return to |  |
| R4-2110747 | CR for TS 38.104: introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | Return to |  |
| R4-2111222 | CR to 37.104: Introduction of requirements for 35 and 45MHz channel bandwidths | Nokia, Nokia Shanghai Bell | Return to |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2107819 | WF on REFSENS for Asymmetric Uplink Downlink | Huawei, HiSilicon, Skyworks Solutions Inc., | Agreeable |  |
| R4-2107820 | WF on REFSENS table split and simplication | Apple, Skyworks Solutions Inc., ZTE | Agreeable |  |
| R4-2110485 | CR to TS 37.105: Intoduction of 35 MHz and 45 MHz | Ericsson | Postponed |  |
| R4-2110486 | CR to TS 38.141-1: Introduction of CBWs 35 MHz and 45 MHz | Ericsson | Postponed |  |
| R4-2110598 | CR to TS 38.141-2: Introduction of 35MHz and 45MHz | ZTE Corporation | Postponed |  |
| R4-2110599 | CR to TS 37.145-1: introduction of 35MHz and 45MHz | ZTE Corporation | Postponed |  |
| R4-2110659 | CR for TS 37.141: introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | Postponed |  |
| R4-2110660 | CR for TS 37.145-2: introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | Postponed |  |
| R4-2110747 | CR for TS 38.104: introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | Postponed |  |
| R4-2111222 | CR to 37.104: Introduction of requirements for 35 and 45MHz channel bandwidths | Nokia, Nokia Shanghai Bell | Postponed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents