3GPP TSG-RAN WG4 Meeting # 99-e	                                                                   R4-2107935

Electronic Meeting, May. 19-27, 2021

Agenda item:			8.33, 8.34

Source:	Moderator (CMCC)

Title:             	         Email discussion summary for [99-e][125] HPUE_n34_n39

Document for:	Information

Introduction

WI(s) on introduction of PC2 n34 band (RP-210844) and PC2 n39 band (RP-210845) for NR was approved in RAN#91-e meeting.

This email discussion includes contributions in agenda 8.33 and 8.34, the targets of email discussion based on companies’ contributions submitted in this e-meeting are as below:

- 1st round:

  - Discuss RF requirements for PC2 n34 and n39, and provide comments on the CRs and contributions.

- 2nd round:

  - Strive to approve CRs or WF.

Topic #1: Introduction of PC2 n34 for NR

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2108975 | CMCC | Proposal 1: The MOP and Tolerance for single antenna port are to be specified as 26dBm ±2 dB for band n34 of power class 2, and The MOP and Tolerance for UL MIMO are to be specified as 26dBm +2/-3 dB for n34 of power class 2. |
| R4-2108943 | CMCC | CR on PC2 UE RF requirements of n34 in Rel-17 TS 38.101-1 |
| R4-2109003 | Nokia, Nokia Shanghai Bell | Proposal: PC2 n34/n39 MOP lower tolerance should be +2/-2 dB. |
| R4-2109677 | vivo | Proposal 1: Considering the power consumption and extra insertion loss of PC 2 UE, the power tolerance +2/-3 for n34 is proposed. |
| R4-2110474 | ZTE Corporation | Proposal 1: Option1, i.e. The MOP and Tolerance for single antenna port are to be specified as 26dBm ±2 dB for band n34 of power class 2, and The MOP and Tolerance for UL MIMO are to be specified as 26dBm +2/-3 dB for n34 of power class 2. Proposal 2: If possible, it should revisit the current PC2 single band MOP tolerance requirement defined for band n41/n77/n78/n79. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 UE RF Requirements

Issue 1-1-1: UE MOP and Tx power tolerance for n34 of Power class 2

- Proposals

  - Option1: The MOP and Tolerance for single antenna port are to be specified as 26dBm ±2 dB for band n34 of power class 2, and The MOP and Tolerance for UL MIMO are to be specified as 26dBm +2/-3 dB for n34 of power class 2.

  - Option2: The power tolerance for PC2 for n34 is +2/-3. Regardless of single antenna port or UL MIMO case.

  - Option3: PC2 n34/n39 MOP lower tolerance should be +2/-2 dB.

- Recommended WF

  - TBA. Collect companies’ view in 1st round

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Nokia | Issue 1-1-1: Option 3 |
| ZTE | Option 1. In addition, we are wondering if option 1 is agreeable, then if it is possible to revisit the current PC2 single band MOP tolerance requirement defined for band n41/n77/n78/n79, since a uniform approach is better for all PC2 single band. To Nokia: In your R4-2109003, it seems the reason for Option 3 is: there is no reason to relax the lower tolerance for PC2 MOP without UL MIMO and/or TxD. It seems it is consitency with the sentence in Option 1: “The MOP and Tolerance for single antenna port are to be specified as 26dBm ±2 dB for band n34 of power class 2”. Also, according to R4-2109003, “Observation 1: +2/-3 comes from dual PA configurations.”, which corresponding to UL-MIMO case. So after reading the R4-2109003, we would like to check with Nokia, is Option 1 also your preference? |
| Huawei | Issue 1-1-1: So far all PC2 bands have tolerance of 2/-3 dB. We support option 2. |
| Xiaomi | Prefer Option 2. If most companies are OK with option 1, option 1 is also acceptable to us. |
| CMCC | Issue 1-1-1: UE MOP and Tx power tolerance for n34 of Power class 2 We prefer option1 and option 3 |
| Samsung | Issue 1-1-1: Option 2. It is better to have the same tolerance with other PC2 bands. Changing the lower tolerance for single antenna port can be discussed with other bands in the future. |
| Skyworks | Issue 1-1-1: for bands above 3.3GHz +2/-3 is used even in LTE. This wider range is also needed because of the large BW (up to 100MHz) for bands n40, n41, n77/78/79 which see more impact from the PA fractional BW. We do not agree to revisit the PC2 single CC tolerance for n41/n77/78/79. ZTE: Our intention is simply, to find out if it is possible to use a uniform approach for all PC2 band. Also after checking LTE spec, we see for PC2 band 42, +2/-3dB is used. So we are fine to keep the current telorence values for the existing PC2 bands. |
| Apple | Issue 1-1-1: We prefer Option 2 as other PC2 definitions use the same tolerance. |
| Vivo | Issue 1-1-1: Option 2 |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2108943 | Huawei: The decision on the CR should depend on the outcome of issue 1-1-1. |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Issue 1-1-1: UE MOP and Tx power tolerance for n34 of Power class 2 - Proposals: - Option1: - The MOP and Tolerance for single antenna port are to be specified as 26dBm ±2 dB for band n34/n39 of power class 2 - The MOP and Tolerance for UL MIMO are to be specified as 26dBm +2/-3 dB for band n34/n39 of power class 2 - Option2: The power tolerance for PC2 for n34 is +2/-3dB. Regardless of single antenna port or UL MIMO case. - Option3: PC2 n34/n39 MOP lower tolerance should be +2/-2 dB. - Agreement: - Down-select to Option 1 and Option 2. [Moderator]:The agreement is based on 1st round of comments collection and GTW discussion. Companies need to further identify which optiob to define PC2 Tx power tolerance for n34/n39 on 2nd round of email discussion. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2108943 | To be revised |

Discussion on 2nd round (if applicable)

| CR | Comments collection |
| --- | --- |
| R4_XXXXX （Rev of R4-2108943） CR on PC2 UE RF requirements of n34 in Rel-17 TS 38.101-1 | Huawei: we prefer to set the tolerance to 2/-3 dB, same as other PC2 bands. |
|  | MediaTek: share same view with Huawei |
|  | … |
|  | … |
|  | … |
|  | … |
|  | … |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #2: Introduction of PC2 n39 for NR

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2108976 | CMCC | Proposal 1: The MOP and Tolerance for single antenna port are to be specified as 26dBm ±2 dB for band n39 of power class 2, and The MOP and Tolerance for UL MIMO are to be specified as 26dBm +2/-3 dB for n39 of power class 2. Proposal 2: No changes to PC2 A-MPR requirements for n39. |
| R4-2108944 | CMCC | CR on PC2 UE RF requirements of n39 in Rel-17 TS 38.101-1 |
| R4-2109677 | vivo | Proposal 2: Considering the power consumption and extra insertion loss of PC 2 UE, the power tolerance +2/-3 for n39 is proposed. |
| R4-2109003 | Nokia, Nokia Shanghai Bell | Proposal: PC2 n34/n39 MOP lower tolerance should be +2/-2 dB. |
| R4-2109257 | Nokia, Nokia Shanghai Bell | This contribution presents simulated PC2 A-MPR results for additional spurious emission limit NS_50 on n39. |
| R4-2109259 | Nokia, Nokia Shanghai Bell | Observation 1: PC3 A-MPR regions are sufficient for PC2 A-MPR. Observation 2: PC2 CIM3 A-MPR exceeds PC3 A-MPR when considering 40 MHz channel bandwidth. Proposal 1: Re-use NS_50 PC3 A-MPR regions for PC2 A-MPR. Proposal 2: For 40 MHz CIM3 A-MPR use values defined in A1 column of the PC3 A-MPR. Observation 3: PC3 A-MPR refers to A-MPR value column A9 which is not defined. |
| R4-2110475 | ZTE Corporation | Proposal 1: Option1, i.e. The MOP and Tolerance for single antenna port are to be specified as 26dBm ±2 dB for band n39 of power class 2, and The MOP and Tolerance for UL MIMO are to be specified as 26dBm +2/-3 dB for n39 of power class 2. |
| R4-2111014 | Apple | Observation 1: In the case of DFT-s-OFDM allowance is not always enough to comply with coexistence requirements. Proposal 1: To comply with emission limits, add a new note which allows 2dB power backoff for outer allocations and 1dB for inner allocations in case of RBstart <= 4.32MHz and PC2, DFT-s-OFDM and CBW larger than 5MHz. In case of CP-OFDM the power backoff need seems to be inside MPR allowance. Therefore, no additional power backoff allowance is required. The simulation results can be found in section 3. Proposal 2: Use allocations regions found in Table 1 and A-MPR proposed in Table 2 for 25MHz, 30MHz and 40MHz CBW NS_50. |
| R4-21XXXX | Huawei，HiSilicon | late submission for information https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_99-e/Inbox/Drafts/%5B99-e%5D%5B125%5D%20HPUE_PC2_n34_n39/R4-2111xxx%20AMPR%20for%20n39%20NS_50%20PC2.docx |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1 UE RF Requirements

Issue 2-1-1: UE MOP and Tx power tolerance for n39 of Power class 2

- Proposals

  - Option1: The MOP and Tolerance for single antenna port are to be specified as 26dBm ±2 dB for band n39 of power class 2, and The MOP and Tolerance for UL MIMO are to be specified as 26dBm +2/-3 dB for n39 of power class 2.

  - Option2: The power tolerance for PC2 for n39 is +2/-3. Regardless of single antenna port or UL MIMO case.

  - Option3: PC2 n34/n39 MOP lower tolerance should be +2/-2 dB.

- Recommended WF

  - TBA. Collect companies’ view in 1st round

Issue 2-1-2: A-MPR

- Proposals

  - Option1: No changes to PC2 A-MPR requirements for n39.

  - Option2: Re-use NS_50 PC3 A-MPR regions for PC2 A-MPR.

For 40 MHz CIM3 A-MPR use values defined in A1 column of the PC3 A-MPR.

Table 1. A-MPR regions for NS_50, together with A-MPR for PC3 and PC2 (PC2 highlighted in yellow)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR, PC3 | A-MPR, PC2 |
| --- | --- | --- | --- | --- |
| 25 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 | A7 |
|  | ≤ 20 | ≤ 1.44 | A8 |  |
| 30 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 | A7 |
|  | ≤ 25 | ≤ 1.44 | A8 | A8 |
|  |  | ≤ 3.6 | A9 |  |
| 40 MHz | ≤ 4.32 | > 0 | A1 | A1 |
|  | > 4.32, ≤ 10.44 | ≤ 10.8 | A3 | A3 |
|  | > 4.32, ≤ 18 | > 10.8 | A2 | A2 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A6 | A6 |
|  | > 31.68 | > 0 | A5 | A1 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.19-2. |  |  |  |  |

Table 2. A-MPR values for NS_50

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 2 |
|  | QPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2 |
|  | 16 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2.5 |
|  | 64 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 |  | ≤ 5 |  |
|  | 256 QAM | ≤ 11 | ≤ 7 |  | ≤ 5 |  | ≤ 5 |  |
| CP-OFDM | QPSK | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  |
|  | 16 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  |
|  | 64 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 |  | ≤ 6.5 |  |
|  | 256 QAM | ≤ 12 | ≤ 8 |  |  |  | ≤ 6.5 |  |

  - Option3: Use allocations regions found in Table 1 and A-MPR proposed in Table 2 for 25MHz, 30MHz and 40MHz CBW NS_50.

Table 1: PC2 A-MPR regions for NS_50 Channel Bandwidth (MHz); RBstart*12*SCS (MHz); LCRB*12*SCS (MHz); A-MPR. 25 MHz; ≤ LCRB*12*SCS - 5; > 5; A7. ≤ 20; ≤ 1.44; A8. ≤ max(0, 6.48 - LCRB *12*SCS); > 1.44, ≤ 5.0; A6. 30 MHz; ≤ LCRB*12*SCS - 5; > 5; A7. ≤ 25; ≤ 1.44; A8. ≤ 3.6; A9. > LCRB *12*SCS – 5, ≤ 5.04; > 3.6; A6. 40 MHz; ≤ 4.32; > 0; A1. > 4.32, ≤ 10.44 12.96; ≤ 10.8; A3. > 4.32, ≤ 18; > 10.8; A2. > 18, ≤ 31.68; > max (31.68 – RBstart*12*SCS, 0); A6. > 31.68; > 0; A5. NOTE 1: The A-MPR values are specified in Table 6.2.3.19-2.. Table 2: PC2 A-MPR for NS_50 Modulation/Waveform; A1 (dB); A2 (dB); A3 (dB); A5 (dB); A6 (dB); A7 (dB); A8 (dB). Outer/Inner; Outer/Inner; Outer/Inner; Outer/Inner; Outer/Inner; Outer/Inner; Outer/Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ 11 11.5; ≤ 7 8.0; ≤ 3 4.0; ≤ 5 7.5; ≤ 2 2.5; ≤ 4 5; ≤ 2 2.5. QPSK; ≤ 11 11.5; ≤ 7 8.0; ≤ 3 4.0; ≤ 5 7.5; ≤ 2 2.5; ≤ 5 6; ≤ 2 2.5. 16 QAM; ≤ 11 11.5; ≤ 7 8.0; ≤ 3 4.0; ≤ 5 7.5; ≤ 2 2.5; ≤ 5 6; ≤ 2.5 3.0. 64 QAM; ≤ 11 11.5; ≤ 7 8.0; ≤ 3 4.0; ≤ 5 7.5; ≤ 5 6. 256 QAM; ≤ 11 11.5; ≤ 7 8.0; ≤ 5 7.5; ≤ 5 6. CP-OFDM; QPSK; ≤ 12 12.5; ≤ 8 9.0; ≤ 4.5 5.5; ≤ 5 7.5; ≤ 3.5 4.5; ≤ 6.5 7.5. 16 QAM; ≤ 12 12.5; ≤ 8 9.0; ≤ 4.5 5.5; ≤ 5 7.5; ≤ 3.5 4.5; ≤ 6.5 7.5. 64 QAM; ≤ 12 12.5; ≤ 8 9.0; ≤ 4.5 5.5; ≤ 5 7.5; ≤ 6.5 7.5. 256 QAM; ≤ 12 12.5; ≤ 8 9.0; ≤ 5 7.5; ≤ 6.5 7.5.

- Recommended WF

  - TBA. Collect companies’ view in 1st round

Issue 2-1-3: MPR

- Proposals

  - Option1: No changes to 1Tx PC2 MPR general requirements. (The agreement captured in the WF R4-2105386 has been approved in RAN4#98-bis-e meeting)

  - Option2: To comply with emission limits, add a new note which allows 2dB power backoff for outer allocations and 1dB for inner allocations in case of RBstart <= 4.32MHz and PC2, DFT-s-OFDM and CBW larger than 5MHz.

- Recommended WF

  - TBA. Collect companies’ view in 1st round

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Nokia | Issue 2-1-1: Option 3 |
| ZTE | Issue 2-1-1: Same view as Issue 1-1-1: Option 1. |
| Huawei | Issue 2-1-1: So far all PC2 bands have tolerance of 2/-3 dB. We support option 2. Issue 2-1-2: Generally speaking, the A-MPR values in option 2 are too optimistic. According to the contribution paper R4-2109257, the PA was calibrated at MPR=0.5 dB rather than the typical value of 1 dB. Consequently, the PA was less compressed, which might explain the relatively optimistic results. Based on our simulation results, option 3 seems more reasonable. However, there remain a number of questions: 1) The A-MPR values for A5 are underestimated. Prefer to follow the approach in option 2, i.e. replace A5 with A1; 2) Our simulation suggests about 1 dB higher value is needed for A1 and A2; 3) A9 is defined but no A-MPR value proposed. Suggest to use 2 and 3 dB for DFT-s-OFDM and CP-OFDM, respectively; 4) For BW=25/30 MHz, A-MPR should also be defined for RBs allocated near the upper edge of the CBW (i.e. near the diagonal of the square plots). More study on the issue is needed. Issue 2-1-3: We support to further investigate the issue. Clearly, when determining MPR, only general SEM, ACLR and/or EVM are considered. And the additional emission requirements embedded in the UE coexistence table are ignored. Many of them don’t have an associated network signaling (NS). There’re more than 40 notes in the coexistence table. Many of them imposes restrictions on RB numbers or frequency range for the sake of facilitating coexistence. And they were defined assuming 23 dBm max output power (PC3), which may not be adequate for PC2. We suspect the problem is not for n39 PC2 alone. Other PC2 bands may have similar problems, given the complexity of coexistence requirements. And the solution in option 2 may not be the only way. Other options such as modifying NS_50 to include BW<=20, or adding restrictions in note 33 of the coexistence table could also be considered. We suggest to thoroughly study the problem and take a holistic approach for all high power bands. |
| Xiaomi | Issue 2-1-1: Prefer Option 2. If most companies are OK with option 1, option 1 is also acceptable to us. |
| CMCC | Issue 2-1-1: UE MOP and Tx power tolerance for n39 of Power class 2 We prefer option1 and option 3 Issue 2-1-3: MPR The PC2 MPR issue is not just for n39/n34 bands, but for all the defined PC2 bands (n41\n77\n78\n79) and existing UEs on the market. Companies are encouraged to evaluate whether this general requirements needs to be modified, and we need to specify the PC2 MPR carefully to avoid complex specification implications. |
| Samsung | Issue 2-1-1: Option 2. Same as Issue 1-1-1. |
| Apple | Issue 2-1-1: We prefer Option 2 as other PC2 definitions use the same tolerance. Issue 2-1-2: Option 3. The simulations for PC2 NS_50 showed that outer allocations (and also a view inner) with RB_start near the lower edge of the channel required increased power backoff. Therefore, a new allocation region was defined for 25MH and 30MHz. For 40MHz it was sufficient to update one of the regions. While we believe that the additional coverage is required, we could also consider different shapes/approaches to provide those allocations with additional MPR. Issue 2-1-3: Option 2. We proposed a certain solution which could solve the increased MPR need for PC2. However, we are happy to discuss alternative approaches and to further investigate the issue. |
| Nokia | For Issue 2-1-2 To Huawei “3) A9 is defined but no A-MPR value proposed. Suggest to use 2 and 3 dB for DFT-s-OFDM and CP-OFDM, respectively;” => Why A9 would need to be higher than A8? Based on typical A-MPR patterns, we’d expect A9 to be lower than A8. |
| vivo | Issue 2-1-1: Option 2. Issue 2-1-2: Option 3. Issue 2-1-3: Option 2 or UE coexistence need be further study. |
| Qualcomm | Issue 2-1-2: We haven’t fully evaluated the A-MPR, but our initial look suggests option 3 but still some of the numbers may need to be revised. |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2108944 | Huawei: The decision on the CR should depend on the outcome of issue 2-1-1. |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Issue 2-1-1: UE MOP and Tx power tolerance for n39 of Power class 2 - Proposals: - Option1: - The MOP and Tolerance for single antenna port are to be specified as 26dBm ±2 dB for band n34/n39 of power class 2 - The MOP and Tolerance for UL MIMO are to be specified as 26dBm +2/-3 dB for band n34/n39 of power class 2 - Option2: The power tolerance for PC2 for n34 is +2/-3dB. Regardless of single antenna port or UL MIMO case. - Option3: PC2 n34/n39 MOP lower tolerance should be +2/-2 dB. - Agreement: - Down-select to Option 1 and Option 2. Issue 2-1-3: MPR - Proposals - Option1: No changes to 1Tx PC2 MPR general requirements. (The agreement captured in the WF R4-2105386 has been approved in RAN4#98-bis-e meeting) - Option2: To comply with emission limits, add a new note which allows 2dB power backoff for outer allocations and 1dB for inner allocations in case of RBstart <= 4.32MHz and PC2, DFT-s-OFDM and CBW larger than 5MHz. - Option 3: have further study. - Agreement - Keep MPR requirements for PC2 unchanged - Work on A-MPR requirements or other solutions to address the potential issues if the issues are fully justified. - Try to avoid the change of general requirements Issue 2-1-2: A-MPR - Proposals - Option1: No changes to PC2 A-MPR requirements for n39. - Option2: Re-use NS_50 PC3 A-MPR regions for PC2 A-MPR. For 40 MHz CIM3 A-MPR use values defined in A1 column of the PC3 A-MPR - Option3: Use allocations regions found in Table 1 and A-MPR proposed in Table 2 for 25MHz, 30MHz and 40MHz CBW NS_50. [Moderator]: Companies need to further identify PC2 Tx power tolerance and A-MPR for n39 on 2nd round of email discussion. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on Tx power tolerance for PC2 n34 and n39 | CMCC |
| #2 | WF on A-MPR for PC2 n39 | Huawei，HiSilicon |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2108944 | To be revised |

Discussion on 2nd round (if applicable)

| WF | Comments collection |
| --- | --- |
| R4-21XXXX WF on Tx power tolerance for PC2 n34 and n39 | Huawei: we prefer to set the tolerance to 2/-3 dB, same as other PC2 bands. Besides, TxD also uses single antenna port (not physical antenna connector), which is discussed in a separate thread. For the above reasons, we cannot support the agreement in the WF. |
|  | MediaTek: We prefer to set the tolerance to 2/-3 dB and we do have concern on the sub-bullet “PC2 n34 and n39 Tx tolerance for 1PA will can be re-evaluated in the future”. The sub-bullet shall be removed. If one company has concern on the majority view, the discussion shall continue. Once the MOP and its tolerance is being specified, it shall not say re-visit when specifying specs that sounds like one company do not agree and have intension to change it in the future. RAN4 shall not agree on such approach. |
|  | … |
|  | … |
|  | … |
|  | … |
|  | … |

| WF | Comments collection |
| --- | --- |
| R4-21XXXX WF on A-MPR for PC2 n39 | Qualcomm: We are ok with the WF. In GTW, I expressed the same concerns about this WF that Huawei had expressed about another WF on a different topic – i.e., that the simulation assumptions were not clearly spelled out in the contributions, that PA calibration was not shown, that ET PA was not represented, that we cannot be sure it is right but can’t find anything wrong with it and therefore the WF could not be agreed. However, Qualcomm does not believe that type of argument is productive and as a technical contributor to 3GPP we strongly believe in making progress. We hope other companies feel the same way |
|  | Company B: |
|  | … |
|  | … |
| CR | Comments collection |
| R4_XXXXX （Rev of R4-2108944） CR on PC2 UE RF requirements of n39 in Rel-17 TS 38.101-1 | Huawei: we prefer to set the tolerance to 2/-3 dB, same as other PC2 bands. |
|  | MediaTek: share same view with Huawei |
|  | … |
|  | … |
|  | … |
|  | … |
|  | … |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2107829 | Agreeable |
| R4-2107738 | Agreeable |
| R4-2107830 | Technically endorse. |
| R4-2107831 | Technically endorse. |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| AMPR for n39 NS_50 PC2.docx | Huawei，HiSilicon | late submission for information |
| WF on Tx power tolerance for PC2 n34 and n39 | CMCC |  |
| WF on A-MPR for PC2 n39 | Huawei，HiSilicon |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2108943 | CR on PC2 UE RF requirements of n34 in Rel-17 TS 38.101-1 | CMCC | Revised |  |
| R4-2108975 | Discussion on the UE RF requirements of PC2 n34 | CMCC | Noted |  |
| R4-2109003 | PC2 MOP lower tolerance for n34/n39 | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2109677 | Discussion on Tx power tolerance for n34 | vivo | Noted |  |
| R4-2110474 | Discussion on HPUE band n34 | ZTE Corporation | Noted |  |
| R4-2108944 | CR on PC2 UE RF requirements of n39 in Rel-17 TS 38.101-1 | CMCC | Revised |  |
| R4-2108976 | Discussion on the UE RF requirements of PC2 n39 | CMCC | Noted |  |
| R4-2109257 | PC2 A-MPR simulation results for NS_50 on n39 | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2109259 | PC2 A-MPR for NS_50 on n39 | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2110475 | Discussion on HPUE band n39 | ZTE Corporation | Noted |  |
| R4-2111014 | Considerations for PC2 n39 and A-MPR results | Apple | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2107829 | WF on Tx power tolerance for PC2 n34 and n39 | CMCC | Agreeable |  |
| R4-2107738 | WF on A-MPR for PC2 n39 | Huawei，HiSilicon | Agreeable |  |
| R4-2107830 | CR on PC2 UE RF requirements of n34 in Rel-17 TS 38.101- | CMCC | Technically endorse | Moderator: For PC2 n34 and n39, A package CR will be used to capture the agreement in RAN4#100e meeting. |
| R4-2107831 | CR on PC2 UE RF requirements of n39 in Rel-17 TS 38.101-1 | CMCC | Technically endorse | Moderator: PC2 A-MPR is under discussion, and the WI target completion time is in September. We can submit the CR capture agreement in next RAN4 meeting. |
| R4-2107828 | AMPR for n39 NS_50 PC2.docx | Huawei，HiSilicon | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents