3GPP TSG-RAN WG4 Meeting # 99-e 													R4-2107937

Electronic Meeting, May. 19-27, 2021

Agenda item:			8.36.1

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [99-e][127] NR_PC2_CA_R17_2BDL_2BUL

Document for:	Information

Introduction

This email discussion thread is related to NR PC2 CA basket WI, and will focus on the topic of following aspects:

- Topic #1: draft TR and big CR

- Topic#2: UE RF requirements

  - Sub-topic 2-1: UE RF requirements for proposed CA

  - Sub-topic 2-2: MSD improvement

Note that the tables for collecting comments for sub-topic issues are arranged just below each issue.

Topic #1: draft TR and big CR

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/Abstracts |
| --- | --- | --- |
| R4-2110051 | China Telecom | Abstract: This draft TR provides the draft TR v0.4.0, which was reserved for email approval and aims to reflect the TP approved in this meeting. |
| R4-2110052 | China Telecom | Big CR to Introduce the completed combos to 38101-1 which was reserved for email approval |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: draft TR and big CR

This sub-topic will discuss rapporteur input for draft TR and big CR.

Issue 1-1-1: draft TR

- Recommended WF

  - It is recommended for email approval for the draft TR of R4-2110051

| Company | Comments on Issue 1-1-1: draft TR |
| --- | --- |
| ZTE | Of course it is for email approval. It is the normal procedure in RAN4. |

Issue 1-1-2: big CR

- Recommended WF

  - It is recommended for email approval for the big CR of R4-2110052

| Company | Comments on Issue 1-1-2: Big CR |
| --- | --- |
| ZTE | Of course it is for email approval. It is the normal procedure in RAN4. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: Topic 1 was closed the draft TR of R4-2110051 and the big CR of R4-2110052 are for email approval. |

Discussion on 2nd round (if applicable)

Topic #2: UE RF requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/Abstracts |
| --- | --- | --- |
| R4-2110070 | China Telecom | TP to 38.841: MSD requirement due to harmonic mixing for PC2 CA_n3A-n78A with up to 2 uplink |
| R4-2110460 | ZTE Corporation | TP for TR38.841_ PC2 CA_n41A-n79A |
| R4-2110790 | China Telecom | Discussion on how to reflect HPUE CA with 1 up link in 38101 |
| R4-2110791 | CHTTL | Proposal 1: RAN4 to consider introducing an optional capability for additional sets of improved requirements, starting from the following target requirements. - MSD of 2nd and 3rd harmonic, and MSD of IMD2 and IMD3 for EN-DC and NR CA - Target on PC2 first then PC3 Proposal 2: if proposal 1 is not agreeable in this meeting, we would like to collect more feedbacks from companies including the following aspects: - Whether the operators think current MSD with values larger than 20dB (or x dB) is sufficient. - Whether there is any issue or impact due to the introduction of the optional capability. |
| R4-2111489 | T-Mobile USA | TP for TR38.841: PC2 CA_n25A-n77A |
| R4-2111490 | T-Mobile USA | TP for TR38.841: PC2 CA_n41A-n77A |
| R4-2111491 | T-Mobile USA | TP for TR38.841: PC2 CA_n71A-n77A |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1: UE RF requirements for proposed CA

This sub-topic will discuss UE RF requirements for proposed combinations.

Issue 2-1-1: Reflect HPUE CA with 1 uplink in 38101

- Recommended WF

  - Collect views on this issue, discussion in R4-2110790 could be a starting point.

  - It is intended to use a WF to capture the final agreement and the agreement will be captured into big CR.

| Company | Comments on Issue 2-1-1: Reflect HPUE CA with 1 uplink in 38101 |
| --- | --- |
| ZTE | This table is not only for PC3, but also for PC2. For PC3 band combination, it is only for 2UL NR CA configuration, 1UL single band case for PC3 is not included in this table. But if the ‘uplink’ is removed, then it would be interpreted as DL CA configuration which would include 1UL single band case for PC3, but it is not true for PC3. Our understanding for this paper is to capture single PC2 band in UL for PC2 band combination, we don’t think it is a good idea to implement it in the existing UL CA table. A separate table is recommended. |
| Xiaomi | A separate text description similar as PC3 in one band is preferred. |
| LGE | Need to clarify why to define the single UL for inter-band CA UE? As ZTE mentioned above, this is only for 2UL NR CA band combination. Also the WID scope is to define PC2 inter-band CA with 2 bands DL & 2 bands UL. |
| T-Mobile USA | To LGE, the WID scope includes 2 DL/1UL. From RP-210476: “Specify the band-combination specific RF requirements for all listed power class 2 NR inter-band UL CA combinations with 2 band DL / 2 band UL Also, specify the band-combination specific RF requirements for all listed inter-band DL CA combinations with 2 band DL / 1 band UL power class 2 and/or power class 1.5 NR single band UL.” We support the proposal in R4-2110790. However, we should also add a column for PC1.5 with a note that it currently only applies to single uplink on the TDD band. We think a separate table could work, too, but would mostly be redundant information and is unnecessary. |
| Huawei | The changes proposed in R4-2110790 seems confusing. The statement to be deleted is for PC3 CA and should be kept. Table 6.2A.1.3-1 is clearly for UL, don’t see why the word “uplink” should be removed. If the CA has only one UL, the maximum output power requirements for single bands should apply. |

Issue 2-1-2: TPs for approval

- Proposed TPs

  - R4-2110070, R4-2110460, R4-2111489, R4-2111490, R4-2111491,

- Recommended WF

  - Collect the comments for proposed TPs. If no comments for certain of TP’s, the TP’s will be recommended as approved.

| CR/TP number | Comments collection for Issue 2-1-2: TPs for approval |
| --- | --- |
| R4-2110070 | ZTE2: MSD for 25MHz and 30MHz for n3 are missing. |
|  | China Telecom: Ok, we will add the MSD for n3 more bandwidths |
| R4-2110460 |  |
| R4-2111489 | LGE: we provided our analysis for MSD for CA_n25A-n77A in R4-2104973 to follow DC_2A_n77A PC2 UE. So it can be acceptable. |
| R4-2111490 | LGE: For the PC2 cross band isolation issue, LGE proposed following MSDrequirmeents in R4-2104973. But in TP, the 6.7dB MSD for n41 DL and 11.0dB MSD for n77 was proposed. How can derive the MSD requirements? Are you apply average manner to find the MSD requirements with all interested companies results? |
|  | T-Mobile USA: Thanks for the input, LGE. We will provide a revision with averaged values. Are there any proposals other than LGE and TMUS? |
| R4-2111491 | LGE: For the PC2 dual uplink issue, LGE proposed MSD 12.2dB by 5th IMD problem. But in this TP, 7.8dB MSD by 5th IMD was provided. How can derive the MSD requirements? Are you apply average manner to find the MSD requirements with all interested companies results? |
|  | T-Mobile USA: Thanks for the input, LGE. We will provide a revision with averaged values. Are there any proposals other than LGE and TMUS? |

Sub-topic 2-2: MSD improvement

Issue 2-2-1: MSD improvement

- Proposals (R4-2110791)

  - Proposal 1: RAN4 to consider introducing an optional capability for additional sets of improved requirements, starting from the following target requirements.

    - MSD of 2nd and 3rd harmonic, and MSD of IMD2 and IMD3 for EN-DC and NR CA

    - Target on PC2 first then PC3

  - Proposal 2: if proposal 1 is not agreeable in this meeting, we would like to collect more feedbacks from companies including the following aspects:

    - Whether the operators think current MSD with values larger than 20dB (or x dB) is sufficient.

    - Whether there is any issue or impact due to the introduction of the optional capability.

- Recommended WF

  - Collect views on the proposals

| Company | Comments on Issue 2-2-1: MSD improvement |
| --- | --- |
| Nokia | We support both proposals. |
| ZTE | Either proposal 1 or proposal 2. |
| Qualcomm | We support proposal 1 |
| Xiaomi | Proposal 2 |
| LGE | Main problem is that 5G UE and LTE-A CA support over 10~20 operating bands and other feature (CA,DC, UL-MIMO, SUL, TxD…). So the additional IL and PCB isolation is not easy to improve compare to previous LTE-A UE. We think the current RF parameters and performance shall be considered to derive MSD requirements. Based on the fact, UE vendor provided the reasonable MSD value and RAN4 applied average manner to derive MSD levels. The concerning point to introduce the new optional capability is that if RAN4 define new capability signaling, then operator always request and provide UE to satisfy the additional capability. So, UE vendor do not want to make new capability signaling for MSD improvement. |
| T-Mobile USA | We support Proposal 1, and if that is not agreeable Proposal 2. As we described in R4-2111492, real world devices can have 20 dB less sensitivity degradation than allowed by the MSD specs. This results in operators ignoring MSD and expecting 0 dB MSD. If some devices use different architectures, like discrete instead of integrated RFFE, then the performance between devices could be significant. It would be helpful if the network was able to distinguish between the different types of devices in their network. |
| Skyworks | If such improved MSD capability is agreed it is necessary to clarify what is signalled: no MSD? 10dB less MSD? The MSD value? Or a second set of improved values in the spec vs actual? The ripple effect of such improvement may be huge for little gain: good UEs exist and they already appear so in the network if that improvement is maintained in radiated but also if the link is not interference limited (ie the MSD is very large and there is no easy escape). We do understand that it is important that the NW does not try to implement scheduler restrictions if these are not needed for all/most UEs. But with this in mind we may want to first look at large MSD cases and agree on what a better UE can do. Also if we start revisiting MSD everywhere it may become a real catch 22 issue: if REFSENS improvement is also discussed single band/CC case then we need to revisit MSDs that will automatically increase. |
| Huawei | Contrary to the claim by several companies that real world UEs perform much better than the 3GPP spec, the field result in the contribution paper actually show that harmonic interference do impact UEs in the field. Half of the UEs in the sample set show significant degradation. If only a small portion of the UEs in the market can cope with the potential interference, would the operator want to deploy the network at such frequencies? We are not convinced that optional capability is feasible. |
| Vivo | Proposal 2, optional capability need FFS. |
| CHTTL | We also support propsol 1, obviously. Regarding to the result, even if it shows some UEs have the impact, still we believe they are still better than the >20dB MSD, as you can see the DL signal is very weak in the cell edge. And note that even the DL signal is very weak still some UE shows zero impact. And we are confident that lots of UE can achieve much better UE than the current spec. Regarding what is signalled, our thinking is to have a 1 bit signalling linked to the second set of improved values in the spec, how much the MSD values will be can be further discussed. Regarding UE vendors concern, we are wondering if no signalling is introduced, the operators will have their own request outside 3GPP if they think the current MSD in the spec is not pracitcal, which might result in different requirement raised by different operators. So we would like to ask whether the operators think current MSD with values larger than 20dB (or x dB) is sufficient. |
| Huawei | To CHTTL: I’d like to double check my understanding of your paper. It shows NR DL RSRP -107 dB for UE A. If I understand correctly, this is RSRP in15kHz MBW. I don’t think this can be counted as “very weak”. Assuming NF=5, the SNR would be 20 dB, ignoring other interferences. And the Tx power is 20 dBm, i.e. 3 dB back off from PC3. So the power of IMD is also reduced from the worst case scenario in the spec. Even under such conditions, significant degradation is shown. I consider this as strong evidence why we have to have large MSDs in the spec. On the other hand, I can understand operators’ desire for potential MSD improvement. But without proven feasibility, it’s premature to discuss the design of capability signalling. |
| Apple | Many concerns have already been named by Huawei, Skyworks and LGE. We agree with most of them. Currently the discussion mostly revolves around the introduction of a capability. But the important question is the impact of such a capability on the network. What is the actual benefit for the network if a view devices support this optional capability? Does throughput get significantly higher? Does it change network planning if only a certain devices support this? |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| 2.2.1 Sub-topic 2-1: UE RF requirements for proposed CA | Issue 2-1-1: Reflect HPUE CA with 1 uplink in 38101 Recommendations for 2nd round: According to the comments, it seems that a separate table for HPUE CA with 1 uplink in 38101 is preferred. The discussion will continue in WF#1 in 2nd round. Issue 2-1-2: TPs for approval Tentative agreements: R4-2110460 and R4-2111489 are recommended as approved since no comment or objection. Recommendations for 2nd round: Focus on the four revised TPs, they are R4-2110070, R4-2111490, R4-2111491. The corresponding revised number will be allocated in chairman’s report. |
| 2.2.2 Sub-topic 2-2: MSD improvement | Recommendations for 2nd round: Continue discussion in 2nd round. |

Discussion on 2nd round (if applicable)

Sub-topic 2-1: UE RF requirements for proposed CA

Issue 2-1-1: Reflect HPUE CA with 1 uplink in 38101

This issue is captured in the WF#1.

| WF | Tdoc number assigned | WF/LS t-doc Title | Assigned Company, WF lead |
| --- | --- | --- | --- |
| #1 |  | WF on Reflecting HPUE CA with 1 uplink in 38101 | China Telecom |

This table below will collect the comments for the WF on Reflecting HPUE CA with 1 uplink in 38101.

| Company | Comments for Reflecting HPUE CA with 1 uplink in 38101 in WF#1 |
| --- | --- |
| Huawei | It seems the original wording has covered the case in concern. “For inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in clause 6.2 apply.” Not sure what benefits the proposed changes bring. |
| LGE | The Table is raised more confusing how to support PC2 UE with 1Uplink. We are same understanding with Huawei. Just add the above sentence by Huawei proposal. |
| China Telecom | We are a little confusing to the comments from Huawei and LGE. I think Huawei’s comment is to not change anything, as the sentence mentioned by Huawei is already in the spec. Here is an example: in clause 6.2, n78 supports PC2. Does that mean all the CA with n78 single uplink will support PC2? |
| Huawei | To China Telecom: We didn’t propose new wording. The quote is the original wording of the spec. Regarding your question, it can be answered by the sentence you propose to delete, which clearly states PC3 (the CA power class) overrides PC2 (per-band power class). Therefore we’re confused about the proposed changes. |
| China Telecom | But now what we are doing it to introduce some of PC2 2DL 1UL CAs. This is in the scope of the WI. |
| ZTE | Except the new added table( i.e. Table 6.2A.1.3-1), i wonder if it is feasibel to introduce new configuration tables for HPUE inter-band NR CA. Otherwise, it seem we may not know the BCS information for HPUE inter-band NR CA, also we may not know the uplink carrier/uplink CA configuration for HPUE inter-band NR CA in the spec. |
| Apple | We have two comments, 1. The Table number for the newly added table should be 6.2A.1.3-2 instead of 6.2A.1.3-1. 2. Table 6.2A.1.3-2 is for UE power class except of class 3 for inter-band CA (two bands). It may be better to specifically indicate inter-band “downlink” CA to have the differentiation with Table 6.2A.1.3-1. Also in NOTE 5, PC3 is still mentioned which may create confusion with the table caption. Maybe in NOTE 5, only PC2 for TDD band needs to be indicated. |
| China Telecom | To ZTE, we just follow the method for uplink CA. It may be necessary to keep the tables format aligned for both 2 uplink CA and 1 uplink CA. Otherwise, both of these two types of CAs tables need to be modified. To Apple, ok, we will update the WF according to your comments, but with a little wording change, which could be found in the version 1 in the ftp. Thanks all for helping to check and improve on the ways to introduce the basket HPUE Downlink CA (1 UL) to the spec. |
| T-Mobile USA | The UE power class table probably has to include the band number for the single band, in case there is more than 1 PC2 or PC1.5 capable bands in the combination. Also, the title of the table could be “Table 6.2A.1.3-1 UE Power Class except of class 3 for inter-band CA (two bands DL/ 1 band UL). Please find a proposed revised table in the inbox. |

Issue 2-1-2: TPs for approval

Focus on the 3 revised TPs

| CR/TP number | Comments collection |
| --- | --- |
| Rev of R4-2110070 |  |
| Rev of R4-2111490 | LGE: thanks for revision. We are fine with the revised version. |
| Rev of R4-2111491 | LGE: thanks for revision. We are fine with the revised version. |
|  | Apple: Would like to ask for a clarification regarding the test frequencies used for IMD5. It seems odd that the UL carrier frequency of n77 is directly at band edge (3300MHz). Furthermore, the frequencies do not seem to match so that DL is subject to IMD5. The same issue is also found for the PC3 CA version in 38.101-1. Is this an error or did I miss something? In case it is an error: The test-frequencies which are specified for CA_n71A-n78A could be used for CA_n71A-n77A. |
| T-Mobile USA | Thanks to Apple for the comment. We just copied what was there for PC3. We are fine with using what is in place for CA_n71-n78. Please find an r2 version in the inbox. CA_n71-n77 PC3 will need to be corrected in August. Further input: Please find revision 3 in the inbox. Qualcomm noticed that PC3 CA_n71-n78 had a different frequency in the uplink and downlink. For CA_n71-n77 PC2 the DL frequency has now been changed to 3361.5 MHz in Rev. 3. Maybe there was a copy and paste error from LTE Band 42. So, CA_n71-n77 and CA_n71-n78 PC3 will need to be corrected in August. |

Sub-topic 2-2: MSD improvement

This issue is captured in the WF#2. There is a similar discussion on better MSD capability happening in [116]. According to Chairman’s guidelines, the discussions were combined and moved here in this [127] thread.

| WF | Tdoc number assigned | WF/LS t-doc Title | Assigned Company, WF lead |
| --- | --- | --- | --- |
| #2 | R4-2107808 | WF on MSD capability | T-Mobile US |

This table below will collect the comments for the WF on MSD capability.

| Company | Comments for MSD capability in WF#2 |
| --- | --- |
| LGE | Based on previous discussion and argument point, we do not accept to define new capability for MSD improvement. |
| Huawei | We also think it’s premature to define MSD capability/signaling. |
| Apple | The current discussion is still too controversial to define or to agree on a new signaling or capability. |
| T-Mobile USA | We understand the concerns with the signalling, but the current situation puts operators in a situation where we don’t know the performance of the UEs. Would it be more acceptable to add a single bit to indicate integrated vs. discrete RFFE? |
| Qualcomm | We support the WF and the proposal to introduce a MSD capability. It has been observed that some devices perform far beyond the 3GPP minimum requirement and it would be beneficial for the network to be aware of such devices directly by signaling. |
| Verizon | We support the WF and the proposals of capability to enhance MSD. We repeat to discuss this topic in RAN4 as it has been observed the higher MSD values will impact the network performance. Start of revisiting the device requirements can be benefits to the network service, and the capability signalling can help the network to know alternative UE requirements to enhance the performance. |
| AT&T | We also support the WF to define a MSD capability. The details can be determined later but the benefit of distinguishing UE performance by the network can be used to optimize performance for CA/DC configurations. |
| Vivo | We don’t support the WF. |
| DISH Network | We support the WF. This is probably the only remaining way to address most current excessive MSD requirements in the specification. |
| Deutsche Telekom | We support the WF. |
| CMCC | We support the WF |
| CHTTL | We also support the WF. (I think we had already support and left comment in v07, but seems like our comment is disappear…?) |
| Huawei | We can understand operators’ desire for potential improvement. However, as we have discussed before and in view of the most recent comments by Skyworks over the email reflector, this issue may be even more complicated than originally anticipated. Without proper feasibility study, we cannot go straight into capability/signaling design. In view of these factors, the WF in R4-2107808 is not agreeable to us. |
| Skyworks | I guess the need for a capability really depends if the networks really has to distinguish the “good” from the “bad”, if there are enough “good” UEs then the network may not have to do anything. With MSD based on different architecture/assumptions for the high values MSD in TRs, operators and network vendors can have an understanding whether most UEs will perform well in their network. Signaling may then only be needed in cases most of the UEs would perform badly (but then does it really help the network to know that there are a few “good” UEs). I even guess that if a UE reports much worse than most UEs, operators would have a way to know. Our concern is really that defining the signaling first may result into a beauty contest on a band combination by band combination (the 3GPP way) while the most advanced UEs that are also usually the most performant do need to have design freedom and margin to solve a lot of trade-offs for a multitude of combinations especially for WW phone with large roaming capabilities. |

Summary for 2nd round

|  | Status summary |
| --- | --- |
| 2.2.1 Sub-topic 2-1: UE RF requirements for proposed CA | Issue 2-1-1: Reflect HPUE CA with 1 uplink in 38101 Recommendations for conclusion: According to the discussion, no more comments were raised for the R4-2107832 WF on reflecting HPUE CA with 1 uplink in 38101_v2.doc. So the WF R4-2107832 is recommended as agreeable. Issue 2-1-2: TPs for approval Recommendations for conclusion: It seems the revised TPs, rev of R4-2110070, rev of R4-2111490, rev of R4-2111491 have addressed the concerns, so they are recommended as agreeable. |
| 2.2.2 Sub-topic 2-2: MSD improvement | Recommendations for conclusion: Some of the companies don’t support the WF. The status of the WF R4-2107808 is recommended as noted. |

Recommendations for Tdocs

1st round

New tdocs

|  | Tdoc num | Title | Source | Comments |
| --- | --- | --- | --- | --- |
| #1 | R4-2107832 | WF on Reflecting HPUE CA with 1 uplink in 38101 | China Telecom |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2110051 | Draft TR 38.841 v0.4.0: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2) | China Telecom | For email approval |  |
| R4-2110052 | CR to 38.101-1 Introduce RF requirements for HPUE CA with 2 bands downlink and x bands uplink (x =1,2) | China Telecom | For email approval |  |
| R4-2110070 | TP to 38.841: MSD requirement due to harmonic mixing for PC2 CA_n3A-n78A with up to 2 uplink | China Telecom | revised to R4-2107833 |  |
| R4-2110460 | TP for TR38.841_ PC2 CA_n41A-n79A | ZTE Corporation | agreeable |  |
| R4-2110790 | Discussion on how to reflect HPUE CA with 1 up link in 38101 | China Telecom | noted |  |
| R4-2110791 | Discussion on UE capability for improved PC2 MSD for EN-DC and NR CA | CHTTL | noted |  |
| R4-2111489 | TP for TR38.841: PC2 CA_n25A-n77A | T-Mobile USA | agreeable |  |
| R4-2111490 | TP for TR38.841: PC2 CA_n41A-n77A | T-Mobile USA | revised to R4-2107834 |  |
| R4-2111491 | TP for TR38.841: PC2 CA_n71A-n77A | T-Mobile USA | revised to R4-2107835 |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2107832 | WF on Reflecting HPUE CA with 1 uplink in 38101 | China Telecom | Agreeable |  |
| R4-2107808 | WF on MSD capability | T-Mobile US | Noted |  |
| R4-2107833 | TP to 38.841: MSD requirement due to harmonic mixing for PC2 CA_n3A-n78A with up to 2 uplink | China Telecom | Agreeable | Rev of R4-2110070 |
| R4-2107834 | TP for TR38.841: PC2 CA_n41A-n77A | T-Mobile USA | Agreeable | Rev of R4-2111490 |
| R4-2107835 | TP for TR38.841: PC2 CA_n71A-n77A | T-Mobile USA | Agreeable | Rev of R4-2111491 |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents