3GPP TSG RAN WG4 Meeting #99-e		R4-2108259

Electronic Meeting, May 19th - 27th, 2021

Agenda item:       6.1.6

Source: 	Nokia, Nokia Shanghai Bell

Title: 	WF on general test configurations for NR-U RRM performance requirements

Document for:     Discussion and Decision

# 1	Introduction

This document contains the agreements and discussions in RAN4#99-e in the email thread [99-e][210] NR_unlic_RRM_2 for the topics concerning general and test case specific RRM performance test configurations. The summary containing the complete discussion on that topic is found on the document [1].

Background information: In RAN4#97-e meeting, NR-U RRM performance requirements we first discussed, with the agreements captured in the WF [2]. In RAN#98-e meeting and RAN#98-bis-e, general test configurations for NR-U RRM performance requirements were captured in the WF [3][4].

The following colour code is used to highlight the agreements in this meeting:

Agreements in the GTW session

Agreements in the 1st round

Agreements in the 2nd round

# 2	Applicability rules

- Add a note in each NR-U test case for verifying the legacy requirements as follows:

  - In EN-DC test: The UE supporting EN-DC only on NR band(s) with shared spectrum access is required to be tested.

  - In SA test: The UE supporting SA operation only on NR band(s) with shared spectrum access is required to be tested.

- Above the NR-U tests include:

  - Handover

    - NR-U - > E-UTRAN (FDD,TDD)

  - Legacy DCI/timer/RRC-based BWP switching on NR-U SCell

  - Interruption

    - Due to inter-RAT SFTD measurements

    - Due to NR-U PSCell addition/release

  - Timing advance

# 3	General configurations of the RRM tests

SI decoding time

- SI decoding time, TSI,CCA, is kept at 1280ms during RRC re-establishment and RRC release with re-direction in NR-U networks

  - remove the following editor’s note from section 6.2.1A.2.1 of TS 38.133

    - Editor’s note: The actual value for TSI-NR_CCA is to be discussed in the performance part, considering LBT failures and receiver assumptions, etc.

# 4	Test case specific configurations

## 4-1: RRC IDLE cell re-selection

How to introduce RRC IDDLE cell re-selection test cases

- Define separate tests for the test cases below:

  - Cell reselection for E-UTRAN (FDD,TDD)->NR-U and NR-U -> E-UTRAN (FDD,TDD).

  - Cell reselection for NR(FR1) -> NR-U and NR-U -> NR(FR1).

## 4-2: HO (delay and interruptions)

HO scenarios

- Specify test cases to verify correct UE behavior for all scenarios:

  - E-UTRAN (FDD,TDD)->NR-U,

  - NR-U -> E-UTRAN (FDD,TDD),

  - NR(FR1) -> NR-U

  - NR-U -> NR(FR1).

HO test behaviour after T304 expires

- Configure LCCA_DL and LCCA_UL such that T304 is not expired due to CCA failures in the HO test case.

- Configure LCCA_DL = LCCA_UL = 5, WCCA =T304, and T304=500ms in the HO test case with CCA.

## 4-3: RRC Re-establishment

Tests for RRC re-establishment

- At least the following NR to NR-U RRC re-establishment tests to verify core requirements in clause 6.2.1A, TS 38.133, are defined:

  - TC1: Intra-frequency RRC Re-establishment in FR1 with serving cell without CCA and known target cell subject to CCA

  - TC2: Inter-frequency RRC Re-establishment in FR1 with serving cell without CCA and with unknown target cell subject to CCA

  - TC3: Intra-frequency RRC Re-establishment in FR1 with serving cell without CCA, with unknown target cell subject to CCA and without serving cell timing.

Test configuration for RRC re-establishment test cases

- NR to NR-U RRC re-establishment tests are defined for the following configuration related to SSB SCS and BW for both serving and target cells:

| Configuration | Source cell without CCA | Target cell with CCA |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Out of sync detection evaluation period in tests with CCA

- In the test under the following parameter settings (non-DRX, no gaps are used and SSB periodicity is 20 ms), the out of sync detection evaluation period = 480 ms when the serving cell is inactivated (RLM-RS SSB Es/Iot <-7 dB).

## 4-4: Random access

Differentiation of FBE and LBE configurations in random access test cases

- Define configuration of random access test cases that help differentiating the UE behaviour when configured with semi-static and dynamic channel access modes.

Limitation of CCA failures in UL for random access TCs

- Define random access test cases that limit the number of CCA failures in UL and DL to prevent reaching preambleTransMax for both LBE and FBE configurations.

- Define preambleTransMax, LCCA_DL and LCCA_UL in random access test cases with CCA such that preambleTransMax > 5 + L LCCA_DL + LCCA_UL for both LBE and FBE configurations

- Define preambleTransMax = n20, LCCA_DL =4 and LCCA_UL =5 in random access test cases with CCA for both LBE and FBE configurations.

- NOTE: LCCA_DL is only configured if PCCA_DL≠1

Configuration of DL CCA for random access test cases

- Configure DL CCA failures for the random access test cases for semi-static channel access configuration

Preamble received target power configuration

- Test equipment to increase by 6dB NR configurations for preambleReceivedTargetPower for msg1 and msgA-PreambleReceivedTargetPower for msgA for RA test cases with UL CCA failures

## 4-5: Timing (transmit timing and TA)

CCA configuration on timing test cases

- UE timing advance adjustment accuracy tests are defined for the following LBT configuration/setting in SpCell: PCCA_UL=1 and PCCA_DL =1 in all test times.

## 4-6: BWP switching delay and interruptions

Configurations for BWP switch test cases

- Endorse the configurations with consistent UL BWP switch on consistent UL LBT failure recovery:

| Active BWP in SpCell | PCCA_UL | PCCA_DL |
| --- | --- | --- |
| UL active BWP before active BWP switching (UL BWP-1) | 0 | 1 |
| UL active BWP after active BWP switching (UL BWP-2) | 1 | 1 |
| DL active BWP before active BWP switching (DL BWP-1) | 1 | 1 |
| DL active BWP after active BWP switching (DL BWP-2) | 1 | 1 |

## 4-7: Beam management (BFD and link recovery)

CCA parameters for link recovery

- Set the CCA parameters in the link recovery tests for NR-U as follows:

CCA parameters in link recovery tests for NR-U

|  |  | T1 | T2 | T3 | T4 | T5 |
| --- | --- | --- | --- | --- | --- | --- |
| PCCA_DL | semi-static channel access | 1.0 | 0.9375 | 0.9375 | 0.9375 | 0.9375 |
|  | dynamic channel access (Note 1) | (0.75,0.75) | (0.75,0.75) | (0.75,0.75) | (0.75,0.75) | (0.75,0.75) |
| PCCA_UL |  | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| Note 1: For dynamic channel access, the probability (X,Y) indicates PCCA_DL_1=X and PCCA_DL_2=Y. |  |  |  |  |  |  |

## 4-8: RSSI/CO measurement accuracy (intra-frequency, inter-frequency, inter-RAT)

Test cases for RSRP and RSRQ

- NR-U RSRP and RSRQ measurement accuracy test cases are designed for following cases:

  - Intra-frequency RSRP measurement accuracy

  - Inter-frequency RSRP measurement accuracy

  - Intra-frequency RSRQ measurement accuracy

  - Inter-frequency RSRQ measurement accuracy

Configuration for RSRP and RSRQ test cases

- NR-U RSRP and RSRQ measurement accuracy test cases are designed for following test configuration:

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

- Cell specific test parameters should contain following new or modified parameters to account for the LBT impact:

  - DL CCA model

  - UL CCA model

  - DBT Window Configuration

  - DL CCA probability PCCA_DL

  - UL CCA probability PCCA_UL

  - SSB configuration

  - New RMCs

## 4-9: TCI state switching

Timing difference between RSs in two TCI states

- Do not introduce test cases for TCI state switching.

# References

- R4-2105804, “Email discussion summary: [98-bis-e][202] NR_unlic_RRM_2”, Nokia, Nokia Shanghai Bell, RAN4#98-bis-e.

- R4-2017089, “WF on NR-U RRM performance,” Nokia, Nokia Shanghai Bell, RAN4#97-e.

- R4-2103518, “WF on general test configurations for NR-U RRM performance requirements”, Nokia, Nokia Shanghai Bell, RAN4#98-e.

- R4-2105709, “WF on general test configurations for NR-U RRM performance requirements”, Nokia, Nokia Shanghai Bell, RAN4#98-bis-e