3GPP TSG-RAN WG4 Meeting #99-e 	R4-2108513

# Online, 17th – 29th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 265 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for the Introduction of NR-U UE Demodulation Requirements (PDSCH and CQI) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2021-05-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introducing PDSCH Demodulation performance and CQI Reporting tests, to guarantee minimum performances for UEs operating on bands with shared spectrum access (PCell on band with shared spectrum access or SCell on band with shared spectrum access or both). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce PDSCH Demodulation performance and CQI Reporting tests and corresponding requirements for UEs supporting bands with operations in shared spectrum access. Introduce Downlink Transmission Model and Fixed Reference Channel necessary for the test setup. The changes from the following TDocs have been integrated into this CR: R4-2108713, R4-2108714, R4-2108715, R4-2108716, R4-2108717, R4-2108092 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No UE Demodulation performance or CSI reporting test is defined for UEs operating on bands with shared spectrum access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 5.1.1.3 and new section 5.1.1.9, 5.2.2.2.15, 5.2.3.2.15 New section 5.2A.2.3, 5.2A.3.3 Section 6.1.1, 6.1.2, 6.2.2.2.1.4, 6.2.3.2.1.4, 6.2A.3.1, 6.2A.4.1 Section A.1.2, A.3.2.2.2, A.4 New section B.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 5	Demodulation performance requirements (Conducted requirements)

## 5.1	General

### 5.1.1	Applicability of requirements

Start of Change 1

#### 5.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 5.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 3-1) Clause 5.2.3.1.1 (Test 5-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 3-1) Clause 5.2.3.2.1 (Test 5-1) |  |
| Alternative additional DMRS position for co-existence with LTE CRS (additionalDMRS-DL-Alt) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Test 1-2) Clause 5.2.3.1.4 (Test 1-2) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.4 (Test 1-2) Clause 5.2.3.2.4 (Test 1-2) |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 5.5A.1 | 1)Up to 16 DL carriers 2)Same numerology across carrier for data/control channel at a given time |
| Enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h | FR1 FDD | PDSCH | Clause 5.2.2.1.9 (Test 1-1) Clause 5.2.3.1.9 (Test 1-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.9 (Test 1-1) Clause 5.2.3.2.9 (Test 1-1) |  |
| Alternative 64QAM MCS table for PDSCHNew 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |
| CQI table with target BLER of 10^-5New CQI table (cqi-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 |  |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR1 FDD | PDSCH | Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |
| UE PDSCH processing capability #2 (pdsch-ProcessingType2) | FR1 FDD | PDSCH | Clause 5.2.2.1.7 Clause 5.2.3.1.7 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.7 Clause 5.2.3.2.7 |  |
| Pre-emption indication for DL (pre-EmptIndication-DL) | FR1 FDD | PDSCH | Clause 5.2.2.1.8 Clause 5.2.3.1.8 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.8 Clause 5.2.3.2.8 |  |
| Single DCI based SDM transmission for multi-TRxP (singleDCI-SDM-scheme-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.11 Clause 5.2.3.1.11 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.11 Clause 5.2.3.2.11 |  |
| Multi DCI based multi-TRxP support (multiDCI-MultiTRP-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.12 Clause 5.2.3.1.12 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.12 Clause 5.2.3.2.12 |  |
| Single DCI based FDM Scheme-A for multi-TRxP(supportFDM-SchemeA-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.13 Clause 5.2.3.1.13 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.13 Clause 5.2.3.2.13 |  |
| Single DCI based inter-slot TDM for multi-TRxP (supportInter-slotTDM-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |
| DRX Adaptation (drx-Adaptation-r16) | FR1 FDD | PDCCH | Clause 5.3.2.1.3 | If the Test 1 in Clause 5.3.2.1.3 is passed, the test coverage can be considered fulfilled without executing Test 3 in clause 5.3.2.1.1. |
|  | FR1 TDD | PDCCH | Clause 5.3.2.2.3 | If the Test 1 in Clause 5.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 2 in clause 5.3.2.2.1. |
|  | FR1 FDD | PDCCH | Clause 5.3.3.1.3 | If the Test 1 in Clause 5.3.3.1.3 is passed, the test coverage can be considered fulfilled without executing Test 3 in clause 5.3.3.1.1. |
|  | FR1 TDD | PDCCH | Clause 5.3.3.2.3 | If the Test 1 in Clause 5.3.3.2.3 is passed, the test coverage can be considered fulfilled without executing Test 2 in clause 5.3.3.2.1. |
| Validating P/SP-CSI-RS reception (periodicAndSemi-PersistentCSI-RS-r16) | FR1 TDD | PDSCH | Clause 5.2.2.2.15 Clause 5.2.3.2.15 Clause 5.2A.2.3 Clause 5.2A.3.3 | The requirements apply only in case tested UE supporting operations in shared spectrum access and validation of P/SP-CSI-RS reception based on DCI |
| Supported UL channels for dynamic channel access mode (ul-DynamicChAccess-r16) or UL channel access for semi-static channel access mode (ul-Semi-StaticChAccess-r16) or both | FR1 TDD | PDSCH | Clause 5.2.2.2.15 Clause 5.2.3.2.15 | The requirements apply only in case tested UE supports one of UL channels for dynamic channel access mode and UL channel access for semi-static channel access mode |

End of Change 1

Start of Change 2

#### 5.1.1.9	Applicability of requirements for PDSCH on bands with shared spectrum access

| Tests | Applicability notes |
| --- | --- |
| All tests in Clause 5.2.2.2.15 and 5.2.3.2.15 | Only test the supported largest channel bandwidth. |
| All tests in Clause 5.2A.2.3 and 5.2A.3.3 | Only test the supported largest channel bandwidth on SCell. |

End of Change 2

## 5.2	PDSCH demodulation requirements

### 5.2.2	2RX requirements

#### 5.2.2.2	TDD

Start of Change 3

##### 5.2.2.2.15	Minimum requirements for PDSCH of PCell on band with shared spectrum access

The performance requirements are specified in Table 5.2.2.2.15-3, with the addition of test parameters in Table 5.2.2.2.15-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.15-1.

Table 5.2.2.2.15-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance for UE supporting operations in shared spectrum access | 1-1, 1-2, 1-3, 1-4 |

Table 5.2.2.2.15-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| DL transmission model |  |  | As specified in B.5 |
| Downlink Model Parameters | SSB Q factor |  | 8 |
|  | Downlink transmission duration values | Slots | {2,4,6,7} |
|  | Occupied OFDM symbols in slot other than the last slot of the downlink duration | Symbols | 14 |
|  | Occupied OFDM symbols in the last slot of the downlink duration | Symbols | {6,9,12,14} (Note 1) |
|  | Downlink period | ms | 5 |
|  | LBT failure probability (pLBT) |  | 0.25 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: If DL Transmission duration is 2 Slot, the occupied OFDM symbols in the last slot of the downlink duration is 14. |  |  |  |

Table 5.2.2.2.15-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-18.1 TDD | 20 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | [13.8] |
| 1-2 | R.PDSCH.1-18.2 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | [14.0] |
| 1-3 | R.PDSCH.1-18.3 TDD | 60 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | [14.2] |
| 1-4 | R.PDSCH.1-18.4 TDD | 80 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | [14.5] |

End of Change 3

### 5.2.3	4RX requirements

#### 5.2.3.2	TDD

Start of Change 4

##### 5.2.3.2.15	Minimum requirements for PDSCH of PCell on band with shared spectrum access

The performance requirements are specified in Table 5.2.3.2.15-3, with the addition of test parameters in Table 5.2.3.2.15-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.15-1.

Table 5.2.3.2.15-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance for UE supporting operations in shared spectrum access | 1-1, 1-2, 1-3, 1-4 |

Table 5.2.3.2.15-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| DL transmission model |  |  | As specified in B.5 |
| Downlink Model Parameters | SSB Q factor |  | 8 |
|  | Downlink transmission duration values | Slots | {2,4,6,7} |
|  | Occupied OFDM symbols in slot other than the last slot of the downlink duration | Symbols | 14 |
|  | Occupied OFDM symbols in the last slot of the downlink duration | Symbols | {6,9,12,14} (Note 1) |
|  | Downlink period | ms | 5 |
|  | LBT failure probability (pLBT) |  | 0.25 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: If DL Transmission duration is 2 Slot, the occupied OFDM symbols in the last slot of the downlink duration is 14. |  |  |  |

Table 5.2.3.2.15-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-18.1 TDD | 20 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x4, ULA Low | 70 | [8.8] |
| 1-2 | R.PDSCH.1-18.2 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x4, ULA Low | 70 | [8.8] |
| 1-3 | R.PDSCH.1-18.3 TDD | 60 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x4, ULA Low | 70 | [9.0] |
| 1-4 | R.PDSCH.1-18.4 TDD | 80 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x4, ULA Low | 70 | [9.2] |

End of Change 4

## 5.2A	PDSCH demodulation requirements for CA

### 5.2A.2	2RX requirements

Start of Change 5

##### 5.2A.2.3	Minimum requirements for PDSCH of SCell on band with shared spectrum access

The performance requirements for SCell on band with shared spectrum access are specified in Table 5.2.2.2.15-3, with the additional test parameters for SCell in Table 5.2.2.2.15-2, the test parameters for PCell in Table 5.2A.2.3-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2A.2.3-1. During the test, only the PDSCH performance of the SCell should be verified.

Table 5.2A.2.3-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance of SCell for UE supporting operations in shared spectrum access | 1-1, 1-2, 1-3, 1-4 |

Table 5.2A.2.3-2: Test parameters for PCell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | Table 5.2-1 |
|  | CSI-RS periodicity | Slots | Table 5.2-1 |
|  | CSI-RS offset | Slots | Table 5.2-1 |
|  | Frequency Occupation |  | Table 5.2-1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

End of Change 5

### 5.2A.3	4RX requirements

Start of Change 6

##### 5.2A.3.3	Minimum requirements for PDSCH of SCell on band with shared spectrum access

The performance requirements for SCell on band with shared spectrum access are specified in Table 5.2.3.2.15-3, with the additional test parameters for SCell in Table 5.2.3.2.15-2, the test parameters for PCell in Table 5.2A.3.3-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2A.3.3-1. During the test, only the PDSCH performance of the SCell should be verified.

Table 5.2A.3.3-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance of SCell for UE supporting operations in shared spectrum access | 1-1, 1-2, 1-3, 1-4 |

Table 5.2A.3.3-2: Test parameters for PCell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | Table 5.2-1 |
|  | CSI-RS periodicity | Slots | Table 5.2-1 |
|  | CSI-RS offset | Slots | Table 5.2-1 |
|  | Frequency Occupation |  | Table 5.2-1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

End of Change 6

# 6	CSI reporting requirements (Conducted requirements)

Start of Change 7

### 6.1.1	Applicability of requirements

#### 6.1.1.1	General

The minimum performance requirements are applicable to all FR1 operating bands defined in TS 38.101-1 [6] except for test cases listed in Clause 6.2.2.2.1.4, Clause 6.2.3.2.1.4, Clause 6.2A.3.1.2 and Clause 6.2A.4.1.2 which are only applicable for FR1 bands restricted to operation with shared spectrum.

The minimum performance requirements in Clause 6 are mandatary for UE supporting NR operation, except test cases listed in Clause 6.1.1.3, 6.1.1.4.

#### 6.1.1.2	Applicability of requirements for different number of RX antenna ports

The number of RX antenna ports for different RF operating bands is up to UE declaration.

The UE shall support 2 or 4 RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in clause 7.2 of TS 38.101-1 [6]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 6.1.1.2-1.

Table 6.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports only 2RX | CQI | All tests in Clause 6.2.2 |
|  | PMI | All tests in Clause 6.3.2 |
|  | RI | All tests in Clause 6.4.2 |
| UE supports only 4RX or both 2RX and 4RX | CQI | All tests in Clause 6.2.3 |
|  | PMI | All tests in Clause 6.3.3 |
|  | RI | All tests in Clause 6.4.3 |

#### 6.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 6.1.1.3-1 shall apply for UEs which support optional UE features with capability signalling only.

Table 6.1.1.3-1: Requirements applicability for optional features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| CQI table with target BLER of 10^-5New CQI table (cqi-TableAlt) | FR1 FDD | CQI | Clause 6.2.2.1.1.2 Clause 6.2.3.1.1.2 |  |
|  | FR1 TDD | CQI | Clause 6.2.2.2.1.2 Clause 6.2.3.2.1.2 |  |
| Alternative 64QAM MCS table for PDSCH New 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR1 FDD | CQI | Clause 6.2.2.1.1.2 Clause 6.2.3.1.1.2 |  |
|  | FR1 TDD | CQI | Clause 6.2.2.2.1.2 Clause 6.2.3.2.1.2 |  |
| Validating P/SP-CSI-RS reception (periodicAndSemi-PersistentCSI-RS-r16) | FR1 TDD | CQI | Clause 6.2.2.2.1.4 Clause 6.2.3.2.1.4 Clause 6.2A.3.1.2 Clause 6.2A.4.1.1 | The requirements apply only in case tested UE supporting operations in shared spectrum access and validation of P/SP-CSI-RS reception based on DCI |
| Supported UL channels for dynamic channel access mode (ul-DynamicChAccess-r16 ) or UL channel access for semi-static channel access mode (ul-Semi-StaticChAccess-r16) or both | FR1 TDD | CQI | Clause 6.2.2.2.1.4 Clause 6.2.3.2.1.4 | The requirements apply only in case tested UE supports one of UL channels for dynamic channel access mode and UL channel access for semi-static channel access mode |

The performance requirements in Table 6.1.1.3-2 shall apply for UEs which support optional UE features only.

Table 6.1.1.3-2: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Support of Type II codebook (CodebookParameters contains type2, supportedCSI-RS-ResourceList, parameterLx, amplitudeScalingType, amplitudeSubsetRestriction) | FR1 FDD | PMI | Clause 6.3.2.1.5 Clause 6.3.3.1.5 |  |
|  | FR1 TDD | PMI | Clause 6.3.2.2.5 Clause 6.3.3.2.5 |  |
| Support of Enhanced Type II codebook with at least 16 ports per CSI-RS resource(codebookParametersAddition-r16 contains etype2R1-r16, supportedCSI-RS-ResourceListAdd-r16, maxNumberTxPortsPerResource) | FR1 FDD | PMI | Clause 6.3.2.1.6 Clause 6.3.3.1.6 |  |

#### 6.1.1.4	Applicability of requirements for mandatory UE features with capability signalling

The performance requirements in Table 6.1.1.4-1 shall apply for UEs which support mandatory UE features with capability signalling only.

Table 6.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR1 FDD | CQI | Clause 6.2.3.1.1.1 | The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed UE PDSCH MIMO layers capability |
|  |  | PMI | Clause 6.3.3.1.2 |  |
|  |  | RI | Clause 6.4.2.1 Clause 6.4.3.1 |  |
|  | FR1 TDD | CQI | Clause 6.2.3.2.1.1 |  |
|  |  | PMI | Clause 6.3.3.2.2 |  |
|  |  | RI | Clause 6.4.2.2 Clause 6.4.3.2 |  |
| Supported maximum number of ports across all configured NZP-CSI-RS resources per CC (maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC) | FR1 FDD | PMI | Clause 6.3.2.1.1 Clause 6.3.2.1.2 Clause 6.3.2.1.3 Clause 6.3.2.1.4 Clause 6.3.3.1.1 Clause 6.3.3.1.2 Clause 6.3.3.1.3 Clause 6.3.3.1.4 | The requirements apply only in case the number of NZP-CSI-RS ports in the test case satisfies UE capability on maximum number of NZP-CSI-RS ports |
|  |  | RI | Clause 6.4.3.1 (Test 4) |  |
|  | FR1 TDD | PMI | Clause 6.3.2.2.1 Clause 6.3.2.2.2 Clause 6.3.2.2.3 Clause 6.3.2.2.4 Clause 6.3.3.2.1 Clause 6.3.3.2.2 Clause 6.3.3.2.3 Clause 6.3.3.2.4 |  |
|  |  | RI | Clause 6.4.3.2 (Test 4) |  |

#### 6.1.1.5	Applicability of Channel Quality Indicator (CQI) reporting requirements for CA

##### 6.1.1.5.1	Applicability and test rules for different duplex modes and SCS combinations

The applicability and test rules for different duplex modes and SCS combinations are defined in Table 6.1.1.5.1-1.

Table 6.1.1.5.1-1: Applicability for different duplex modes and SCS combinations

| Tests | PCell CC configuration |
| --- | --- |
| Test 1 in Clause 6.2A.3.1.1 | TDD CC if supported, otherwise FDD CC |
| Test 2 in Clause 6.2A.3.1.1 (NOTE 2) | Any of CCs |
| Test 3 in Clause 6.2A.3.1.1 | Any of CCs |
| NOTE 1: The test coverage can be considered fulfilled if UE passes one of the CC as PCell in Test 1. NOTE 2: These scenarios are only tested for UEs which are not verified with Test 1 in Clause 6.2A.3.1.1. |  |

##### 6.1.1.5.2	Applicability and test rules for different CA configurations and bandwidth combination sets

The performance requirement for CA CQI tests in clause 6.2A are defined independent of CA configurations and bandwidth combination sets specified in clasue 5.5A in TS 38.101-1 [6].

For UEs supporting multiple CA capabilities, test any one of the supported CA capabilities with largest aggregated CA bandwidth combination. The categorization of CA capability is specified in clasue 5.1.1.7.1.

For UEs supporting multiple CA configurations from the selected CA capability, test any one of the supported CA configurations with largest aggregated CA bandwidth combination. For simplicity, the CA configuration refers to combination of CA configuration and bandwidth combination set.

A single uplink CC is configured for all tests.

##### 6.1.1.5.3	Test coverage for different number of componenet carriers

For CA CQI tests specified in clause 6.2A, among all supported CA capabilities, if corresponding CA tests with the largest number of CCs supported by the UE are tested, the test coverage can be considered fulfilled without executing the CA tests with less than the largest number of CCs supported by the UE.

##### 6.1.1.5.4	Applicability rule and antenna connection for CA tests with 4 RX

All the requirements specified in clause 6.2A for CA with 2 RX are applied for 4 RX capable UEs by connecting all 4 RX with data source from system simulator and reducing the signal power density by 3 dB compared to the signal power density for 2 RX in the test configurations.

### 6.1.2	Common test parameters

Parameters specified in Table 6.1.2-1 are applied for all test cases in this clause unless otherwise stated.

Table 6.1.2-1: Test parameters for CSI test cases

| Parameter |  |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  |  |  | Transmission scheme 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 3) |  |  |  | RBs | 0 |
|  | Subcarrier spacing |  |  |  | kHz | 15 or 30 |
| DL BWP configuration #1 | Cyclic prefix |  |  |  |  | Normal |
|  | RB offset |  |  |  | RBs | 0 |
|  | Number of contiguous PRB |  |  |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |
| Active DL BWP index |  |  |  |  |  | 1 |
| Common serving cell parameters |  | Physical Cell ID |  |  |  | 0 |
|  |  | SSB position in burst |  |  |  | First SSB in Slot #0 |
|  |  | SSB periodicity |  |  | ms | 20 |
| PDCCH configuration |  | Slots for PDCCH monitoring |  |  |  | Each slot |
|  |  | Symbols with PDCCH |  |  |  | 0,1 |
|  |  | Number of PDCCH candidates and aggregation levels |  |  |  | 1/AL8 |
|  |  | DCI format |  |  |  | 1_1 |
|  |  | TCI state |  |  |  | TCI state #1 |
| Cross carrier scheduling |  |  |  |  |  | Not configured |
| PDSCH configuration |  | Mapping type |  |  |  | Type A |
|  |  | k0 |  |  |  | 0 |
|  |  | Starting symbol (S) |  |  |  | 2 |
|  |  | Length (L) |  |  |  | 12 |
|  |  | PDSCH aggregation factor |  |  |  | 1 |
|  |  | PRB bundling type |  |  |  | Static |
|  |  | PRB bundling size |  |  |  | 2 |
|  |  | Resource allocation type |  |  |  | type 0 |
|  |  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |
|  |  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |
|  |  | PDCCH & PDCCH DMRS Precoding configuration |  |  |  | Multi-path fading propagation conditions: Single Panel Type I, Random per slot with equal probability of each applicable i1, i2 combination, and with REG bundling granularity for number of Tx larger than 1 Static propagation conditions: Single Panel Type I, Random precoder chosen from precoder index 0 an 2, selection updated per slot |
| PDSCH DMRS configuration |  | DMRS Type |  |  |  | Type 1 |
|  |  | Number of additional DMRS |  |  |  | 1 |
|  |  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |
|  |  | DMRS ports indexes |  |  |  | {1000} for Rank1 {1000,1001} for Rank2 {1000,1001,1002} for Rank3 {1000,1001,1002,1003} for Rank4 |
|  |  | Number of PDSCH DMRS CDM group(s) without data |  |  |  | 2 |
| PTRS configuration |  | Frequency density (KPT-RS) |  |  |  | N/A |
|  |  | Time density (LPT-RS) |  |  |  | N/A |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 for CSI-RS resource 1,2,3,4 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 4 for CSI-RS resource 1 and 3 8 for CSI-RS resource 2 and 4 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  |  |  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  |  |  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  |  |  | CSI-RS periodicity |  | slot | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource |
|  |  |  | CSI-RS offset |  | slot | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  |  |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  |  |  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition |  |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  |  |  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition |  |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| TCI state #0 |  |  | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  |  |  | QCL Type |  | Type C |
|  |  |  | Type 2 QCL information | SSB index |  | N/A |
|  |  |  |  | QCL Type |  | N/A |
| TCI state #1 |  |  | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  |  |  | QCL Type |  | Type A |
|  |  |  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  |  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  |  |  | 4 For FDD 8 for TDD |
| HARQ ACK/NACK bundling |  |  |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  |  |  | {0,2,3,1} |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  |  |  |  | 2 for FDD For FR1.30-1: 8 if mod(i,10) = 0 6 if mod(i,10) = 2 5 if mod(i,10) = 3 5 if mod(i,10) = 4 4 if mod(i,10) = 5 3 if mod(i,10) = 6 Where i is slot index per radio frame with 0~19 For FR1.30-7: 8 if mod(i,10) = 0 7 if mod(i,10) = 1 6 if mod(i,10) = 2 5 if mod(i,10) = 3 4 if mod(i,10) = 4 3 if mod(i,10) = 5 2 if mod(i,10) = 6 Where i is the slot index of all slots in every 5ms i = {0,…,9} |
| Symbols for unused REs |  |  |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| Physical signals, channels mapping and precoding |  |  |  |  |  | As specified in Annex B.4.1 |
| Note 1: PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL. Note 2: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 3: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. |  |  |  |  |  |  |

End of Change 7

## 6.2	Reporting of Channel Quality Indicator (CQI)

### 6.2.2	2RX requirements

#### 6.2.2.2	TDD

##### 6.2.2.2.1	CQI reporting definition under AWGN conditions

Start of Change 8

###### 6.2.2.2.1.4	Minimum requirement for CQI reporting for PCell on band with shared spectrum access

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [12] for PCell on band with shared spectrum access. For each Downlink Transmission Duration the transmission power offset is randomly chosen between [0, +6] dB and 2 sets of CQI reports are obtained for each transmission power offset. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median for each power offset. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

For the parameters specified in Table 6.2.2.2.1.4-1, and using the downlink physical channels specified in Annex A.4, the minimum requirements are specified by the following:

a)	For each transmission power offset the reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	For each transmission power offset, if the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. For each transmission power offset, if the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

c)  The absolute difference in median CQI for each of transmission power offset shall be ≥ 2.

Table 6.2.2.2.1.4-1: CQI reporting test parameters for PCell on band with shared spectrum access

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 |  |
| Subcarrier spacing |  |  | kHz | 30 |  |
| Duplex Mode |  |  |  | TDD |  |
| Downlink Transmission Model |  |  |  | As specified in Annex B.5 |  |
| Downlink Transmission Model Parameters |  | Downlink period |  | ms | 5 |  |
|  |  | LBT failure probability (pLBT) |  |  | 0.25 |  |
|  |  | Downlink transmission duration values set |  | slot | {4,6,7} |  |
|  |  | Occupied OFDM symbols in slot other than the last slot of the downlink duration |  | symbol | 14 |  |
|  |  | Occupied OFDM symbols in the last slot set of the downlink duration |  | symbol | 14 |  |
| TDD UL-DL pattern |  |  |  | FR1.30-7 |  |
| SNR |  |  | dB | 8 | 9 |
| Es for power offset 1 |  |  | dBm/Hz | -112 |  |
| Es for power offset 2 |  |  | dBm/Hz | -106 |  |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  | 4 |  |
|  |  | CDM Type |  | FD-CDM2 |  |
|  |  | Density (ρ) |  | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |
|  |  | CSI-RS interval and offset |  | slot | Not configured |  |
|  |  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource Type |  | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  | 2 |  |
|  |  | CDM Type |  | FD-CDM2 |  |
|  |  | Density (ρ) |  | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3,(6,-) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 3 |  |
|  |  | CSI-RS interval and offset | slot | Not configured |  |
|  |  | aperiodicTriggeringOffset | slot | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  | Aperiodic |  |
|  |  | CSI-IM RE pattern |  | 0 |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) |  |
|  |  | CSI-IM timeConfig interval and offset |  | slot | Not configured |  |
| ReportConfigType |  |  |  | Aperiodic |  |
| CQI-table |  |  |  | Table 2 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 8 |  |
| csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report interval and offset |  |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  |  | 7 |  |
| CSI request |  |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| reportTriggrtSize |  |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUSCH |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-8 |  |

End of Change 8

### 6.2.3	4RX requirements

#### 6.2.3.2	TDD

##### 6.2.3.2.1	CQI reporting definition under AWGN

Start of Change 9

###### 6.2.3.2.1.4	Minimum requirement for CQI reporting for PCell on band with shared spectrum access

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [12] for PCell on band with shared spectrum access.  For each Downlink Transmission Duration the transmission power offset is randomly chosen between [0, +6] dB and 2 sets of CQI reports are obtained for each transmission power offset. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median for each power offset. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

For the parameters specified in Table 6.2.3.2.1.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	For each transmission power offset the reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	For each transmission power offset, if the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. For each transmission power offset, if the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

c)  The absolute difference in median CQI for each of transmission power offset shall be ≥ 2.

Table 6.2.2.2.1.4-1: CQI reporting test parameters for PCell on band with shared spectrum access

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 |  |
| Subcarrier spacing |  |  | kHz | 30 |  |
| Duplex Mode |  |  |  | TDD |  |
| Downlink Transmission Model |  |  |  | As specified in Annex B.5 |  |
| Downlink Transmission Model Parameters |  | Downlink period |  |  | 5 |  |
|  |  | LBT failure probability (pLBT) |  |  | 0.25 |  |
|  |  | Downlink transmission duration values set |  |  | {4,6,7} |  |
|  |  | Occupied OFDM symbols in slot other than the last slot of the downlink duration |  |  | 14 |  |
|  |  | Occupied OFDM symbols in the last slot set of the downlink duration |  |  | 14 |  |
| TDD UL-DL pattern |  |  |  | FR1.30-7 |  |
| SNR |  |  | dB | 5 | 6 |
| Es for power offset 1 |  |  | dBm/Hz | -112 |  |
| Es for power offset 2 |  |  | dBm/Hz | -106 |  |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 2×4 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  | 4 |  |
|  |  | CDM Type |  | FD-CDM2 |  |
|  |  | Density (ρ) |  | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |
|  |  | CSI-RS interval and offset |  | slot | Not configured |  |
|  |  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource Type |  | Periodic |  |
|  |  | Number of CSI-RS ports (X) |  | 2 |  |
|  |  | CDM Type |  | FD-CDM2 |  |
|  |  | Density (ρ) |  | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3,(6,-) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 3 |  |
|  |  | CSI-RS interval and offset | slot | Not configured |  |
|  |  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  | Aperiodic |  |
|  |  | CSI-IM RE pattern |  | 0 |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) |  |
|  |  | CSI-IM timeConfig interval and offset |  | slot | Not configured |  |
| ReportConfigType |  |  |  | Aperiodic |  |
| CQI-table |  |  |  | Table 2 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 8 |  |
| csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report interval and offset |  |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  |  | 7 |  |
| CSI request |  |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| reportTriggrtSize |  |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUSCH |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-8 |  |

## 6.2A	Reporting of Channel Quality Indicator (CQI) for CA

### 6.2A.3	2RX requirements

#### 6.2A.3.1	CQI reporting definition under AWGN conditions

Start of Change 10

6.2A.3.1.2	Minimum requirement for CQI reporting for SCell on band with shared spectrum access

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [12] for Scell on band with shared spectrum access. For each downlink transmission duration the transmission power offset is randomly chosen between [0, +6] dB and 2 sets of CQI reports are obtained for each transmission power offset. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median for each power offset. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

For the parameters specified in Table 6.2A.3.1.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	For each transmission power offset the reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	For each transmission power offset, if the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. For each transmission power offset, if the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

c)  The absolute difference in median CQI for each of transmission power offset shall be ≥ [2].

The test parameters for configuring the PCell are specified in Table 6.2A.3.1.2-2, but requirements are only applicable to SCell on band with shared spectrum access.

Table 6.2A.3.1.2-1: CQI reporting test parameters for SCell on band with shared spectrum access

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 |  |
| Subcarrier spacing |  |  | kHz | 30 |  |
| Duplex Mode |  |  |  | TDD |  |
| Downlink Transmission Model |  |  |  | As specified in Annex B.5 |  |
| Downlink Transmission Model Parameters |  | Downlink period |  | ms | 5 |  |
|  |  | LBT failure probability (pLBT) |  |  | 0.25 |  |
|  |  | Downlink transmission duration values set |  | slot | {3,4,6,7} |  |
|  |  | Occupied OFDM symbols in slot other than the last slot of the downlink duration |  | symbols | 14 |  |
|  |  | Occupied OFDM symbols in the last slot of the downlink duration |  | symbols | [14] |  |
| TDD UL-DL pattern |  |  |  | FR1.30-7 |  |
| SNR |  |  | dB | [8] | [9] |
| Es for power offset 1 |  |  | dBm/Hz | -112 |  |
| Es for power offset 2 |  |  | dBm/Hz | -106 |  |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  | Periodic |  |
|  |  | Number of CSI-RS ports (X) |  | 4 |  |
|  |  | CDM Type |  | FD-CDM2 |  |
|  |  | Density (ρ) |  | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |
|  |  | CSI-RS periodicity and offset | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource Type |  | [Aperiodic] |  |
|  |  | Number of CSI-RS ports (X) |  | 2 |  |
|  |  | CDM Type |  | FD-CDM2 |  |
|  |  | Density (ρ) |  | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3,(6,-) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 3 |  |
|  |  | NZP CSI-RS-timeConfig periodicity and offset | slot | [Not configured] |  |
|  |  | aperiodicTriggeringOffset |  | [0] |  |
| CSI-IM configuration |  | CSI-IM resource Type |  | Periodic |  |
|  |  | CSI-IM RE pattern |  | 0 |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) |  |
|  |  | CSI-IM timeConfig periodicity and offset | slot | 10/1 |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |
|  |  | RI Restriction |  | N/A |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-8 |  |

Table 6.2A.3.1.2-2: Configuration parameters for PCell

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Bandwidth | MHz | 20 |
| Subcarrier spacing | kHz | 30 |
| Duplex Mode |  | TDD |
| TDD UL-DL pattern |  | FR1.30-1 |
| Propagation channel |  | AWGN |
| Antenna configuration |  | 2×2 with static channel specified in Annex B.1 |
| Beamforming Model |  | As specified in Annex B.4.1 |
| ReportConfigType |  | Aperiodic |
| CQI-table |  | Table 2 |
| reportQuantity |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  | configured |
| timeRestrictionForInterferenceMeasurements |  | configured |
| cqi-FormatIndicator |  | Wideband |
| pmi-FormatIndicator |  | Wideband |
| Sub-band Size | RB | 16 |
| Csi-ReportingBand |  | 1111111 |
| CSI-Report periodicity and offset | slot | Not configured |
| aperiodicTriggeringOffset |  | 7 |
| CSI request |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  | 1 |
| CSI-AperiodicTriggerStateList |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Physical channel for CSI report |  | PUSCH |

End of Change 10

Start of Change 11

### 6.2A.4	4RX requirements

#### 6.2A.4.1	CQI reporting definition under AWGN conditions

6.2A.4.1.1	Minimum requirement for CQI reporting for SCell on band with shared spectrum access

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [12] for Scell on band with shared spectrum access. For each downlink transmission duration the transmission power offset is randomly chosen between [0, +6] dB and 2 sets of CQI reports are obtained for each transmission power offset. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median for each power offset. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

For the parameters specified in Table 6.2A.4.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	For each transmission power offset the reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	For each transmission power offset, if the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. For each transmission power offset, if the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

c)  The absolute difference in median CQI for each of transmission power offset shall be ≥ [2].

The test parameters for configuring the PCell are specified in Table 6.2A.4.1.1-2, but requirements are only applicable to SCell on band with shared spectrum access.

Table 6.2A.4.1.1-1: CQI reporting test parameters for SCell on band with shared spectrum access

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 |  |
| Subcarrier spacing |  |  | kHz | 30 |  |
| Duplex Mode |  |  |  | TDD |  |
| Downlink Transmission Model |  |  |  | As specified in Annex B.5 |  |
| Downlink Transmission Model Parameters |  | Downlink period |  | ms | 5 |  |
|  |  | LBT failure probability (pLBT) |  |  | 0.25 |  |
|  |  | Downlink transmission duration values set |  | slot | {3,4,6,7} |  |
|  |  | Occupied OFDM symbols in slot other than the last slot of the downlink duration |  | symbols | 14 |  |
|  |  | Occupied OFDM symbols in the last slot of the downlink duration |  | symbols | [14] |  |
| TDD UL-DL pattern |  |  |  | FR1.30-7 |  |
| SNR |  |  | dB | [5] | [6] |
| Es for power offset 1 |  |  | dBm/Hz | -112 |  |
| Es for power offset 2 |  |  | dBm/Hz | -106 |  |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 2×4 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  | Periodic |  |
|  |  | Number of CSI-RS ports (X) |  | 4 |  |
|  |  | CDM Type |  | FD-CDM2 |  |
|  |  | Density (ρ) |  | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |
|  |  | CSI-RS periodicity and offset | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource Type |  | [Aperiodic] |  |
|  |  | Number of CSI-RS ports (X) |  | 2 |  |
|  |  | CDM Type |  | FD-CDM2 |  |
|  |  | Density (ρ) |  | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3,(6,-) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 3 |  |
|  |  | NZP CSI-RS-timeConfig periodicity and offset | slot | [Not configured] |  |
|  |  | aperiodicTriggeringOffset |  | [0] |  |
| CSI-IM configuration |  | CSI-IM resource Type |  | Periodic |  |
|  |  | CSI-IM RE pattern |  | 0 |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) |  |
|  |  | CSI-IM timeConfig periodicity and offset | slot | 10/1 |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |
|  |  | RI Restriction |  | N/A |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-8 |  |

Table 6.2A.4.1.1-2: Configuration parameters for PCell

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Bandwidth | MHz | 20 |
| Subcarrier spacing | kHz | 30 |
| Duplex Mode |  | TDD |
| TDD UL-DL pattern |  | FR1.30-1 |
| Propagation channel |  | AWGN |
| Antenna configuration |  | 2×4 with static channel specified in Annex B.1 |
| Beamforming Model |  | As specified in Annex B.4.1 |
| ReportConfigType |  | Aperiodic |
| CQI-table |  | Table 2 |
| reportQuantity |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  | configured |
| timeRestrictionForInterferenceMeasurements |  | configured |
| cqi-FormatIndicator |  | Wideband |
| pmi-FormatIndicator |  | Wideband |
| Sub-band Size | RB | 16 |
| Csi-ReportingBand |  | 1111111 |
| CSI-Report periodicity and offset | slot | Not configured |
| aperiodicTriggeringOffset |  | 7 |
| CSI request |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  | 1 |
| CSI-AperiodicTriggerStateList |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Physical channel for CSI report |  | PUSCH |

End of Change 11

## A.1.2	TDD UL-DL configurations for FR1

Start of Change 12

Table A.1.2-2b: TDD UL-DL configuration for SCS 30 kHz for PDSCH on band with shared spectrum access

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
|  |  |  | FR1.30-7 |
| TDD Slot Configuration pattern (Note 1) |  |  | 7DS2U |
| Special Slot Configuration (Note 2) |  |  | 6D+4G+4U |
| referenceSubcarrierSpacing |  | kHz | 30 |
| Pattern 1 | dl-UL-TransmissionPeriodicity | ms | 5 |
|  | nrofDownlinkSlots | slot | 7 |
|  | nrofDownlinkSymbols | symbol | 6 |
|  | nrofUplinkSlot | slot | 2 |
|  | nrofUplinkSymbols | symbol | 4 |
| Pattern 2 | dl-UL-TransmissionPeriodicity | ms | N/A |
|  | nrofDownlinkSlots |  | N/A |
|  | nrofDownlinkSymbols |  | N/A |
|  | nrofUplinkSlot |  | N/A |
|  | nrofUplinkSymbols |  | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  |  | 8 if mod(i,10) = 0 7 if mod(i,10) = 1 6 if mod(i,10) = 2 5 if mod(i,10) = 3 4 if mod(i,10) = 4 3 if mod(i,10) = 5 2 if mod(i,10) = 6 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index of all slots in every 5ms i = {0,…,9} Note 4: The slot i, mod (i,10)=9 is idle slot with no UL transmission. |  |  |  |

End of Change 12

## A.3.2	Reference measurement channels for PDSCH performance requirements

### A.3.2.2	TDD

#### A.3.2.2.2	Reference measurement channels for SCS 30 kHz FR1

Start of Change 13

Table A.3.2.2.2-18: PDSCH Reference Channel for PDSCH on band with shared spectrum access with TDD UL-DL pattern FR1.30-7

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-18.1 TDD | R.PDSCH.1-18.2 TDD | R.PDSCH.1-18.3 TDD | R.PDSCH.1-18.4 TDD |
| Channel bandwidth | MHz | 20 | 40 | 60 | 80 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 51 | 106 | 162 | 217 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slot 0 and slot i, if mod(i, 10) =7 for i from {0,…,39} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {3, 5, 6} for i from {1,…,39} | symbol | {4,7,10,12} | {4,7,10,12} | {4,7,10,12} | {4,7,10,12} |
| For Slot i, if mod(i, 10) = {0, 1, 2, 4} for i from {1,…,39} | symbol | 12 | 12 | 12 | 12 |
| 3 Allocated slots per 2 frames | slot | 31 | 31 | 31 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 13 | 13 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slot 0 and slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {3, 5, 6} for i from {1,…,39} | symbol | {6,12} | {6,12} | {6,12} | {6,12} |
| For Slot i, if mod(i, 10) = {0, 1, 2, 4} for i from {1,…,39} | symbol | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slot 0 and slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {3, 5, 6} for i from {1,…,39} | Bits | {8192,14088， 16392,25608} | {16896,29192,44040,53288} | {26120,45096, 67584,81976,} | {34816,60456, 90176,110632} |
| For Slot i, if mod(i, 10) = {0, 1, 2, 4} for i from {1,…,39} | Bits | 25608 | 53288 | 81976 | 110632 |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slot 0 and slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slot 0 and slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {3, 5, 6} for i from {1,…,39} | CBs | {1,2,4,4} | {3,4,6,7} | {4,6,9,10} | {5,8,11,14} |
| For Slot i, if mod(i, 10) = {0, 1, 2, 4} for i from {1,…,39} | CBs | 4 | 7 | 10 | 14 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slot 0 and slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {3, 5, 6} for i from {1,…,39} | Bits | {17136,29376,44064,53865} | {35616,61056,91854,11193} | {54432,93312, 139968,171072} | {72912,124992,187488,229152} |
| For Slot i, if mod(i, 10) = {0, 1, 2, 4} for i from {1,…,39} | Bits | 53865 | 111936 | 171073 | 229152 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: For {a1,a2,a3,a4 }, a1, a2, a3 and a4 stand for the setup when the number of OFDM sybmols is 6,9,12,14 respectively. Note 5: The slot i, mod (i,10)=9 is idle slot with no UL transmission. |  |  |  |  |  |

End of Change 13

# A.4	CSI reference measurement channels

This clause defines the DL signal applicable to the reporting of channel status information (Clause X).

Tables in this clause specifies the mapping of CQI index to Information Bit payload, which complies with the CQI definition specified in clause 5.2.2.1 of TS 38.214 [12] and with MCS definition specified in clause 5.1.3 of TS 38.214 [12].

Start of Change 14

Table A.4-2: Mapping of CQI Index to Information Bit payload (CQI table 2)

| TBS Scheme |  |  |  | TBS.2-1 | TBS.2-2 | TBS.2-3 | TBS.2-4 | TBS.2-5 | TBS.2-6 | TBS.2-7 | TBS.2-8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 256QAM |  |  |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 52 | 106 | 106 | 8 | 16 | 32 | 51 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 | 1 | 2 | 1 | 1 | 1 | 2 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 0 |
| Available RE-s for PDSCH |  |  |  | 6240 | 6240 | 12720 | 12720 | 960 | 1920 | 3680 | 6120 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 1 | 0.1523 | 0 | QPSK | 1480 | 2976 | 2976 | 5896 | 224 | 456 | 848 | 1864 |
| 2 | 0.3770 | 1 |  | 2408 | 4744 | 4744 | 9480 | 368 | 736 | 1416 | 4616 |
| 3 | 0.8770 | 3 |  | 5504 | 11016 | 11016 | 22536 | 848 | 1736 | 3240 | 10760 |
| 4 | 1.4766 | 5 | 16QAM | 9224 | 18432 | 18960 | 37896 | 1416 | 2856 | 5376 | 17928 |
| 5 | 1.9141 | 7 |  | 12040 | 24072 | 24576 | 49176 | 1864 | 3752 | 6912 | 23568 |
| 6 | 2.4063 | 9 |  | 15112 | 30216 | 30728 | 61480 | 2408 | 4608 | 8712 | 29192 |
| 7 | 2.7305 | 11 | 64QAM | 16896 | 33816 | 34816 | 69672 | 2600 | 5248 | 9992 | 33816 |
| 8 | 3.3223 | 13 |  | 20496 | 40976 | 42016 | 83976 | 3240 | 6400 | 12040 | 40976 |
| 9 | 3.9023 | 15 |  | 24576 | 49176 | 49176 | 98376 | 3752 | 7424 | 14344 | 48168 |
| 10 | 4.5234 | 17 |  | 28168 | 56368 | 57376 | 114776 | 4352 | 8712 | 16392 | 55304 |
| 11 | 5.1152 | 19 |  | 31752 | 63528 | 65576 | 131176 | 4864 | 9736 | 18432 | 62504 |
| 12 | 5.5547 | 21 | 256QAM | 34816 | 69672 | 69672 | 139376 | 5248 | 10760 | 20496 | 67584 |
| 13 | 6.2266 | 23 |  | 38936 | 77896 | 79896 | 159880 | 6016 | 12040 | 22536 | 75792 |
| 14 | 6.9141 | 25 |  | 43032 | 86040 | 88064 | 176208 | 6656 | 13320 | 25104 | 83976 |
| 15 | 7.4063 | 27 |  | 46104 | 92200 | 94248 | 188576 | 7040 | 14088 | 27144 | 90176 |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity |  |  |  |  |  |  |  |  |  |  |  |

End of Change 14

Start of Change 15

# B.5	Downlink Transmission Model for requirements on bands with shared spectrum access

This clause provides a description of the Downlink Transmission Model to be used in PDSCH Demodulation and CQI reporting performance tests on bands with shared spectrum access.

The model as designed in the following applies to both configurations for channelAccessMode=’semiStatic’ or ‘dynamic’.

## B.5.1	Downlink Transmission Model for bands with shared spectrum access

The Downlink Period for performance tests on bands with shared spectrum access is defined as the duration included in the Test Configuration Parameters.

For tests configured with the RRC Parameter channelAccessMode=’semiStatic’, the duration of the Fixed Frame Period (FFP) equals the duration of the Downlink Period.

For each Downlink Period, the downlink signal to be transmitted is allocated according to the steps listed below:

- Select the Downlink Transmission Duration in number of slots, randomly and with equally distributed probability, from the set of possible Downlink Transmission Duration values as included in the Test Configuration Parameters;

  - This duration includes occupied OFDM symbols and non-occupied OFDM symbols within the Downlink Transmission;

- Depending on the Downlink Transmission Duration chosen in the previous step:

  - If the Downlink Transmission Duration equals 2 slots, all the OFDM symbols in both slots are fully allocated to downlink transmission, else;

  - If the Downlink Transmission Duration is larger than 2 slots, the configuration of occupied symbols in the last slot included in the downlink duration is selected in number of symbols, randomly and with equally distributed probability, from the set of possible ‘Occupied OFDM symbols in the last slot of the downlink duration’ as included in the Test Configuration Parameters;

For each Downlink Period, the last Slot is not scheduled for downlink transmission. This is to comply with the Idle period requirement in case of channelAccessMode=’semiStatic’, and to align the test setup. In the case of channelAccessMode=’semiStatic’, it can be assumed that the Channel Occupancy Time (COT) covers the entire duration of the Downlink Period except for the last slot.

For each Downlink Period, a uniform random variable from [0, 1] is generated. If the random variable is less than the pLBT value included in the Test Configuration Parameters, the entire Downlink Period duration is muted across the entire bandwidth. This applies to all the signals that were scheduled for transmission, including but not limited to PDSCH, PDCCH, SSB, TRS, CSI-RS, etc.

End of Change 15