3GPP TSG-RAN WG4 Meeting #99-e	R4-2108654

Electronic Meeting, 19th – 27th May, 2021

Source:		Rohde & Schwarz

Title:	Analysis of NF based solutions

Agenda Item:		10.1.2

Document for:	Approval

# Introduction

According to the SID [1] the first objective is to improve the test methodology for high DL and low UL power test cases based on the feedback RAN5 provided in [2] declaring testability issues on some of the core requirements in TS 38.101-2 [3].

In this contribution we present our views and proposal for this first objective.

# CFFNF based on asymptotic expansion

During past few meetings, Near Field systems based on asymptotic expansion approach using a rate-of-decay estimation has been presented in [4][5][6]. Although the detailed implementation is still not fixed, we have tried our own implementation with the available information in order to analyze the proposed methodology.

## Formulation of CFFNF based on asymptotic expansion

During last meeting, [4] proposed a revised formulation for the asymptotic expansion approach:

The estimate of the EIRP based on NF measurements can be expressed as follows

EIRPdf=pd1+ d1df∂p∂d d∆d

with

where EIRP(df) is the estimated EIRP in the far-field at range length r and df is the distance between the phase centre of the antenna array and the far-field BP at range length r while p(d1) is the measured power with the probe antenna at a near-field distance d1 (measured from the phase centre of the antenna array to probe antenna), ∂p/∂d is the derivative of power p to distance d. Because the near-field distance d1 is unknown for the black box approach, measurements of the EIRP at multiple measurement distances are needed to determine the parameters of the ∂p/∂d derivative and the first near-field distance d1.

In [9], an initial approximation of power to distance ∂p/∂d was presented

∂p∂d=a1d-2+a2

Following the implementation of the CFFNF simulations and an in-depth analysis of the derivative of the superposition expression above, a more suitable approximation of power to distance ∂p/∂d was determined to be

∂p∂d=b1d-3+b2

Using this formulation based on ∂p∂d=b1d-3+b2 , a set of simulations following the assumptions presented in section 3, including Black&White box approach, were performed in order to validate the implementation.

Table 2-1: Statistical results for CFFNF based on black&white box approach.

| UE Antenna Array Configuration | Range Length [m] | Mean EIRP error [dB] | EIRP Std. Deviation [dB] |
| --- | --- | --- | --- |
| 4x1 | 0.20 and 0.21 | 0.005 | 0.005 |
| 8x2 | 0.20 and 0.21 | 0.062 | 0.030 |
|  | 0.25 and 0.26 | 0.027 | 0.014 |

Comparing the results in Table 2-1 to those presented in [4], they show good correlation although the constant b2 had to be set to 0. Otherwise, inconsistent results were observed.

Observation 1: inconsistent results are observed for asymptotic expansion approach when b2≠0.

Proposal 1: a clear and detailed formulation of the CFFNF methodology based on the asymptotic expansion must be provided.

## Sensitivity of the expansion to SNR and measurement error

As presented in [12], extrapolation techniques like the asymptotic expansion approach are very much sensitive to measurement errors and noise effects. In order to assess these effects, a set of simulations were performed using the implementation described in 2.1 and the corresponding assumptions in section 3.

Following the approach described in [13] for the influence of measurement uncertainty on asymptotic expansion approach, Table 2-2 shows the simulation results of 500 UE antenna offsets as described in section 3 and 100 iterations for each of them, where an error with std. deviation σ is added to each of the 2 measurements at d1 and d2, 20 and 22cm respectively.

Table 2-2: Statistical results for the sensitivity of CFFNF expansion approach to measurement error.

| UE Antenna Array Configuration | Range Length [m] | Number of averages | Relative measurement error [σ] | Mean EIRP error [dB] | EIRP Std. Deviation [dB] |
| --- | --- | --- | --- | --- | --- |
| 8x2 | 0.20 and 0.22 | None | 0.1 | 0.035 | 0.249 |
|  |  |  | 0.3 | 0.017 | 0.730 |
|  |  |  | 0.5 | 0.136 | 1.347 |
|  |  | 30 | 0.1 | 0.041 | 0.093 |
|  |  |  | 0.3 | 0.039 | 0.152 |
|  |  |  | 0.5 | 0.035 | 0.229 |

Based on these results, there is a non-negligible source of error dependent on the selected number of averages and the relative measurement error, and thus a MU contributor is required to account for it.

Proposal 2: a MU contributor for the sensitivity of CFFNF expansion approach to measurement error is required.

## Error due to declared antenna offset error

Under Black&White box approach conditions, where the active antenna phase center offset from center of the DUT is declared by the manufacturer, there might be errors introduced in the EIRP measurement due to an incorrect estimation of the beam peak and the corresponding approximation of power to distance for CFFNF based on the asymptotic expansion approach.

Table 2-3 shows the simulation results of 500 UE antenna offsets as described in section 3 including random offset error in x, y and z directions to simulate the error in the manufacturer declaration.

Table 2-3: Statistical results for the sensitivity of CFFNF expansion approach to declared antenna offset error under black&white box approach.

| UE Antenna Array Configuration | Range Length [m] | Error in declared antenna offset [mm] | Mean EIRP error [dB] | EIRP Std. Deviation [dB] |
| --- | --- | --- | --- | --- |
| 8x2 | 0.20 and 0.22 | 1 | 0.041 | 0.083 |
|  |  | 5 | 0.031 | 0.126 |
|  |  | 10 | 0.050 | 0.420 |

Given that it was agreed in [7] that black&white box approach can be based on manufacturer declaration as a baseline, a MU contributor is required to account for the error in the declared antenna offset.

# Proposal 3: a MU contributor for the sensitivity of CFFNF expansion approach to declared antenna offset error under black&white box approach is required.CFFDNF based solutions

Following the discussion in previous meetings, the applicability for CFFDNF was defined in [8] as follows:

The CFFDNF has the following applicability:

-	Beam peak searches and spherical coverage test cases are performed with black box approach using the FF probe. Performing these tests with the NF measurement probe would require the extensive black&white-box approach which is not deemed a feasible enhancement of the methodology..

-	The low UL power/high DL power EIRP/EIS test cases in the known FF BP direction are applicable to the black&white-box approach.

-	Whether a local search to determine the NF test direction and/or optimize EIRP/EIS is FFS.

-	EIRP/EIS can be approximated in the NF (min. range lengths for PC1 and PC3 are FFS)

-	TRP test cases at very close distances require offset compensation while range lengths beyond 32cm for PC3 do not necessarily require offset compensations. At those range lengths, the relaxations are minimized by up to 10dB.

In order to clarify the applicability for EIRP/EIS measurements and corresponding minimum range length, an extensive simulation campaign using Matlab was defined following the assumptions presented in Table 3-1.

Table 3-1: Simulation assumptions for CFF(D)NF

| Parameter | Value(s) / Assumptions | Notes |
| --- | --- | --- |
| Methodology | CFFDNF with black&white box approach | Phase centre of active antenna yielding Far Field Beam Peak direction is known |
| UE Antenna Array Configuration | PC3: 8x2 and 4x1 PC1: 12x12 |  |
| Beam Steering Assumptions | N/A | Not needed for CFFDNF or CFFNF as beam peak searches and spherical coverage measurements are based on FF probe |
| HPBW of Individual Array Element | 90o/90o | as suggested in [9] |
| Offsets of Active Array Panel | PC3 (8x2 and 4x1): 0 ≤ xoffset ≤ 12.5cm -12.5cm ≤ yoffset ≤ 12.5cm -12.5cm ≤ zoffset ≤ 12.5cm (The maximum radial offset cannot exceed 12.5cm) PC1 (12x12): 0 ≤ xoffset ≤ 10 cm -10cm ≤ yoffset ≤ 10cm -10cm ≤ zoffset ≤ 10cm (The maximum radial offset cannot exceed 10cm) | 500 offsets selected randomly with uniform distribution |
| Path Loss Correction | Compensation of antenna array offset | Path loss applied to the EIRP measurements is not referenced to the centre of QZ but to the phase centre of the active antenna array. |
| NF Measurement Direction | Determined theoretically from range length, FF BP direction, and array offsets | - |
| Probe antenna pattern/gain compensation | With compensation (uniform pattern assumed in simulations) | A wider beamwidth probe might be considered. |
| Tool Used for Simulations | Matlab, using the superposition approach outlined in [4] | CST was used to generate the realistic single element pattern as provided in the Appendix. |
| Range Lengths | 20cm, 25cm, 30cm, 35cm, 40cm, 45cm, 20m | - |
| Frequency | 28GHz | - |

For completeness, the figures of the single element antenna model used for the simulations and the reference radiation patterns are provided in the Appendix to this contribution.

The following figures show the histograms of the respective offsets in x, y, and z for the 500 simulations.

(a)						(b)

Figure 3-1: Distribution of 500 random offsets within 12.5cm in a single hemisphere for 8x2 array (a) Histogram of the offsets in x, y, and z; (b) 3D distribution of offsets

The results for are presented in Table 3-2:

Table 3-2: CFFDNF impact of range length on EIRP measurements (with offset correction).

| UE Antenna Array Configuration | Range Length [m] | Mean EIRP error [dB] | EIRP Std. Deviation [dB] |
| --- | --- | --- | --- |
| 4x1 | 0.20 | 0.034 | 0.015 |
|  | 0.25 | 0.016 | 0.005 |
|  | 0.30 | 0.010 | 0.002 |
|  | 0.35 | 0.006 | 0.003 |
|  | 0.40 | 0.003 | 0.001 |
|  | 0.45 | 0.002 | 0.000 |
|  | 20 | 0.000 | 0.000 |
| 8x2 | 0.20 | 0.391 | 0.174 |
|  | 0.25 | 0.188 | 0.058 |
|  | 0.30 | 0.113 | 0.026 |
|  | 0.35 | 0.075 | 0.016 |
|  | 0.40 | 0.054 | 0.008 |
|  | 0.45 | 0.041 | 0.006 |
|  | 20 | 0.000 | 0.000 |
| 12x12 | 0.20 | 2.697 | 0.832 |
|  | 0.25 | 1.450 | 0.333 |
|  | 0.30 | 0.913 | 0.166 |
|  | 0.35 | 0.627 | 0.097 |
|  | 0.40 | 0.460 | 0.061 |
|  | 0.45 | 0.351 | 0.040 |
|  | 20 | 0.000 | 0.000 |

Observation 2: preliminary assessment results for EIRP measurement error in [7] are consistent with the results in Table 3-2.

## Error due to declared antenna offset error

Under Black&White box approach conditions, where the active antenna phase center offset from center of the DUT is declared by the manufacturer, there might be errors introduced in the EIRP measurement due to an incorrect estimation of the beam peak and the corresponding distance correction for CFFDNF.

Table 3-3 shows the simulation results of 500 UE antenna offsets as described in section 3 including random offset error in x, y and z directions to simulate the error in the manufacturer declaration.

Table 3-3: Statistical results for the sensitivity of CFFDNF to declared antenna offset error under black&white box approach.

| UE Antenna Array Configuration | Range Length [m] | Error in declared antenna offset [mm] | Mean EIRP error [dB] | EIRP Std. Deviation [dB] |
| --- | --- | --- | --- | --- |
| 8x2 | 0.4 | 1 | 0.056 | 0.019 |
|  |  | 5 | 0.074 | 0.087 |
|  |  | 10 | 0.129 | 0.221 |

Given that it was agreed in [7] that black&white box approach can be based on manufacturer declaration as a baseline, a MU contributor is required to account for the error in the declared antenna offset.

Proposal 4: a MU contributor for the sensitivity of CFFDNF expansion approach to declared antenna offset error under black&white box approach is required.

# Impact of NF probe on DUT antenna

Depending on the test distance and type of DUT antenna, the coupling between them might change the Tx/Rx characteristics of the DUT. This effect can be analyzed by effects on the S11 seen from the DUT antenna when test probe is placed at different distances.

A set of simulations using CST 3D EM simulator were performed in order to analyze this effect using an FR2 in-band corrugated horn antenna as near field probe and a 8x2 array as UE antenna.

Figure 4-1: antenna NF coupling diagram between an FR2 corrugated horn and an 8x2 UE antenna array

Figure 4-2: 8x2 UE antenna array S11 coupling effect variation over distance

The plot shown in Figure 4-2 present the variation of the S11 seen at the input port of the UE antenna array when the NF probe is placed at different distances, but always in the axis where the UE antenna main beam is created.

The concrete minimum range length (i.e. distance from probe antenna to center of the QZ) proposed so far for CFFNF and CFFDNF, 20cm and 32cm respectively, correspond to an actual worst case minimum distance of 7.5cm and 19.5cm (i.e. range length - maximum displacement from center of QZ).

Looking at the data presented in Figure 4-2, CFFNF with 20cm range length corresponds to a 0.5dB worst case S11 degradation at the input port of the UE antenna, while CFFDNF with 32cm range length introduces no degradation.

Observation 3: CFFNF with 20cm range length corresponds to a 0.5dB worst case S11 degradation at the input port of the UE antenna, while CFFDNF with 32cm range length introduces no degradation.

It has to be noted that simulation results are a best case since the scattering around the UE and NF probe antennas are not considered. Therefore, real effect is expected to be worse.

# Conclusion

In this contribution we make the following observations and proposals:

Observation 1: inconsistent results are observed for asymptotic expansion approach when b2≠0.

Observation 2: preliminary assessment results for EIRP measurement error in [7] are consistent with the results in Table 3-2.

Observation 3: CFFNF with 20cm range length corresponds to a 0.5dB worst case S11 degradation at the input port of the UE antenna, while CFFDNF with 32cm range length introduces no degradation.

Proposal 1: a clear and detailed formulation of the CFFNF methodology based on the asymptotic expansion must be provided.

Proposal 2: a MU contributor for the sensitivity of CFFNF expansion approach to measurement error is required.

Proposal 3: a MU contributor for the sensitivity of CFFNF expansion approach to declared antenna offset error under black&white box approach is required.

Proposal 4: a MU contributor for the sensitivity of CFFDNF expansion approach to declared antenna offset error under black&white box approach is required.

# References

- RP-210633, “Revised SID: Study on enhanced test methods for FR2”, Apple Inc., vivo, 3GPP RAN #91-e, March 2021.

- R4-1900018, “LS on the testability of FR2 transmitter and reception tests”, TSG RAN WG5, 3GPP RAN4 #90, February 2019.

- 3GPP TS 38.101-2 v15.10.0

- R4-2106130, “On CFFNF and CFFDNF test methodologies for high DL power and low UL power test cases”, Keysight Technologies, 3GPP RAN4 #98-bis-e, April 2021.

- R4-2102616, “On Test methodology for high DL power and low UL power test cases”, Keysight Technologies, 3GPP RAN4 #98-e, February 2021.

- R4-2011218, “On Test methodology for high DL power and low UL power test cases”, Keysight Technologies, 3GPP RAN4 #97-e, November 2020.

- R4-2106127, “WF on agreements and remaining issues with FR2 test method enhancements”, Apple, 3GPP RAN4 #98-bis-e, April 2021.

- R4-2103985, “Draft TR38.884 Study on enhanced test methods for FR2 NR UEs v0.2.0”, Apple Inc., vivo, 3GPP RAN4 #98-e, February 2021.

- R5-206618, “PC1 Assumptions for CDF Curve for MOP EIRP Spherical Coverage”, Qualcomm Incorporated, 3GPP RAN5 #89-e, November 2020.

- R4-2016562, “Views on test methods for high DL power and low UL power TCs”, Rohde & Schwarz, 3GPP RAN4 #97-e, November 2020.

- R4-2102620, “NF based solutions and Enhancement of permitted methods”, Rohde & Schwarz, 3GPP RAN4 #98-e, February 2021.

- R4-2107187, “Analysis of NF based solutions“, Rohde & Schwarz, 3GPP RAN4 #98-bis-e, April 2021.

- R4-2111005, “On CFFNF and CFFDNF test methodologies for high DL power and low UL power test cases”, Keysight Technologies, 3GPP RAN4 #99-e, May 2021.

# Appendix – Informative figures

Table 7-1: Single element antenna model

| Antenna model |  |
| --- | --- |
| 3D radiation pattern |  |
| E-plane cut |  |
| H-plane cut |  |