| 3GPP TR 38.860 V0.2.0 (2021-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Study on extended 600MHz NR band; (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	9

4	Background	9

4.1	Options for study: B1	9

4.2	Options for study: B2	10

4.3	Options for study: BX (if required)	11

5	Regulatory landscape	12

5.1	General	12

5.2	ITU Region 3	14

6	Frequency band arrangements and regulatory background	15

6.1	Operating band and channel bandwidth	15

6.2	Adjacent 3GPP bands	16

6.3	Nearby non-3GPP Services	17

6.3.1	Introduction	17

6.3.2	Coexistence between 600 MHz and TV Services	17

6.3.3	Coexistence between 600 MHz Radio Astronomy Service (RAS)	17

6.4	Duplex filter implementation issues	17

6.4.1	Option B1	17

6.4.2	Option B2	17

6.4.3	Option B2a	17

7	List of band specific issues for APT600 MHz frequency band	18

8	Study of NR specific issues	18

8.1	UE aspect issues	18

8.1.1	UE transmitter requirements	18

8.1.2	UE receiver requirements	18

8.2	BS aspect issues	18

X	Examples for styles	18

X.1	Heading styles	18

X.2	Other common styles	19

B.1	Heading levels in an annex	20

Annex <B>: <Informative annex title for a Technical Report>	21

Annex <X> (informative): Change history	22

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This clause is mandatory; do not alter the text in any way other than to choose between "Specification" and "Report".

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

During the RAN #90e meeting a new study item was agreed [2] to study various band configurations for the 600 MHz band in ITU R Region 3. A liaison statement from the APT Wireless Group (AWG) [3] was also received that had two filter combinations- namely B1 and B2 as candidates for study. This report documents the results of this study.

1	Scope

The present document is a technical report for the study item of APT 600 MHz NR FDD band.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]                        RP-202924, “New SID on extended 600 MHz NR band, “Spark NZ, CBN.

[3]                        R4-2016598, “Frequency arrangements for IMT in the band 470- 703 MHz’, APT Wireless Group…

[4]                       TR 36.820 LTE for 700 MHz digital dividend

[5]                        TR 36.755 US 600 MHz Band for LTE

[6]                         R4- 2104891 TP on band plan for 600 MHz

[10]	3GPP TS 38.101: " 5G; NR; User Equipment (UE) radio transmission and reception

[11]	3GPP TS 38.104: " 5G; NR; Base Station (BS) radio transmission and reception

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

ACLR	Adjacent Channel Leakage Ratio

BS	Base Station

DL	Downlink

DTV	Digital Television

EARFCN 	E-UTRA Absolute Radio Frequency Channel Number

ETSI	European Telecommunications Standards Institute

FCC	Federal Communications Commission

FDD	Frequency Division Duplex

MOP	Maximum Output Power

MR	Medium Range

MSR	Multi-Standard Radio

PA 	Power Amplifier

RAS	Radio Astronomy Service

REFSENS	Reference Sensitivity

RF	Radio Frequency

TX	Transmitter

UE	User Equipment

UHF	Ultra High Frequency

UL	Uplink

# 4	Background

During the RAN #90e meeting a new study item was agreed [2] to study various band configurations for the 600 MHz band in ITU R Region 3.A liaison statement from the APT Wireless Group (AWG) [3] was also received that had two filter combinations- namely B1 and B2 as candidates for study.

The objectives for the extended 600 MHz study item are listed below:

- Regulatory study of the frequency range around 600 MHz in Region 3

- Co -existence study for the frequency range of 612-652/663-703MHz such as with DTV (if needed)

- Study potential frequency arrangements and conclude the possible implications such as insertion loss, transmitter and receiver characteristics for both the BS and UE, system limitations such as channel bandwidths etc.) of different duplex filter implementations.

- Consider options B1 and B2 from AWG LS but other options are not precluded.

- Answer the request from AWG regarding technical feasibility of option B1 and B2 respectively. Further options are not precluded and may be included in LS to AWG.

NOTE: Since regulatory study of frequency range around 600 MHz is for Region 3, the SI outcome will not impact any requirements defined for US 600 MHz band.

It has also been agreed that the above studies will be completed in time for the AWG meeting to be held in Sept 2021.

The above studies have been on going in RAN4 meetings. This report documents the results of the studies.

3GPP band 28 [4] and US 600 MHz bands are  shown below in Fig (1):

Fig (1); US 600 MHz band and 3GPP band 28

By examining the US 600 MHz band [5] it is noted that the band extends only up to 698 MHz leaving the range 698- 703 MHz un useable

The objective of the SI is to study spectrum options that will enable usage of  698- 703 MHz

# There are three duplex filter  options considered in this report.

## 4.1	Options for study: B1

## There are two cases, a single filter and  an overlapping duplexer  case as shown below

Fig (2); Filter options B1

Editors Note: The above dual duplexer approach for B1 needs further study as the duplexers have different duplex spacing Other duplexer arrangements may be possible.and should be studied

## 4.2	Options for study: B2

## There are three cases, all with overlapping duplexers but in all cases one of the duplexers is that of US 600 MHz band

## Fig (3) : Filter options B2

## 4.3	Options for study: BX (if required)

Fig(4): Filter option B2a

## The above option consists of two bands, band n 71 and a new band labelled as band nX.

# 5	Regulatory landscape

## 5.1	General

The 470 - 694 MHz frequency range is allocated to the broadcasting service and mobile service on a co-primary basis in Region 3 [1]. The frequency band 470 - 698 MHz, or parts thereof, was identified by WRC-15 in 7 countries in Region 3 through footnote No. 5.296A for use by those administrations as listed wishing to implement terrestrial IMT systems. In addition, there was interest from other significant markets to do the same. Elsewhere, USA, Mexico and several other countries in Region 2 also identified this band for IMT through footnotes 5.295 and 5.308A. It is noted that resolves 2 of Resolution 224 (Rev.WRC-19) encourages the following:

encourages administrations to take into account results of the existing relevant ITU Radiocommunication Sector studies, when implementing IMT applications/systems in the frequency bands

- 694-862 MHz in Region 1,

- in the frequency band 470-806 MHz in Region 2,

- in the frequency band 790-862 MHz in Region 3,

- in the frequency band 470-698 MHz, or portions thereof, for those administrations mentioned in No. 5.296A, and

- in the frequency band 698-790 MHz, or portions thereof, for those administrations mentioned in No. 5.313A.

The regulatory landscape overview for all regions in frequency range 470 – 694 MHz is provided in table x.1-1, based on ITU radio regulations [3].

Table x.1-1: ITU-R frequency allocation to services [3]

NOTE: 	Since regulatory study of frequency range around 600 MHz is for Region 3, the outcomes of this study item will not impact any requirements defined for US 600 MHz band, i.e. band n71.

Below, ITU RR footnotes referred in this clause are listed:

5.149		In making assignments to stations of other services to which the bands: 13 360-13 410 kHz, 25 550-25 670 kHz, 37.5-38.25 MHz, 73-74.6 MHz in Regions 1 and 3, 150.05-153 MHz in Region 1, 322-328.6 MHz, 406.1-410 MHz, 608-614 MHz in Regions 1 and 3, 1 330-1 400 MHz, 1 610.6-1 613.8 MHz, 1 660-1 670 MHz, 1 718.8-1 722.2 MHz, 2 655-2 690 MHz, 3 260-3 267 MHz, 3 332-3 339 MHz, 3 345.8-3 352.5 MHz, 4 825-4 835 MHz, 4 950-4 990 MHz, 4 990-5 000 MHz, 6 650-6 675.2 MHz, 10.6-10.68 GHz, 14.47-14.5 GHz, 22.01-22.21 GHz, 22.21-22.5 GHz, 22.81-22.86 GHz, 23.07-23.12 GHz, 31.2-31.3 GHz, 31.5-31.8 GHz in Regions 1 and 3, 36.43-36.5 GHz, 42.5-43.5 GHz, 48.94-49.04 GHz, 76-86 GHz, 92-94 GHz, 94.1-100 GHz, 102-109.5 GHz, 111.8-114.25 GHz, 128.33-128.59 GHz, 129.23-129.49 GHz, 130-134 GHz, 136-148.5 GHz, 151.5-158.5 GHz, 168.59-168.93 GHz, 171.11-171.45 GHz, 172.31-172.65 GHz, 173.52-173.85 GHz, 195.75-196.15 GHz, 209-226 GHz, 241-250 GHz, 252-275 GHz are allocated, administrations are urged to take all practicable steps to protect the radio astronomy service from harmful interference. Emissions from spaceborne or airborne stations can be particularly serious sources of interference to the radio astronomy service (see Nos. 4.5 and 4.6 and Article 29). (WRC-07)

5.295		In the Bahamas, Barbados, Canada, the United States and Mexico, the frequency band 470-608 MHz, or

portions thereof, is identified for International Mobile Telecommunications (IMT) – see Resolution 224 (Rev.WRC-19). This identification does not preclude the use of these frequency bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. Mobile service stations of the IMT system within the frequency band are subject to agreement obtained under No. 9.21 and shall not cause harmful interference to, or claim protection from, the broadcasting service of neighbouring countries. Nos. 5.43 and 5.43A apply. (WRC-19)

5.296A	In Micronesia, the Solomon Islands, Tuvalu and Vanuatu, the frequency band 470-698 MHz, or portions thereof, and in Bangladesh, Maldives and New Zealand, the frequency band 610-698 MHz, or portions thereof, are identified for use by these administrations wishing to implement International Mobile Telecommunications (IMT) – see Resolution 224 (Rev.WRC-19). This identification does not preclude the use of these frequency bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. The mobile allocation in this frequency band shall not be used for IMT systems unless subject to agreement obtained under No. 9.21 and shall not cause harmful interference to, or claim protection from, the broadcasting service of neighbouring countries. Nos. 5.43 and 5.43A apply. (WRC-19)

5.305	Additional allocation: in China, the band 606-614 MHz is also allocated to the radio astronomy service on a primary basis.

5.306	Additional allocation: in Region 1, except in the African Broadcasting Area (see Nos. 5.10 to 5.13), and

in Region 3, the band 608-614 MHz is also allocated to the radio astronomy service on a secondary basis.

5.307		Additional allocation: in India, the band 608-614 MHz is also allocated to the radio astronomy service on

a primary basis.

5.308A 	In the Bahamas, Barbados, Belize, Canada, Colombia, the United States, Guatemala and Mexico, the

frequency band 614-698 MHz, or portions thereof, is identified for International Mobile Telecommunications (IMT) – see Resolution 224 (Rev.WRC-19). This identification does not preclude the use of these frequency bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. Mobile service stations of the IMT system within the frequency band are subject to agreement obtained under No. 9.21 and shall not cause harmful interference to, or claim protection from, the broadcasting service of neighbouring countries. Nos. 5.43 and 5.43A apply. (WRC-19)

5.313A	The frequency band, or portions of the frequency band 698-790 MHz, in Australia, Bangladesh, Brunei Darussalam, Cambodia, China, Korea (Rep. of), Fiji, India, Indonesia, Japan, Kiribati, Lao P.D.R., Malaysia, Myanmar (Union of), New Zealand, Pakistan, Papua New Guinea, the Philippines, the Dem. People’s Rep. of Korea, Solomon Islands, Samoa, Singapore, Thailand, Tonga, Tuvalu, Vanuatu and Viet Nam, are identified for use by these administrations wishing to implement International Mobile Telecommunications (IMT). This identification does not preclude the use of these frequency bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. (WRC-19)

## 5.2	ITU Region 3

Editor’s note: refer to the APT LS; any APT specific regulatory matters

ITU Region 3 specific regulatory aspects for the frequency range 470 – 694 MHz are provided in this clause.

Referring to the APT LS, AWG-26 agreed to undertake the study to revise the APT/AWG/REP-79 [4] to develop frequency arrangements in the band 470-703 MHz for APT Members that wish to implement both the APT700 and a 600 MHz frequency arrangements that is optimal for APT Members.

According to Radio Regulations, in Region 3, the 610 – 890 MHz band is allocated to Fixed Service, Mobile Service and Broadcasting Service on a primary basis. Additionally, 606 – 614 MHz band is allocated for Radio Astronomy Service in China (i.e. footnote 5.305) and band 608 - 614 MHz is allocated for Radio Astronomy Service in India (i.e. footnote 5.307). Footnote 5.149 urges administrator to protect the radio astronomy service from harmful interference.

Digital television has various channel allocations depending on country allocations in Region 3. IMT and DTV coexistence may require additional consideration on a per-country basis, leading to potential need for a guard bands.

No specific regional regulatory requirements on unwanted emissions were identified for the IMT operation in frequency range 470 – 694 MHz in Region 3, so far.

# 6	Frequency band arrangements and regulatory background

## 6.1	Operating band and channel bandwidth

The band plan for the extended 600 MHz NR band are shown below in Tables 6.1-1 to 6.1-7.  Options B1 and B2 have split duplexers, partially overlapping, but part of the same band. Option B2a has two duplexers, but one is band n 71 and the other is a new band.

The Tx-Rx is "reverse-duplex"; in other words, the downlink frequency band is below the duplex gap while the uplink frequency band is above the duplex gap. This arrangement is opposite to conventional notation; however, for this band, it provides the benefit of aligning the uplink band adjacent to 3GPP band 28  thereby minimizing interference conditions at the 703  MHz boundary.

Table 6.1-1: NR operating band ( option B1)

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| TBD | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

The above could be implemented as a single or overlapping duplexers. In case of overlapping duplexers, the following option is studied:

Table 6.1-2: Duplexer arrangements ( option B1)

| Duplexer type | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| Full | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |
| Duplex1 Duplex 2 | TBD TBD |  |  | TBD TBD |  |  | FDD |
|  |  |  |  |  |  |  | FDD |
| Editor’s note: Duplexer arrangements in this case needs to be confirmed |  |  |  |  |  |  |  |

Table 6.1-3.: NR operating band (option B2)

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| TBD | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

The above could be implemented as a single or overlapping duplexers. In case of overlapping/split  duplexers, three sub options are studied;

Table 6.1-4: Duplexer arrangements ( option B2 35 + 35)

| Duplexer type | Uplink (UL) operating band BS receive UE transmit | Downlink (DL) operating band BS transmit UE receive | Duplex Mode |
| --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| Duplex 1 Duplex 2 | 663 MHz – 698 MHz 668 MHz – 703 MHz | 617MHz – 652 MHz 622MHz – 657 MHz | FDD |
|  |  |  | FDD |
| NOTE: Both duplexers will be part of the same band |  |  |  |

Table 6.1-5: Duplexer arrangements ( option B2 35+30)

| Duplexer type | Uplink (UL) operating band BS receive UE transmit | Downlink (DL) operating band BS transmit UE receive | Duplex Mode |
| --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| Duplex1 Duplex 2 | 663 MHz – 698 MHz 673 MHz – 703 MHz | 617MHz – 652 MHz 627MHz – 657 MHz | FDD |
|  |  |  | FDD |
| NOTE: Both duplexers will be part of the same band |  |  |  |

Table 6.1-6: Duplexer arrangements ( option B2 35+25)

| Duplexer type | Uplink (UL) operating band BS receive UE transmit | Downlink (DL) operating band BS transmit UE receive | Duplex Mode |
| --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| Duplex 1 Duplex 2 | 663 MHz – 698 MHz 678 MHz – 703 MHz | 617MHz – 652 MHz 632MHz – 657 MHz | FDD |
|  |  |  | FDD |
| NOTE: Both duplexers will be part of the same band |  |  |  |

Table 6.1-7: NR operating bands (option B2a) [6]

| Operating Bands | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| nX | 673 MHz | – | 703 MHz | 627 MHz | – | 657 MHz | FDD |
| NOTE : These are two bands, band n 71 plus band nX. A UE that complies with NR requirements in this specification shall also comply with NR Band 71 minimum requirements |  |  |  |  |  |  |  |

Supported bandwidths are shown in Table 6.1-8

Editors Note: support for asymmetric channel bandwidths in DL and UL is for further study

Table 6.1-8: Supported channel bandwidths

| NR Band |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TBD |  |  |  |  |  |  |
| Editor’s note : This table needs completion |  |  |  |  |  |  |

Tx-Rx separation is shown in Table 6.1-9 and is dependent upon the filter options .

Table 6.1-9: Default UE TX-RX frequency separation

| Filter Option | TX – RX carrier centre frequency separation |
| --- | --- |
| B1 ( Full band) | -51 MHz |
| B1 ( duplexer 1) | TBD |
| B1 (duplexer 2) | TBD |
| B2 (duplexer 1) | 46 MHz |
| B2 ( duplexer 2) | 46 MHz |
| B2a | 46 MHz for both bands |

Editor’s Note; The above table needs completion after filter options are finalised

Editor’s Note: In case of B1 overlapping duplexers , the duplex arrangement needs to be confirmed .

## 6.2	Adjacent 3GPP bands

The nearby 3GPP bands for Region 3 are shown below in Fig (5) .

The only adjacent band is 3GPP band 28

660

680

700

720

740

760

780

800

820

840

MHz

FDD UL

FDD DL

703

748

758

803

B5 UL

824

849

B26 UL

814

849

B18

845

815

B19

830

Fig (5) 3GPP bands adjacent to extended 600 MHz band

| Band | Frequency separation | Comments/Notes |
| --- | --- | --- |
| Band 5 | 21 MHz | Used in various places in the APAC region |
| Band 18 | 12 MHz | Used in Japan |
| Band 19 | 27 MHz | Used in Japan |
| Band 26 | 11 MHz | Band planned for global use. |

## 6.3	Nearby non-3GPP Services

### 6.3.1	Introduction

Existing primary operations near the 600 MHz Band include Television services and Radio Astronomy Service (RAS), which are entitled to interference protection from 600 MHz broadband wireless services.  The latter is also protected by a regulatory footnote in some countries. While, unlicensed white space device and wireless microphone operations are allowed in the duplex gap, guard band and service band where 600 MHz licensees have not commenced operations; TV channel in Region 3 use either 6 MHz or 7 MHz or 8 MHz as shown below in Fig (6):

Fig(6): [600 MHz band plans  and adjacent frequency use for digital TV]

Coexistence issues and possible impacts on 600 MHz band RF requirements are analysed below.

### 6.3.2	Coexistence between 600 MHz and TV Services

### 6.3.3	Coexistence between 600 MHz Radio Astronomy Service (RAS)

## 6.4	Duplex filter implementation issues

### 6.4.1	Option B1

Editor’s note : To be completed after filter simulations are completed

### 6.4.2	Option B2

Editor’s note : To be completed after filter simulations are completed

## 6.4.3	Option B2a

Editor’s note : To be completed after filter simulations are completed

# 7	List of band specific issues for APT600 MHz frequency band

Below is the list of issues that are studied for the APT 600 MHz Band xx

Editor’s Note : To be completed

# 8	Study of NR specific issues

## 8.1	UE aspect issues

### 8.1.1	UE transmitter requirements

### 8.1.2	UE receiver requirements

## 8.2	BS aspect issues

Editor’s Note : all of the above needs to be completed.

# X	Examples for styles

The main text of the document should start here, after the above clauses have been added.

The following styles and editing techniques are aimed to help in the formatting of the document using the 3GPP Template: 3GPP_70.dot, available from the 3GPP FTP site (ftp://ftp.3gpp.org/Information).

## X.1	Heading styles

Heading styles are included in the 3GPP TS Template and are used as follows:

Do not use any built-in automatic numbering for 3GPP documents. Although this is sometimes useful in the early drafting stages of a document, once the document has been placed under change control, the clause numbering needs to be fixed in order to keep cross-reference consistency as the 3GPP specification set develops.

Heading 1:	Used for Main clauses (1, 2, 3, etc.). Also used for Annex clauses (A.1, A.2, etc.).

Heading 2:	Used for Main clauses (4.1, 4.2, 5.1, 5.2, etc.). Also used for Annex clauses (A.1.1, A.1.2, etc.).

Heading 3:	Used for 2nd level clauses (4.1.1, 4.1.2, 5.1.1, 5.1.2, etc.). Also used for Annex clauses (A.2.1.1, A.2.1.2, etc.).

Heading 4 & 5:	Used for 3rd and 4th level clauses and Annex clauses.

Heading 6 & 7:	Not used, instead use style "H6" so that the title appears in the document, but does not appear in the Table of Contents.

Heading 8:	Used for Main Annex titles in Specifications (3GPP TS) (e.g. Annex A (normative): ).

Heading 9:	Used for Main Annex titles in Reports (3GPP TR) (e.g. Annex A: ).

## X.2	Other common styles

Normal:	Used for main document text.

NO:	Used for Notes in the text (Allows Tab and Indent). See example below.

NW:	Same as NO, but Without line space after. Used when there are many notes in sequence.

NOTE 1:	This is an example of a note formatted in style NW. The style is designed to allow space for note numbering and line wrap with a hanging indent. There is no line space after.

NOTE 2:	This is an example of a note formatted in style NO. The style is designed to allow space for note numbering and line wrap with a hanging indent. There is a line space after.

Bullet styles:	The following bullet styles are provided.

B1:	Bullet level 1 for main bullet points.

B2:	Bullet level 2 for sub bullets.

B3-B5:	for further sub bullets.

NOTE:	Bullets are usually formatted manually, using a hyphen ( - ) or alphanumeric identifiers: a), b), or 1), 2) etc. followed by a tab character. Automatic bullet features should not be used as they may be lost if template styles are re-applied later.

Table styles:	TAH, TAL, TAC, TAR, TAN, for TAble Headers, Left justified, Centred, Right justified and Notes in tables: Style TH is used for the Table Heading (title or caption). See example below.

Table 1: Example of table styles

| Col 1 Header (TAH) | Col 2 Header (TAH) | Col 3 Header (TAH) |
| --- | --- | --- |
| Left Justified (TAL) | Centred (TAC) | Right Justified (TAR) |
| NOTE: A special style is provided for notes within a table (TAN). |  |  |

Warning: The default setting for table cells is to disallow rows to break at a page boundary. If you include tables with very long cells, likely to extend beyond the bottom of the page (bearing in mind the table header and the page header and footers, and the margin settings), then you must enable that row's "Allow row to break across pages" setting.

Figure styles:	Figures and graphics are formatted with style "TH" which keeps the figure with the following paragraph, usually the figure title. Figure Titles (captions) are formatted with style "TF". See example below.

Figure 1: Example figure layout. To remove "float over text" select the graphic and "Format Object ..." - De-select "float over text" in the Position Tab
Annex <B> (informative):
<Informative annex for a Technical Specification>

Informative annexes shall not contain requirements for the implementation of the Technical Specification.

# B.1	Heading levels in an annex

Heading levels within an annex are used as in the main document, but for Heading level selection, the "A.", "B.", etc. are ignored. e.g. B.1.2 is formatted using Heading 2 style.

Annex <B>:
<Informative annex title for a Technical Report>

Informative annexes in Technical Reports do not use "(informative") in the title, since all annexes in TRs are informative. Use style "Heading 9" in TRs.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-02 | RAN4#98-e | R4-2103265 |  |  |  | Report skeleton R4-2103265, “Blank TR for extended 600MHz NR band” | 0.0.0 |
| 2021-04 | RAN4#98-bis-e | R4-21xxxxx |  |  |  | Conversion to the latest version of the TR template and TR number added. Agreed Text Proposals in RAN4#98-e: R4-2103267, "Regulatory aspects for the 600MHz range in APT region” | 0.2.0 |