3GPP TSG-RAN WG4 Meeting # 99-e		                                                    R4-2109036

Electronic Meeting, May. 19-27, 2021

Title:	LS on synchronous operation between Uu and SL in TDD band n79

Response to:

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	RAN4

To:	RAN1

Cc:

Contact person:

Name:			Yuan Gao

E-mail Address	gaoyuan@catt.cn

Attachments:	None

1. Overall Description:

Partially used SL with Uu in TDD band n79 irrespective of TDM or FDM is being discussed for Rel-17 SL enhancement in RAN4.

In Rel-16 NR V2X, SL transmission timing is aligned with DL timing of Uu based on RAN1 agreements. The same SL transmission timing if applied to Rel-17 SL in the scenario of partially used SL with Uu in TDD band n79 may give rise to interference problem between SL and Uu.

Separately, Uu uses network as synchronization reference source, while SL can use GNSS, network and SyncRef UE as synchronization reference source. In Rel-16, the priority of the synchronization reference source is (pre-)configurable and access to resources can be restricted based on synchronization reference source by pre-configuration. Out of coverage UEs may use a synchronization reference source that is different from the network in certain (pre-)configurations.

Therefore, RAN4 respectfully request RAN1 to clarify the following questions regarding partially used SL with Uu in TDD band n79:

Question 1: Considering the Rel-16 agreement by RAN1 to use DL timing, is it feasible from RAN1 perspective to change specifications such that SL transmission timing uses UL timing when synchronized to a network?

Question 2: Considering the Rel-16 agreement by RAN1 to use (pre-)configurable priorities for sidelink synchronization reference sources and the ability to restrict access to resources based on synchronization reference source by pre-configuration, is it feasible from RAN1 perspective to change specifications to specify only the network as synchronization reference source of SL in case of partially used SL with Uu in TDD band n79?

2. Actions:

To RAN WG1:

RAN4 respectfully request RAN1 to clarify the above questions regarding partially used SL with Uu in TDD band n79.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #100-e            Aug. 16 – Aug. 27, 2021                           Online

TSG-RAN WG4 Meeting #100-e            Nov. 01 – Nov. 12, 2021                           Online