3GPP TSG-RAN WG4 Meeting #99-e		R4-2109202

Electronic, 19th – 27th May, 2021

Agenda item:	5.1.1.2.1

Source:	Intel Corporation

Title:	Simulation results for multi-TRP Tx schemes performance requirements introduction

Document for:	Discussion

# Introduction

In RAN4 #98e meeting performance requirements for multi-TRP Tx schemes were defined under square brackets. For some test cases there is a big span among submitted results. In this paper we provide updated results from our side for all defined test cases. Corresponding simulation assumptions can be found in [1]

# Discussion

## Simulation results for multi-DCI based multi-TRP Tx scheme

In Figure 1 simulation results for agreed multi-DCI based multi-TRP Tx scenarios are provided for FDD and TDD duplex modes. The summary of demodulation performance at 70% at max achievable throughput is provided in Table 1 for ideal and impairment results as well.

| Figure 1. Demodulation performance of multi-DCI based Multi-TRP Tx scheme |  |
| --- | --- |

Table 1. Demodulation performance at 70% @max achievable throughput for multi-DCI based Tx scheme

| Duplex mode | Antenna configuration | Ideal results, dB | Impairment results, dB |
| --- | --- | --- | --- |
| FDD | 2x2 | 17.1 | 19.6 |
|  | 2x4 | 11.1 | 13.6 |
| TDD | 2x2 | 17 | 19.5 |
|  | 2x4 | 11 | 13.5 |

## Simulation results for single-DCI based multi-TRP SDM Tx scheme

In Figures 2-3 simulation results for single-DCI based SDM multi-TRP Tx scenarios are provided for FDD and TDD duplex modes for test case 1a and 2b. The summary of demodulation performance at 70% at max achievable throughput is provided in Table 2 for ideal and impairment results as well.

| Test case 1a: Frequency offset + Negative Time offset |  |
| --- | --- |
| Figure 2. Demodulation performance for test case 1a |  |

| Test case 1b: Positive Time offset |  |
| --- | --- |
| Figure 3. Demodulation performance for test case 1b |  |

Table 2. Demodulation performance at 70% @max achievable throughput for SDM Tx scheme

|  | Duplex mode | Antenna configuration | Ideal results, dB | Impairment results, dB |
| --- | --- | --- | --- | --- |
| Test case 1a | FDD | 2x2 | 17.9 | 20.4 |
|  |  | 2x4 | 10.8 | 13.3 |
|  | TDD | 2x2 | 17.3 | 19.8 |
|  |  | 2x4 | 10.7 | 13.2 |
| Test case 1b | FDD | 2x2 | 17.9 | 20,.4 |
|  |  | 2x4 | 10.7 | 13.2 |
|  | TDD | 2x2 | 17.4 | 19.9 |
|  |  | 2x4 | 10.7 | 13.2 |

## Simulation results for single-DCI based multi-TRP Repetition FDM A Tx scheme

In Figure 3 simulation results for the single-DCI based repetition FDM scheme A are provided. Demodulation performance summary is provided in Table 3 for ideal and impairment results as well.

| FDM scheme A |  |
| --- | --- |
| Figure 3. Demodulation performance for FDM scheme A |  |

Table 3. Demodulation performance at 70% @max achievable throughput for FDM scheme A

| Duplex mode | Antenna configuration | Ideal results, dB | Impairment results, dB |
| --- | --- | --- | --- |
| FDD | 2x2 | 14.2 | 16.2 |
|  | 2x4 | 7.6 | 9.6 |
| TDD | 2x2 | 14.4 | 16.4 |
|  | 2x4 | 7.5 | 9.5 |

## Simulation results for single-DCI based multi-TRP Repetition Inter-slot TDM Tx scheme

In Figure 4 simulation results for the single-DCI based repetition Inter-slot TDM scheme are provided. Demodulation performance summary is provided in Table 4 for ideal and impairment results as well.

| Inter-slot TDM |  |
| --- | --- |
| Figure 4. Demodulation performance for FDM scheme A |  |

Table 4. Demodulation performance at 70% @max achievable throughput for Inter-slot TDM scheme

| Duplex mode | Antenna configuration | Ideal results, dB | Impairment results, dB |
| --- | --- | --- | --- |
| FDD | 2x2 | -0.2 | 1.8 |
|  | 2x4 | -3.4 | -1.4 |
| TDD | 2x2 | 0.3 | 2.3 |
|  | 2x4 | -2.8 | -0.8 |

# Conclusion

In this paper we have provided simulation results to define demodulation performance requirements for multi-TRP Tx schemes.

# References

- R4-2017530 “Simulation assumption for PDSCH requirement with single-DCI and Multi-DCI transmission schemes”, Ericsson, Intel, RAN4 #97-e, November 2020