3GPP TSG-RAN4 Meeting #99-e	R4-2109326

# Electronic Meeting, May 19-27, 2021

Agenda item:	4.1.10

Source:	Apple

Title:   	Further discussion on testing of A-GNSS Sensitivity requirements in NR and LTE

Document for:	Discussion

# Introduction

There is an LS from RAN5 [1] asking RAN4 to take the following actions:

ACTION: 	RAN5 respectfully asks RAN 4 for guidance on the LTE and NR frequency bands, and band combinations, impacting the A-GNSS Sensitivity requirements in LTE and NR, and in particular in EN-DC, taking into account possible intermodulation and other interference mechanisms that may affect the GNSS bands.

In the case that this action is expected to take some time to complete, RAN 5 would greatly appreciate one or more status reports on the progress.

In RAN4#98-e, we provided a contribution [2] focusing on the following interference mechanisms for the RNSS band 1559-1610MHz:

- Harmonic interference

- Intermodulation interference up to IMD5

- Cross band isolation

- RX harmonic mixing

After discussions, there was a WF agreed [3]. In this contribution, we provide further views on the options listed in the WF.

# Discussions

  - LTE and NR bands for testing

The WF [3] lists the following options:

- There are three options discussed:

  - Option 1: Repeat the test in all UE supported bands (Spirent)

  - Option 2: Repeat the test in a subset of UE supported bands (Apple, Xiaomi, Qualcomm’s 2nd preference)

  - Option 3: Perform the test in any bearer band supported by the UE (i.e., only 1 (any) band is enough) (Qualcomm’s 1st preference).

In our understanding, the purpose of RAN5 LS is to ask RAN4 to conduct technical analysis and provide a basis for deciding how to reduce unnecessary tests while ensuring good test coverage, meaning tests should be duly conducted for those bands that may cause interference issues. Therefore, we believe option 2 is a proper choice. As for the subset of bands, we can refer to the analysis in [2], where the possible RF interference mechanisms include harmonic interference, cross band isolation, and RX harmonic mixing are analyzed.

Proposal 1: For LTE bands operating in SA single carrier modes, only four bands, i.e., 13, 14, 24, 44 may have interference to A-GNSS operating in the RNSS band 1559-1610MHz and should be tested.

Proposal 2: For NR bands operating in SA single carrier modes, only five bands, i.e., n13, n14, n24, n79 and n96 may have interference to A-GNSS operating in the RNSS band 1559-1610MHz and should be tested.

If RAN4 wants to further reduce the number of bands for testing, we are ok with option 3 too.

  - EN-DC combinations for testing

The follow agreement is reached in the WF:

  - It is agreed to focus the testing on “high-risk UL band combinations” that generate 2nd or 3rd IMDs falling into the GNSS bands.

As such, the following band combinations listed in Table 1 should be tested

Table 1: Interfering EN-DC combinations that have 2nd or 3rd IMDs

| EN-DC combinations | IMD interferences |
| --- | --- |
| DC_1_n3 | 3rd IMD |
| DC_1_n40 | 3rd IMD |
| DC_1_n77 | 2nd, 4th, 5th IMD |
| DC_1_n78 | 2nd, 5th IMD |
| DC_2_n66 | 3rd, 5th IMD |
| DC_2_n78 | 2nd, 5th IMD |
| DC_3_n1 | 3rd IMD |
| DC_3_n34 | 3rd IMD |
| DC_3_n77 | 2nd, 4th, 5th IMD |
| DC_3_n78 | 2nd, 4th, 5th IMD |
| DC_3_n79 | 3rd IMD |
| DC_4_n78 | 2nd, 4th, 5th IMD |
| DC_5_n12 | 2nd IMD |
| DC_5_n40 | 2nd IMD |
| DC_5_n78 | 3rd IMD |
| DC_7_n78 | 3rd IMD |
| DC_7_n28 | 2nd IMD |
| DC_7_n40 | 2nd IMD |
| DC_7_n77 | 3rd, 4th IMD |
| DC_7_n78 | 3rd IMD |
| DC_8_n28 | 2nd IMD |
| DC_8_n41 | 2nd IMD |
| DC_8_n77 | 3rd, 4th IMD |
| DC_8_n78 | 3rd IMD |
| DC_11_n79 | 3rd IMD |
| DC_12_n5 | 2nd IMD |
| DC_13_n5 | 2nd IMD |
| DC_19_n77 | 3rd, 4th IMD |
| DC_19_n78 | 3rd IMD |
| DC_20_n28 | 2nd IMD |
| DC_20_n77 | 3rd, 4th IMD |
| DC_20_n78 | 3rd IMD |
| DC_21_n79 | 3rd IMD |
| DC_25_n41 | 3rd, 4th IMD |
| DC_26_n77 | 3rd, 4th IMD |
| DC_26_n78 | 3rd IMD |
| DC_28_n5 | 2nd IMD |
| DC_28_n8 | 2nd IMD |
| DC_28_n40 | 2nd IMD |
| DC_38_n78 | 3rd, 4th IMD |
| DC_39_n78 | 2nd, 5th IMD |
| DC_40_n1 | 3rd IMD |
| DC_40_n77 | 2nd IMD |
| DC_41_n77 | 2nd, 3rd, 4th IMD |
| DC_41_n78 | 3rd, 4th IMD |
| DC_42_n79 | 2nd, 4th, 5th IMD |
| DC_46_n78 | 2nd, 3rd, 5th IMD |
| DC_66_n2 | 3rd, 5th IMD |
| DC_66_n25 | 3rd, 5th IMD |
| DC_66_n78 | 2nd, 4th, 5th IMD |

.

  - GNSS bands to be considered

The WF considers the following options when it comes to the GNSS band(s) to be considered in RAN4 analysis:

  - Option 1:

    - 1559-1610MHz

    - 1164-1300MHz

  - Option 2:

    - 1559-1610MHz

In TS 38.171, clause 5 A-GNSS minimum performance requirements (UE supports A-GPS L1 C/A only) concerns 1559-1610MHz band only, and clause 6 A-GNSS minimum performance requirements (UE supports other or additional GNSSs) concerns both 1559-1610MHz band and 1164-1300MHz band.

Also, during RAN4#98-e, it was commented that GNSS acquisition is typically done in L1 band, which falls into the band 1559-1610MHz. We also share a similar view.

Considering the above, it seems reasonable to focus on 1559-1610MHz. We therefore propose:

Proposal 3: 1559-1610MHz is considered in RAN4 analysis when identifying LTE/NR bands or high-risk UL band combinations for testing.

  - Maintenance of the list of bands or band combinations

Regarding this aspect, the WF says

- As RAN4 continues to introduce new bands or new EN-DC band combinations, the list of bands or band combinations needs to updated as well:

  - Option 1: the list is documented in TS 38.171, i.e., maintained by RAN4

  - Option 2: the list is documented in TS 37.571-1, i.e., maintained by RAN5

We don’t have a strong preference, but since the tests are specified in RAN5, it seems fit to ask RAN5 to maintain such a list, as long as the list of bands or band combinations are based on RAN4 agreement.

Proposal 4: The list based on RAN4 agreement is documented in TS 37.571-1, i.e., maintained by RAN5.

Finally, according to the agreed plan in the WF, RAN4 should send a reply LS to RAN5 with the list of bands and band combinations at this meeting.

Proposal 5: RAN4 sends a reply LS to RAN5 with the agreed list of bands and band combinations at this meeting.

# Conclusion

In this contribution, we continued the discussion based on the agreed WF. In particular, the following proposals are made:

Proposal 1: For LTE bands operating in SA single carrier modes, only four bands, i.e., 13, 14, 24, 44 may have interference to A-GNSS operating in the RNSS band 1559-1610MHz and should be tested.

Proposal 2: For NR bands operating in SA single carrier modes, only five bands, i.e., n13, n14, n24, n79 and n96 may have interference to A-GNSS operating in the RNSS band 1559-1610MHz and should be tested.

Proposal 3: 1559-1610MHz is considered in RAN4 analysis when identifying LTE/NR bands or high-risk UL band combinations for testing.

Proposal 4: The list based on RAN4 agreement is documented in TS 37.571-1, i.e., maintained by RAN5.

Proposal 5: RAN4 sends a reply LS to RAN5 with the agreed list of bands and band combinations at this meeting.

# References

- R5-206900, “LS on Frequency Bands for testing of A-GNSS Sensitivity requirements in NR and LTE”

- R4-2100196, “Analysis of RF interferences to A-GNSS Sensitivity requirements in NR and LTE”

- R4-2103287, “R4-2100196, “Analysis of RF interferences to A-GNSS Sensitivity requirements in NR and LTE”