3GPP TSG-RAN WG4 Meeting # 99-e	R4-2109354

# Electronic Meeting, May 19-27, 2021

Agenda item:	6.1.7.3

Source:	Apple Inc.

Title:	Discussion on CQI reporting requirements for NR-U

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#98bis-e UE demodulation requirements for NR-U was discussed and way forward [1] was agreed. The agreements for CQI reporting requirements are summarized below.

Test Scope for CQI Reporting - Two sets of burst transmissions, each with distinct transmission power level and keeping the interference level constant during the test. - Choose power level randomly per each DL period [0, +6]dB with equal probability; - Determine PDSCH transport format for each transmission power level boost independently, depending on the boost applied to the resources measured by the UE to produce the CQI reports received - Collect PDSCH and CQI reporting results separately per each transmission power level boost; - Test metrics - CQI distribution statistics - PDSCH BLER - Minimum difference in median CQI between sets collected per each transmission power level boost. - Periodic CSI-RS with periodicity 10 slots, offset 1 - CSI reporting is scheduled within the same DL periodicity in which CSI-RS for CSI acquisition was received (no Cross-COT reporting). - Type of reporting is FFS - Details of test setup for CSI reporting is FFS - Same LBT model as for PDSCH demodulation requirements - Channel model: AGWN - Requirements shall be defined for 1 SNR pair

In this contribution we present our views on the open issues and simulation results to define requirements for CQI reporting requirements for NR-U.

# 2. Discussion

## Test Scope for CQI reporting

For CQI reporting requirements the following open issues need to be addressed:

- Type of CQI reporting

- Test set-up for CQI reporting to avoid cross-COT reporting.

In [1] it was agreed to use periodic CSI-RS with periodicity of 10 slots and offset 1. With periodic reporting, the minimum number of slots between the CSI-RS and corresponding report is 8, with the current assumptions of DL transmission scheme, we cannot avoid cross-COT reporting.

For CQI reporting test cases, we don’t schedule PDSCH in slots with scheduled CSI-RS transmission, hence with the agreed test-up, the UE will not be able to do CSI-RS validation as no PDCCH for scheduling is transmitted.

Observation #1: For CQI reporting requirements, we don’t transmit CSI-RS and PDSCH in the same slot. The UE will not be able to do CSI-RS validation with periodic CSI-RS transmitted with periodicity 10 slots and offset 1 since no PDCCH scheduling data is transmitted.

To avoid this, we can configure PDCCH with DCI 0-1 with aperiodic CSI trigger in slot 1 of every fixed frame period. The aperiodic report slot offset should be 6 to allow for CSI report transmission on slot 8.

Figure 1: Test setup for CQI reporting

Proposal #1: In slot 1 of FFP configure Aperiodic CSI report with aperiodic report slot offset of 6.

With this configuration we can re-use the same downlink transmission scheme as agreed for PDSCH demod requirements.

- DL transmission duration will be randomly selected between {2,4,6,7} Slots, with equal probability.

- In every slot within the DL Transmission duration allocate PDSCH from Symbol 2 to Symbol 13, except for the last slot of the DL Transmission

- For the last slot of the DL Transmission, allocate PDSCH:

- If DL Transmission duration is 2 Slot, from Symbol 2 to Symbol 13

- If DL Transmission duration is larger than 2 Slot, from Symbol 2 to Symbol {5, 8, 11, 13} with equal probability.

Proposal #2: Use same downlink transmission scheme as agreed for PDSCH demod requirements.

## CQI Reporting Requirements

For CQI reporting requirements for NR-U, we can re-use the test parameters for CQI reporting for unlicensed carrier for TDD mode and static channel conditions. It was agreed to define requirements for 1 SNR pair and the transmission would have a random power boost of [ 0, +6] dB.

Proposal #3: Re-use the test parameters from CQI reporting tests in static channel for unlicensed carrier where applicable.

We propose to reuse the SNR pair for Test 1 from CQI reporting tests in static channel.

Proposal #4: Define CQI reporting requirements for NR-U with the same SNR pair as Test 1 for existing CQI reporting tests in static channel.

Based on the proposals above, we provide results for median CQI for CQI table 2 with wideband CQI reporting in the table below.

Table 1: Median CQI reporting results

|  | SNR | Power Offset | Median CQI |
| --- | --- | --- | --- |
| 2x2 | 8 | 0 | 6 |
|  |  | 6 | 9 |
|  | 9 | 0 | 7 |
|  |  | 6 | 10 |
| 2x4 | 5 | 0 | 7 |
|  |  | 6 | 10 |
|  | 6 | 0 | 7 |
|  |  | 6 | 10 |

Based on the results above, we see delta of 3 in median CQI between 0 dB and 6dB power boost. We recommend that the minimum median CQI delta be set as 2 between 0 dB and 6dB power bursts.

Proposal #5: Define minimum delta of median CQI of 2 for different transmission burst.

# 3. Conclusion

In this paper, we provide our views on the remaining open issues related to CQI reporting requirements for NR-U. Our observations and proposals are captured below:

Observation #1: For CQI reporting requirements, we don’t transmit CSI-RS and PDSCH in the same slot. The UE will not be able to do CSI-RS validation with periodic CSI-RS transmitted with periodicity 10 slots and offset 1 since no PDCCH scheduling data is transmitted.

Proposal #1: In slot 1 of FFP configure Aperiodic CSI report with aperiodic report slot offset of 6.

Proposal #2: Use same downlink transmission scheme as agreed for PDSCH demod requirements.

Proposal #3: Re-use the test parameters from CQI reporting tests in static channel for unlicensed carrier where applicable.

Proposal #4: Define CQI reporting requirements for NR-U with the same SNR pair as Test 1 for existing CQI reporting tests in static channel.

Proposal #5: Define minimum delta of median CQI of 2 for different transmission burst.

# Reference

- R4-2106173, “Way Forward on NR-U UE demodulation requirements”, Qualcomm.